3GPP TSG RAN WG2 Meeting #122                                           	R2-2305946

Incheon, Korea, May 22nd – 26th 2023

Agenda item:			7.15.2

Source:			Intel Corporation

Title:		On SL-U LBT failure

Document for:	 	Discussion

# Introduction

In the last RAN2 meeting, there was some discussion on Rel-18 sidelink evolution and RAN2 made several agreements and working assumptions regarding the SL-LBT failure detection. The following agreements were made in the RAN2#121 meeting [1]:

Agreement

SL C-LBT failure is declared per RB-set

Confirm the following working assumption:

UE uses the MAC CE to report consistent LBT failure to the gNB

Agreement:

Uu MAC CE indicates RB set(s) where C-LBT failure happens.

Confirm the working assumption:

UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets.

Agreement:

RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing.

In this contribution, we discuss the open issues specifically related to SL LBT operation and present our views.

# Discussion

One of the open issues that RAN2 needs to discuss are details for the MAC CE used to report the SL LBT failure to the network. Firstly, it needs to be discussed how to handle the case when SL MAC CE for SL consistent LBT failure is triggered but there is no UL grant. Note that RAN2 has already agreed to support this reporting for both mode-1 and RRC_CONNECTED mode-2 UE. For Uu, the LBT failure prompts the UE to generate an LBT failure MAC CE to inform the network about the cell where LBT failure is triggered. In case UE does not have UL resources for transmission of this MAC CE, the UE triggers a Scheduling Request for LBT failure MAC CE. Given that the UE is allowed to trigger SR for transmitting the LBT failure MAC CE for NR-U, we also need to consider if a new SR needs to be defined for SL-U. Defining a new SR for the LBT MAC CE specifically allows for a specific format, but potentially at the cost of increased specification effort. Alternatively, given that the UE may be configured with SR for SL CSI reporting MAC CE, it can also be reused for SL consistent LBT failure MAC CE.

Proposal 1a: If SL consistent LBT failure MAC CE is triggered and no UL grant is available, UE triggers SR.

Proposal 1b: RAN2 shall discuss whether the SR for SL CSI reporting MAC CE shall be reused for SL consistent LBT failure MAC CE.

Lastly, it is FFS whether and how the trigger for SL consistent LBT failure may be cancelled. In our view, firstly, the same conditions as for NR-U LBT failure detection can be reused, i.e. when sl-LBT-FailureDetectionTimer or sl-LBT-FailureInstanceMaxCount is reconfigured by upper layers, the SL LBT failure should be cancelled. Moreover, upon successful transmission of the SL consistent LBT failure MAC CE, the triggered SL LBT failure should be cancelled.

Proposal 2: The triggered SL consistent LBT failure may be cancelled at least for the following cases:

- when sl-LBT-FailureDetectionTimer or sl-LBT-FailureInstanceMaxCount is reconfigured by upper layers

- upon successful transmission of the SL consistent LBT failure MAC CE

# Conclusion

This contribution discusses open aspects regarding SL consistent LBT failure and makes the following proposals:

Proposal 1a: If SL consistent LBT failure MAC CE is triggered and no UL grant is available, UE triggers SR.

Proposal 1b: RAN2 shall discuss whether the SR for SL CSI reporting MAC CE shall be reused for SL consistent LBT failure MAC CE.

Proposal 2: The triggered SL consistent LBT failure may be cancelled at least for the following cases:

- when sl-LBT-FailureDetectionTimer or sl-LBT-FailureInstanceMaxCount is reconfigured by upper layers

- upon successful transmission of the SL consistent LBT failure MAC CE

# References

| [1] | RAN2#121bis-e meeting, Chariman Notes. |
| --- | --- |