3GPP TSG-RAN WG2 Meeting #122		 R2-2305969

Incheon, Korea: 22nd May – 26th May 2023

Source:	Qualcomm Incorporated

Title:	Discussion on AI/ML Architecture General

Agenda Item:	7.16.2.1 Architecture General

Document for:	Discussion and Decision

# Introduction & Background

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1], with the main goal of exploring the benefits of augmenting the air interface with features enabling improved support of AI/ML-based algorithms for enhanced performance and/or reduced complexity/overhead.

Through studying a few carefully selected use cases, the goal is to identify a common AI/ML framework, including functional requirements of AI/ML architecture, which could be used in subsequent projects. The study should also identify areas where AI/ML could improve the performance of air-interface functions.

The study will serve to identify what is required for an adequate AI/ML model characterization and description establishing pertinent notation for discussions and subsequent evaluations. Various levels of collaboration between the gNB and UE are identified and considered. Specification impact will be assessed to improve the overall understanding of what would be required to enable AI/ML techniques for the air interface.

The SI consists of studying individual use cases as well as deriving a general framework for AI/ML. Below we summarize the goal of the study as shown in [1,2] relevant to the general framework:

AI/ML model, terminology, and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting separate or joint ML operations.

- Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures, and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

The SI further defines responsibility for different WGs for accessing potential specification impacts [1,2], whereas the RAN2 study access protocols aspects of the potential specification impacts, as mentioned below:

- […]

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input

    - Collaboration level-specific specification impact per use case

In this contribution, we will discuss potential architecture (allocation of functionality to entities), and information relevant for model identification and management (i.e., what information should be part of model ID for model identification, and what information should be part of meta information for model management and control).

# Protocols design aspects (Architecture General)

As described in the SID [1, 2], RAN2 should access protocol-related specification impact considering the progress in RAN1 as the reference. In this contribution paper, we will take RAN1 agreements (in RAN1#109-e [3], RAN1#110 [4], RAN1#110bis [5], RAN1#111 [7], RAN1#112 [9], RAN1#112bis-e [11]) and RAN2 agreements (in RAN2#119bis-e [6], RAN2#120 [8], RAN2#121 [10], RAN2#121bis-e [12]) meeting agreements to discuss potential architecture, framework, signaling, and procedural aspects. In the contribution paper, we discuss the following,

- Use of meta info for model identification and model management procedures

- Functional blocks and their mapping

## Use of meta info for model identification and model management procedures

In our understanding, some information may be required for model identification, some information may be required for model inference (i.e., input/output of the model), and some other information may be required for model management and control purposes.

- Model identification, such as an ID, and

- Model inference information, such as model input/output, etc.

- Model management and control purpose, such as applicable conditions, etc.

Observation 1: The following information may be relevant for a model

- Model identification information, such as an ID, and

- Model inference information, such as model input/output, etc.

- Model management and control information, such as applicable conditions, etc.

    - Use of meta info for model identification procedures

During the initial model identification procedure, i.e., when an identifier (ID) is not assigned to the model, some relevant information can be provided such that the network can understand the usage of the model and provide an identifier for subsequent usage. The relevant information can be provided in the meta information.

Observation 2: During the initial model identification procedure, i.e., when an identifier (ID) is not assigned to the model, some relevant information can be provided in the meta information. They can be reused during the subsequent operations (for model management and control) after the model ID is assigned.

Proposal 1: For the model ID assignment procedure, meta information (or metadata) can be used for the initial model identification procedure.

Meta info should contain information for the initial model identification, and model management and control purposes. For example, meta info should contain information about model functionalities, i.e., for which feature/feature group/functionality model is applicable. Similarly, it should contain information on the applicability of the model, e.g., scenarios, configurations, sites, etc. where the model is applicable. It should also contain information on expected performance to facilitate model selection at the NG-RAN. In RAN2#121, RAN2 made the following agreements

RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

Note that when a model is updated, its meta info may or may not be updated. Therefore, when meta info is not updated, to achieve a unique “globally” model ID for the updated model, some information should be provided by the vendor such that the updated model can be differentiated.

Proposal 2: The meta info of the model should contain the following information for initial model identification (when an ID is not assigned to the new/updated model), and control and management purposes

- Model feature/feature group (i.e., for which feature/feature group mode is applicable)

- Applicability of the models (e.g., scenarios, zones, etc.)

- Vendor-provided information (for identifying the model before a model ID is assigned)

- Expected model performance/model performance indicator (for model selection/switching during configuration when multiple models with varying performance are available)

    - Model inference information

Some information such as the input/output of the model may not be required for model management and control rather they are required for model inference. Furthermore, note the output should be defined per use case or sub-use case, i.e., all the models implementing a use case or sub-use case may be required to produce the same output parameters. Therefore, model output may not be required for model management and control.

Note that one cannot expect that if UE knows the input/output of the model it can run the model inference. Therefore, we believe that information relevant to model inference cannot be provided in runtime rather it should be provided to the UE as software/hardware updates.

Observation 3: Some information such as the input/output of the model may not be required for model management and control rather they are required for model inference.

Observation 4: The output should be defined per use case or sub-use case, i.e., all the models implementing a use case or sub-use case may be required to produce the same output parameters.

Proposal 3: The information such as the input/output of the model has no relevance to model management and control, therefore should not be part of meta-information.

Observation 5: UE should not be expected to perform model inference by knowing the input/output of the model. UE may require updates to general and feature-specific procedures, protocols, and signaling.

Proposal 4: The information relevant to model inference, such as model input/output, cannot be provided in runtime rather it should be provided to the UE as software/hardware updates.

    - Use of meta info for model management and control procedures

Note that meta info is not UE-specific information rather it is model-specific information. Therefore, during the initial model identification procedure, the network should map the provided meta information with the assigned ID for subsequent operations. The meta information is stored in the network for control and management purposes.

Observation 6: Meta info is not UE-specific information rather it is model-specific information.

Proposal 5: During the initial model identification procedure, the network should map the provided meta information with the assigned ID for subsequent operations.

Proposal 6: Meta info is stored at the network (e.g., NG-RAN) for further control and management purposes.

Proposal 7: After an ID is assigned to the model, UE indicates the IDs of the supported models for model identification, in subsequent procedures.

## Functional blocks and their mapping

In RAN2#121bis-emeeting [12], RAN2 made the following agreements on the functional framework,

The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

During the RAN2#121bis-emeeting online discussion, some companies argued that for functionality-based LCM, there is no need for model management. To address this concern, the RAN2 chair proposed modifying the name of the functional block to Model Management / Performance Monitoring. We agree with the proposed naming by the chair during the meeting.

Observation 7: During the RAN2#121bis-emeeting online discussions, some companies argued that for functionality-based LCM, there is no need for model management.

Observation 8: To address the concern raised in observation 1, the RAN2 chair proposed modifying the name of the functional block to Model Management / Performance Monitoring.

Proposal 8: Rename the functional block “model management” (in RAN2#121bis-emeeting agreement) to “Model Management / Performance Monitoring”.

Some companies further argued that for the functionality-based LCM, model transfer/delivery may not be required. However, While the model transfer is defined as the “delivery of AI/ML models over the air interface”; model delivery is defined as the “delivery of model from one entity to another entity in any manner”. Therefore, our understanding is that there may be model transfer/delivery even in the case of functionality-based LCM. In the functionality-based LCM, the delivery of the models may be transparent to the 3GPP. Furthermore, note that for the model ID-based LCM and level-Z collaboration between the UE and network, model transfer/delivery becomes an integral part of the functional framework.

Observation 9: While the model transfer is defined as the “delivery of AI/ML models over the air interface”; model delivery is defined as the “delivery of model from one entity to another entity in any manner”.

Observation 10: There may be model transfer/delivery even in the case of functionality-based LCM. In the functionality-based LCM, the delivery of the models may be transparent to the 3GPP.

Observation 11: For the model ID-based LCM and level-Z collaboration between the UE and network, model transfer/delivery becomes an integral part of the functional framework.

On another note, in the RAN1#112 meeting [9], RAN1 made the following agreements

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Observation 12: Based on the RAN1#112 agreement, the training and model storage may happen at different entities. For example, the training may happen on the UE side (e.g., UE vendor, etc.) while the model can be stored on the network side, or vice versa.

Proposal 9: Model storage is a necessary functional block of the general AI/ML framework.

The below figure represents different required functional blocks and signaling/procedures required between two functional blocks.

Figure 1. AI/ML functional architecture

Proposal 10: RAN2 should consider the general AI/ML functional architecture in Figure 1 as the baseline.

In the RAN2#120 meeting [], the chair noted that it is allowed to discuss/determine that functionality can be done outside 3GPP, e.g., at the OTT server. Furthermore, note that for the existing RAN1 use cases, one functional block can be mapped to one or more entities (e.g., UE, gNB, CN, OTT server, Application Service Provider (ASP), etc).

Observation 13: Chair: It is allowed to discuss/determine that functionality can be done outside 3GPP system scope, i.e. OTT server. NO agreement for now on the specifics due to long discussion.

Proposal 11: Function blocks can be mapped to entities, such as OTT servers, Application Functions (inside/outside the 3GPP network), and Application Service Providers (ASPs) (inside/outside the 3GPP network).

Observation 14: For the existing RAN1 use cases, one functional block can be mapped to one or more entities (e.g., UE, gNB, CN, OTT server, Application Service Provider (ASP), etc.).

Proposal 12: As a functional block may map to many entities (UE, RAN node, CN node, OTT server, etc.), RAN2 should avoid spending further time on functional blocks and their mapping.

# Conclusion

Observation 1: The following information may be relevant for a model

- Model identification information, such as an ID, and

- Model inference information, such as model input/output, etc.

- Model management and control information, such as applicable conditions, etc.

Observation 2: During the initial model identification procedure, i.e., when an identifier (ID) is not assigned to the model, some relevant information can be provided in the meta information. They can be reused during the subsequent operations (for model management and control) after the model ID is assigned.

Proposal 1: For the model ID assignment procedure, meta information (or metadata) can be used for the initial model identification procedure.

Proposal 2: The meta info of the model should contain the following information for initial model identification (when an ID is not assigned to the new/updated model), and control and management purposes

- Model feature/feature group (i.e., for which feature/feature group mode is applicable)

- Applicability of the models (e.g., scenarios, zones, etc.)

- Vendor-provided information (for identifying the model before a model ID is assigned)

- Expected model performance/model performance indicator (for model selection/switching during configuration when multiple models with varying performance are available)

Observation 3: Some information such as the input/output of the model may not be required for model management and control rather they are required for model inference.

Observation 4: The output should be defined per use case or sub-use case, i.e., all the models implementing a use case or sub-use case may be required to produce the same output parameters.

Proposal 3: The information such as the input/output of the model has no relevance to model management and control, therefore should not be part of meta-information.

Observation 5: UE should not be expected to perform model inference by knowing the input/output of the model. UE may require updates to general and feature-specific procedures, protocols, and signaling.

Proposal 4: The information relevant to model inference, such as model input/output, cannot be provided in runtime rather it should be provided to the UE as software/hardware updates.

Observation 6: Meta info is not UE-specific information rather it is model-specific information.

Proposal 5: During the initial model identification procedure, the network should map the provided meta information with the assigned ID for subsequent operations.

Proposal 6: Meta info is stored at the network (e.g., NG-RAN) for further control and management purposes.

Proposal 7: After an ID is assigned to the model, UE indicates the IDs of the supported models for model identification, in subsequent procedures.

Observation 7: During the RAN2#121bis-emeeting online discussions, some companies argued that for functionality-based LCM, there is no need for model management.

Observation 8: To address the concern raised in observation 1, the RAN2 chair proposed modifying the name of the functional block to Model Management / Performance Monitoring.

Proposal 8: Rename the functional block “model management” (in RAN2#121bis-emeeting agreement) to “Model Management / Performance Monitoring”.

Observation 9: While the model transfer is defined as the “delivery of AI/ML models over the air interface”; model delivery is defined as the “delivery of model from one entity to another entity in any manner”.

Observation 10: There may be model transfer/delivery even in the case of functionality-based LCM. In the functionality-based LCM, the delivery of the models may be transparent to the 3GPP.

Observation 11: For the model ID-based LCM and level-Z collaboration between the UE and network, model transfer/delivery becomes an integral part of the functional framework.

Observation 12: Based on the RAN1#112 agreement, the training and model storage may happen at different entities. For example, the training may happen on the UE side (e.g., UE vendor, etc.) while the model can be stored on the network side, or vice versa.

Proposal 9: Model storage is a necessary functional block of the general AI/ML framework.

Observation 13: Chair: It is allowed to discuss/determine that functionality can be done outside 3GPP system scope, i.e. OTT server. NO agreement for now on the specifics due to long discussion.

Proposal 11: Function blocks can be mapped to entities, such as OTT servers, Application Functions (inside/outside the 3GPP network), and Application Service Providers (ASPs) (inside/outside the 3GPP network).

Observation 14: For the existing RAN1 use cases, one functional block can be mapped to one or more entities (e.g., UE, gNB, CN, OTT server, Application Service Provider (ASP), etc.).

Proposal 12: As a functional block may map to many entities (UE, RAN node, CN node, OTT server, etc.), RAN2 should avoid spending further time on functional blocks and their mapping.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

- RAN1#109-emeeting report

- RAN1#110 meeting report

- RAN1#110bis-meeting report

- R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online”, RAN2#119bis-e, Oct 2022.

- Final_Minutes_report_RAN1#111_v100, “Final Report of 3GPP TSG RAN WG1 #111 v1.0.0  (Toulouse, France, 14th – 18th November 2022)”, Nov 2022.

- R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France”, RAN2#120, Nov 2022.

- Draft_Minutes_report_RAN1#112_v040, “Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0  (Athens, Greece, 27th February – 3rd March 2023)” Mar 2023.

- Draft_R2_121_meeting_report_v1, “Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece”, Ran2#121, Feb 2023.

- Draft_Minutes_report_RAN1#112b-e_v010, “Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0,” April 2023.

- Draft_R2_121bis_meeting_report_v2, “Draft Report of 3GPP TSG RAN WG2 meeting #121bis-e Online,” April 2023.