3GPP TSG-RAN WG2 Meeting #122	R2-2306128

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:		7.18.2

Source:				ASUSTeK

Title:		Discussion on DL SPS for MT-SDT

Document for: 	Discussion and Decision

- Introduction

The work item for MT-SDT is described as below [1]:

## 3 Justification Rel-17 specified MO-SDT to allow small packet transmission for UL-oriented packets. For DL, MT-SDT (i.e. DL-triggered small data) allows similar benefits, i.e. 1) reducing signalling overhead and UE power consumption by not transitioning to RRC_CONNECTED and reducing latency by allowing fast transmission of (small and infrequent) packets, e.g. for positioning. ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

In RAN2 #121 meeting, the DL SPS for MT-SDT has been discussed without reaching consensus. In this contribution, we further discuss the subsequent DL transmissions and DL SPS in a MT-SDT procedure.

- Discussion

As described in the work item [1], MT-SDT supports initial DL data transmission and subsequent UL/DL data transmissions. It was also agreed in RAN2 #120 meeting that subsequent UL/DL data belonging to SDT bearers while in RRC inactive state is allowed in MT-SDT procedure. Moreover, the MT-SDT is introduced for reducing signalling overhead and UE power consumption in RRC inactive state.

Observation 1: The WI of MT-SDT is introduced for reducing signalling overhead in RRC inactive state with initial DL data transmission and subsequent data transmissions.

In a MT-SDT procedure, there could be multiple subsequent small data transmissions after an initial DL small data transmission from the network. For example, as shown in Figure 1, the UE receives a RRCRelease message and enters RRC inactive state. When there is available DL small data, the network would transmit a paging message to indicate MT-SDT. In response to receiving the MT-SDT indication, after checking conditions such as data volume and RSRP, the UE would transmit a RRCResumeRequest message to trigger a MT-SDT procedure. After receiving the RRCResumeRequest message, the network could dynamically schedule subsequent DL transmissions. When there are multiple DL transmissions, the network needs to send multiple PDCCH transmissions to schedule each PDSCH transmission, which may cause signalling overhead.

Observation 2: When subsequent DL small data transmissions are needed, the network needs to send multiple PDCCH transmissions to schedule DL small data transmissions, causing signalling overhead.

Figure 1: The concept of subsequent DL transmissions in MO-SDT

For MO-SDT, UL CG resources could be used for transmitting the subsequent data following the initial small data transmission. Since the use case for MT-SDT is similar to MO-SDT, i.e., with subsequent transmissions following the initial transmission, DL SPS can be considered to reduce the PDCCH overhead for MT-SDT. The network can transmit the multiple DL small data via the DL SPS resources when suitable.

Observation 3: DL SPS could reduce signalling overhead for subsequent DL small data transmissions in MT-SDT procedure.

Based on above observations, we propose to support DL SPS in MT-SDT.

Proposal 1: Support DL SPS for DL data transmissions in MT-SDT.

Moreover, as discussed in the offline discussion in RAN2 #121 meeting, the network could provide configuration of DL SPS in the RRCRelease message when transiting the UE into RRC inactive state. Similar to CG-SDT, when the UE is in the same serving cell, the UE could use the configured DL SPS for MT-SDT. After RRCResumeRequest message is sent by the UE in response to MT-SDT paging, the network could activate or deactivate the DL SPS by PDCCH. Then the UE could receive multiple DL small data transmissions via the DL SPS resources.

An example is shown in Figure 2, the network could provide configuration of CG-SDT and configuration of DL SPS in a RRCRelease message. In response to receiving the RRCRelease message, the UE enters RRC inactive state. When there are DL small data to transmit in RRC inactive state, the network could transmit a paging message to indicate MT-SDT. In response to receiving the MT-SDT indication, the UE could check the triggering conditions and use CG-SDT resources to transmit a RRCResumeRequest message. After receiving the RRCResumeRequest message, the network could send a PDCCH to activate the DL SPS. Then the UE could receive the initial DL small data transmission and subsequent DL small data transmissions via DL SPS resources.

Figure 2: The concept of subsequent DL transmissions with DL SPS in MO-SDT

Proposal 2: Configuration of DL SPS for MT-SDT is provided by RRCRelease message.

Proposal 3: DL SPS for MT-SDT is activated/deactivated by PDCCH.

- Conclusion

In this contribution, we discuss DL SPS for subsequent DL small data transmissions and have following observations and proposals:

Observation 1: The WI of MT-SDT is introduced for reducing signalling overhead in RRC inactive state with initial DL data transmission and subsequent data transmissions.

Observation 2: When subsequent DL small data transmissions are needed, the network needs to send multiple PDCCH transmissions to schedule DL small data transmissions, causing signalling overhead.

Observation 3: DL SPS could reduce signalling overhead for subsequent DL small data transmissions in MT-SDT procedure.

Proposal 1: Support DL SPS for DL data transmissions in MT-SDT.

Proposal 2: Configuration of DL SPS for MT-SDT is provided by RRCRelease message.

Proposal 3: DL SPS for MT-SDT is activated/deactivated by PDCCH.

- Reference

- RP-213583, “Mobile Terminated-Small Data Transmission (MT-SDT) for NR.”