3GPP TSG-RAN2#122	R2-2306143

Incheon, 22th May – 26th May, 2023

Agenda item:		8.5.3 (NR_XR_enh-Core)

Source:	LG Electronics Inc.

Title: 	DRX enhancement for power saving in XR

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed solutions to resolve non-integer periodicity related issues, i.e. DRX mismatch and SFN wrap-around, and made agreements as follows.

RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration. To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

In this contribution, we present our views on the XR-specific power saving based on the last meeting agreements.

2.	Discussion

According to [1], the packet arrival rate for XR frames are evaluated as 30/60/90/120 fps and this means that the corresponding XR traffic periodicities are non-integer value of 33.33/16.66/11.11/8.33ms. However, currently, DRX cycle is designed to support integer value, e.g. Long DRX cycle: 10, 20, 32, …, 10240ms, Short DRX cycle: 2, 3, 4, …, 640ms, so there is a misalignment between XR traffic periodicities and the current DRX cycle.

This misalignment causes the time difference between the periodicity of XR traffic and DRX cycle, and the XR traffic may not arrive in DRX onDuration due to the accumulation of the time difference. As a result, additional PDCCH monitoring is required to receive the XR traffic.

In the last meeting, RAN2 discussed three options to resolve the mismatch issue. One option was precluded and only two options are on the table, i.e. Rational number for DRX cycle periodicity and DRX cycle adjustments.

Looking on to these options, each options are diverged into different options. Based on the last meeting contribution, each solution can be summarized as follows.

- Option 1: Rational number for DRX cycle periodicity

  - Option 1-1. Uniform DRX cycle expressed as a rational number [2]

: In this option, new values of periodicity represented in rational numbers, i.e. 1000/fps ms, are adapted for drx-LongCycle/drx-ShortCycle and the floor operation is added for the DRX formula. For example, the updated formula for short DRX cycle is as follows.

floor{[(SFN × 10) + subframe number] modulo (drx-LongCycle)} = drx-StartOffset:

floor{[(SFN × 10) + subframe number] modulo (drx-ShortCycle)} = floor{(drx-StartOffset) modulo (drx-ShortCycle)}:

With Option 1-1, the DRX formula can be re-used with small change. Most of all, this option has less impact than other options. In addition, the proposed DRX formula can be used commonly for both integer DRX cycle and non-integer DRX cycle, and we think this is efficient.

- Option 2: DRX cycle adjustments

  - Option 2-1. Non-uniform integer DRX cycle [3]

: In this option, the length of DRX cycle is changed at every n:th DRX cycle.

In the above example, the length of DRX cycle is changed at every 3:rd DRX cycle. So, to calculate the length of each DRX cycle, many changes are unavoidable to the current DRX formula and this complicates the DRX operation. Thus, we don’t prefer Solution 3 considering complexity and changes.

  - Option 2-2. Multiple onDuration within single DRX cycle [3]

: In this option, DRX configuration includes multiple drx-StartOffset, and, as a result, there are multiple onDurations within one DRX cycle.

Currently, the start of onDuration is calculated per DRX cycle. In Option 2-2, onDuration needs to be calculated within one DRX cycle, so big impact on DRX operation is forecasted and this complicates the DRX operation. Thus, we don’t prefer Option 2-2 considering complexity and changes.

  - Option 2-3. Adjustment of DRX cycle length/start offset every number of cycles [3]

: In this option, DRX cycle is fixed and drx-StartOffset is adjusted every DRX cycles.

In Option 2-3, a new formula (or a new method) should be defined to adjust drx-StartOffset. To calculate the start of DRX cycle, the UE needs to manage two formulas. We think this complicates the DRX operation. Thus, we think this option is not desirable in terms of complexity.

Based on the above, we propose Option 1-1, i.e. rational number approach, to resolve mismatch issues.

Proposal 1. In order to resolve mismatch issue, RAN2 introduce rational number for DRX cycle periodicity.

In addition to the mismatch issue, SFN wrap-around issue was discussed. There were two issues. One is that XR traffic and DRX cycle are misaligned after SFN wrap-around because non-integer periodicity (e.g. 50/3ms) cannot fit into 10240ms. The other is that DRX configuration can be retransmitted across hyper frames resulting onDuration mismatch between UE and NW.

To resolve the mismatch problem between XR traffic and DRX cycle, in the last meeting, RAN2 agreed to adopt a counter in DRX formula that increments at every SFN wrap around. Based on the agreement, DRX formula can be changed as follows.

- For Long DRX cycle: [(E-SFN ×10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

- For Short DRX cycle: [(E-SFN ×10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle)} = (drx-StartOffset) modulo (drx-ShortCycle):

For the new counter (i.e. E-SFN), however, it is not decided yet how the counter value is provided. There are two options.

- E-SFN is provided in broadcasting manner

- E-SFN is provided in dedicated manner, i.e. UE-specific E-SFN

In our view, UE-specific E-SFN increase the complexity and don’t see any benefit, whereas the broadcasting manner is simple and does not bring any complexity.

For UE-specific E-SFN, the network sends RRC message to configure E-SFN value. If the RRC message is sent at the end of boundary of hyper frame, the UE can receive RRC message in the next hyper frame due to retransmission. In this case, the UE and the network use different E-SFN value in DRX formula, and onDuration mismatch would occur. Therefore, in the last meeting, RAN2 agreed to signal the DRX reference SFN to avoid this problem.

However, if E-SFN value is broadcasted, the UE acquires E-SFN value from broadcast signaling, so the UIE and the network always use same SFN value. Thus, DRX reference SFN does not need to be signaled if E-SFN is broadcasted.

In addition, currently, SFN and H-SFN are broadcasted in MIB and SIB, respectively. We think that it is simple to use the same manner for E-SFN.

Therefore, we propose that E-SFN is provided in broadcasting manner and DRX reference SFN is not signaled.

Proposal 2. The new counter value (i.e. E-SFN) is provided in broadcasting manner, similar to SFN and H-SFN.

Proposal 3. If E-SFN is broadcasted, DRX reference SFN is not signaled.

Next, whether to introduce a maximum value for E-SFN needs to be discussed. Some companies think that limiting the maximum number of E-SFN introduces another wrap-around issue. So, they argued that E-SFN should keep increasing on the UE side, and the network should reset E-SFN before reaching maximum value.

However, without the knowledge of maximum number, it is unclear for UE how many bits are needed to store the E-SFN. Note that the UE has to store the E-SFN in a state variable, and the size of state variable should be fixed.

According to [1], the packet arrival rate for XR frames are evaluated as 30/60/90/120 fps. If the maximum value of E-SFN is set to 99, E-SFN is wrap-around at 1024000ms, and all of fps cases are covered without wrap-around issue. There may be a concern that 99 may not enough in future release, but we believe that a sufficiently large maximum number, e.g. 999, does not bring wrap-around issue in future release.

Thus, we propose that maximum value of E-SFN is fixed in the specification, taking into account a sufficiently large maximum number.

Proposal 4. Maximum value of E-SFN is fixed in the specification, taking into account a sufficiently large maximum number.

3.	Conclusion

In this document, we present our view on power saving in XR, and made proposals as follows.

Proposal 1. In order to resolve mismatch issue, RAN2 introduce rational number for DRX cycle periodicity.

Proposal 2. The new counter value (i.e. E-SFN) is provided in broadcasting manner, similar to SFN and H-SFN.

Proposal 3. If E-SFN is broadcasted, DRX reference SFN is not signaled.

Proposal 4. Maximum value of E-SFN is fixed in the specification, taking into account a sufficiently large maximum number.

4.	Reference

- 3GPP TR 38.838	Study on XR (Extended Reality) Evaluations for NR

- R2-2302514	DRX enhancements for XR	Qualcomm Incorporated, MediaTek, CATT, vivo, NEC

- R2-2303861	DRX cycle alignment for XR	Nokia, Nokia Shanghai Bell, CMCC, China Unicom, DENSO CORPORATION, Ericsson, Intel, Google Inc., Huawei, HiSilicon, Samsung, Xiaomi