3GPP TSG-RAN WG2 Meeting #122                                    	R2-2306251

# Incheon, South Korea, May 22nd - 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4027 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on configuration upon TN NTN mobility in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The network may provide configuration to the UE when releasing it to RRC_INACTIVE for a feature that requires the UE to perform a certain action. However, it is possible that the UE supports a feature in TN and yet not in NTN or vice versa. The UE behaviour is not clear in the specifications on how performing any actions in such scenarios can be avoided, what to do with the configuration, and whether/how the UE should fallback to. Small Data Transmission (SDT) is one such feature. Let’s consider the scenario where a UE supports SDT in TN and but not it in NTN. ‘A UE may receive the sdt-Config from a TN cell and reselect to an NTN cell that broadcasts the sdt-ConfigCommon. Conditions to initiate SDT have been captured in 5.3.13.1b in the current specification and one may assume that a UE in NTN may initiate SDT provided that the conditions given below are fulfilled. Ideally, the UE should in principle do not initiate a feature that it does not support as confirmed in RAN2#121: “RAN2 confirms that UE should only use/apply configurations of a given feature when UE supports the feature in the corresponding cell in which UE is camping, connecting, or resuming to.” #### 5.3.13.1b Conditions for initiating SDT A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled: - the upper layers request resumption of RRC connection; and - SIB1 includes sdt-ConfigCommon; and - sdt-Config is configured; and - all the pending data in UL is mapped to the radio bearers configured for SDT; and - lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled. NOTE: How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation. Another feature as such is eDRX and considering that there may be similar features specified in the future, there is a need to capture generic text that clarifies the UE behaviour.. |
| Summary of change: |  | It is clarified in clause 5.2.2.4.2 that when in RRC_INACTIVE if the UE is configured for an optional feature that it does not support in the current serving cell upon cell reselection between TN and NTN, the UE should consider that the configuration for that feature does not apply in the current serving cell. Impact analysis: Impacted functionality: RRC_INACTIVE state Inter-operability: No inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how a UE in RRC_INACTIVE should behave when moving from a TN to NTN cell when the UE does not support the configured feature in TN in an NTN cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselectionRedCap is set to allowed;

2> else:

3>	if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 1 Rx branch; or

3>	if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 2 Rx branches; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20];

1>	if the cellAccessRelatedInfo contains an entry of a selected SNPN or PLMN and in case of PLMN the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN;

1>	else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1> if the UE in RRC_INACTIVE is configured for an optional feature that it does not support in the current serving cell, e.g., upon cell reselection between TN and NTN:

2> consider that the configuration for that feature does not apply in the current serving cell;

1>	if in RRC_CONNECTED while T311 is not running:

2>	disregard the frequencyBandList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers, if included;

2>	forward the trackingAreaList to upper layers, if included;

2>	forward the received posSIB-MappingInfo to upper layers, if included;

2>	apply the configuration included in the servingCellConfigCommon;

2>	if the UE has a stored valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

3>	use the stored version of the required SIB or posSIB;

2>	else:

3>	acquire the required SIB or posSIB requested by upper layer as defined in clause 5.2.2.3.5;

NOTE:	Void.

1>	else:

2>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for downlink for TDD, or one or more of the frequency bands indicated in the frequencyBandList for uplink for FDD, and they are not downlink only bands, and

2>	if the UE is IAB-MT or supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported band in the downlink for TDD, or a supported band in uplink for FDD, and

2>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for RedCap UE, of the RedCap-specific initial uplink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP or, for RedCap UE, of the RedCap-specific initial uplink BWP if configured, and

2>	if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured, and

2>	if frequencyShift7p5khz is present and the UE supports corresponding 7.5kHz frequency shift on this band; or frequencyShift7p5khz is not present:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for RedCap UEs, RedCap-specific initial uplink BWP, if configured, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a RedCap UE, of the RedCap-specific initial uplink BWP if configured;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a RedCap UE, of the RedCap-specific initial downlink BWP if configured;

4>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the trackingAreaList to upper layers, if included;

4>	forward the received posSIB-MappingInfo to upper layers, if included;

4>	forward the PLMN identity or SNPN identity or PNI-NPN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.13.8;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the eCallOverIMS-Support to upper layers, if present;

4>	forward the UAC-AccessCategory1-SelectionAssistanceInfo or UAC-AC1-SelectAssistInfo for the selected PLMN/SNPN to upper layers, if present and set to a, b or c;

4>	if the UE is in SNPN access mode:

5>	forward the imsEmergencySupportForSNPN indicators with the corresponding SNPN identities to upper layers, if present;

4>	apply the configuration included in the servingCellConfigCommon;

4>	apply the specified PCCH configuration defined in 9.1.1.3;

4>	if the UE has a stored valid version of a SIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

5>	use the stored version of the required SIB;

4>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s), in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3;

4>	if the UE has a stored valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several required posSIB(s), in accordance with clause 5.2.2.1:

5>	use the stored version of the required posSIB;

4> if the UE has not stored a valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the posSI-SchedulingInfo, contain at least one requested posSIB and for which posSI-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the posSI-SchedulingInfo, contain at least one requested posSIB for which posSI-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3a;

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

5>	apply the additionalPmax for UL;

4>	else:

5>	apply the p-Max in uplinkConfigCommon for UL;

4>	if supplementaryUplink is present in servingCellConfigCommon; and

4>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and

4>	if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported supplementary uplink band; and

4>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP of the SUL:

5>	consider supplementary uplink as configured in the serving cell;

5>	select the first frequency band in the frequencyBandList for the supplementaryUplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

5>	apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial BWP of the SUL;

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList for the supplementaryUplink;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList for the supplementaryUplink:

6>	apply the additionalPmax in supplementaryUplink for SUL;

5>	else:

6>	apply the p-Max in supplementaryUplink for SUL;

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, is set to notAllowed;

END OF CHANGE