3GPP TSG-RAN WG2 Meeting #122	R2- 2306348

Incheon, Korea, May 22th – 26th, 2023

Title	Remaining issues on early indication for Rel-18 eRedCap UE

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	7.19.3

1 	Introduction

Regarding the Msg3-based early indication, it is agreed to define two new LCID as working assuption in RAN2#121bis [1].

Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

This contribution discussed our views on remaining aspects on early indication for Rel-18 eRedCap UE.

2	Discussion

Differentiation between PR1-only and BW3/PR3 + PR1

In the previous meeting, it is assumed that two new LCID values are introduced to support the Msg3-based early indication, and have a final check at the end of this release in order to ensure that there is sufficient number of LCID spaces:

Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

In RANP#99[2], it is agreed that there are two types of R18 eRedCap UE:

- UE capable of 20MHz + PR1 (i.e., PR1-only eRedCap UE)

- UE capable of BW3/PR3 + PR1 (i.e., BW3/PR3 + PR1 eRedCap UE)

That is, it is allowed to use larger bandwidth (i.e., 20 MHz) for PR1-only eRedCap UE, while the scheduling restriction of BW should be applied for BW3/PR3 + PR1 eRedCap UE. Therefore, it should be determined which step the UE capabilities of PR1-only eRedCap UE and BW3/PR3 + PR1 eRedCap UE are differentiated.

Some companies may want to differentiate these two types of UE using Msg3 early indication. However, in our view, it is not desirable to support the early indication to differentiate PR1-only eRedCap UE and BW3/PR3 + PR1 eRedCap. Specifically, if the Msg3-based early indication is additionally supported for PR1-only eRedCap UE, two more LCIDs are needed to differentiate PR1-only eRedCap UE. Considering that the remaining LCID spaces are very limited and two LCID values are already allocated for Msg3 early indication, it should be careful to consume two more LCIDs for Msg3 early indication to separate each feature within the Rel-18 eRedCap UEs, unless it is very critical.

In addition, in RANP#99[2], it is agreed that the initial access procedure for PR1-only eRedCap UE should be same as that of BW3/PR3 + PR1 eRedCap UE:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Therefore, it should be confirmed that Msg3-early indication is commonly used for all R18 eRedCap UE. When the R18 eRedCap UE transmits CCCH message using Msg3 or MsgA PUSCH, the new LCID defined for Rel-18 eRedCap UE should be used without any precondition, regardless of whether the UE is PR1-only eRedCap UE or BW3/PR3 + PR1 eRedCap UE. If the network needs to identify the PR1-only eRedCap UE in early stage, the additional indication in Msg5 can be discussed, i.e., in RRCSetupComplete message or in RRCResumeComplete message.

Proposal 1. The Msg3-based early indication is commonly used for all R18 eRedCap UE and differentiation between PR1-only eRedCap UE and BW3/PR3 + PR1 eRedCap UE can be identified in Msg5.

Other RA aspects for R18 eRedCap UE

In the RAN1#112bis meeting, it is agreed to send an LS [3] to ask whether the additional UE behaviour is needed when the DCI indicates that Msg4 PDSCH transmission has a larger BW than the UE can receive or process.

RAN1 discussed Msg4 PDSCH transmission to Rel-18 eRedCap UEs and made the following agreement: Agreement Confirm the following working assumption by assuming that Msg3 indication is available Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.. RAN1 would like to inform RAN2 about the following case, to consider, if needed, the UE behavior in the RAN2 specifications, and ask RAN2 for feedback if any: - For UE BB complexity reduction, the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process The case was also discussed in RAN1 in Question 2.7-2b of summary R1-2303936.

From RAN2 perspective, in order to determine whether the contention resolution is successful or not while ra-ContentionResolutionTimer is running, the MAC entity needs to receive the notification of a reception of a PDCCH transmission first from lower layer.

In this sense, when the DCI indicates that Msg4 PDSCH transmission has a larger BW than the UE can receive or process, following operation can be considered:

- Option 1: When the DCI indicates that Msg4 PDSCH transmission has a larger BW, the notification of a reception of a PDCCH transmission is not indicated to the MAC layer from the lower layer;

- Option 2: When the DCI indicates that Msg4 PDSCH transmission has a larger BW, the notification of a reception of a PDCCH transmission is indicated to the MAC layer from the lower layer;

  - Option 2-1: Follow the current Contention resolution procedure. (i.e., no spec impact)

  - Option 2-2: Modify the RA procedure in order to declare this Contention Resolution not successful.

For Option 1, if the PHY layer does not notify a reception of a PDCCH transmission to the MAC layer in this case, the MAC entity does not perform anything and continues monitoring the PDCCH until ra-ContentionResolutionTimer expires. Therefore, there is no RAN2 impact foreseen. However, it is up to RAN1 to decide whether the MAC entity receives the notification of a reception of a PDCCH transmission from the lower layer in this case.

Observation 1. When DCI indicates that Msg4 PDSCH transmission has a larger BW, it is up to RAN1 to decide whether to notify a reception of a PDCCH transmission or not.

For Option 2-1, if the MAC layer receives a notification of a reception of a PDCCH transmission for the case that RAN1 asked (i.e., when the DCI indicates that Msg4 PDSCH transmission has a larger BW than the UE can receive or process), the UE behaviour is different depending on the following cases:

- Case 1: the C-RNTI MAC CE is included in Msg3, e.g., the RA procedure is initiated when the UE is in RRC_CONNECTED

- Case 2: CCCH SDU is included and MAC PDU cannot be decoded

- Case 3: CCCH SDU is included and MAC PDU can be decoded (i.e., MAC entity may check the CR identity)

For the Case 1, it is obvious that C-RNTI would not be matched. Therefore, according to the current TS 38.321, UE continues monitoring the PDCCH until ra-ContentionResolutionTimer expires.

Regarding the Case 2, according to the attached agreement in RAN1 LS, the UE is not required to process a Msg4 PDSCH with a larger BW than the UE can receive. Therefore, in some UE implementation, MAC PDU may not be decoded even though the MAC layer detects a reception of a PDCCH transmission. In this case, the UE does nothing on the Msg4 PDSCH and continues monitoring the PDCCH until ra-ContentionResolutionTimer expires.

Regarding the Case 3, in some UE implementation, MAC PDU may be decoded even though Msg4 PDSCH with a larger BW than the UE can receive. In this case, the MAC entity may identify that the CR identity does not match, so the MAC entity may consider that the Contention resolution is not successful.

In summary, for all cases, the contention resolution would be failed anyway, i.e., there is no harm in UE procedure with Option 2-1.

Observation 2. Even though the PDCCH transmission is detected in MAC entity, the current RA procedure has no problem.

For Option 2-2, in order to determine whether the DCI indicates that Msg4 PDSCH transmission has a larger BW or not in MAC layer, the additional information should be received simultaneously with a notification of a reception of a PDCCH transmission from the lower layer. In other words, RAN1 impact is required in order to support the Option 2-2. In addition, further RAN2 discussion would be required to declare the Contention resolution not successful in this case. Given that the RAN1 has not reached any conclusion for this case and no new benefits are foreseen in RAN2 perspective, it is undesirable for RAN2 to modify the RA procedure causing RAN1 impact.

Observation 3. When the RA procedure is modified depending on the BW of Msg4 PDSCH transmission indicated in DCI, both RAN1 and RAN2 impact is foreseen.

Considering the above observations, Option 1 and Option 2-1 is preferred (i.e., no RAN2 work is needed in order to handle the case). The decision between Option 1 and Option 2-1 is up to RAN1, since it is not in RAN2 scope.

Proposal 2. From RAN2 perspective, no further work is needed when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

3	Conclusion

In this paper, we discussed our views on remaining issues on early indication and other RA aspects for eRedCap UE. The discussion includes the following observations:

Observation 1. When DCI indicates that Msg4 PDSCH transmission has a larger BW, it is up to RAN1 to decide whether to notify a reception of a PDCCH transmission or not.

Observation 2. Even though the PDCCH transmission is detected in MAC entity, there is no harm for the current RA procedure.

Observation 3. When the RA procedure is modified pending on the BW of Msg4 PDSCH transmission indicated in DCI, RAN1 impact is foreseen.

Based on the observations, followings are proposed:

Proposal 1. The Msg3-based early indication commonly used for all R18 eRedCap UE and differentiation between PR1-only eRedCap UE and BW3/PR3 + PR1 eRedCap UE can be identified in Msg5.

Proposal 2. From RAN2 perspective, no further work is needed when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

4	Reference

[1] Report from eRedCap breakout session, RAN2#121bis-e meeting

[2] RP-230778, Proposal for PR1 in eRedCap, Moderator (CMCC), RAN#99

[3] R2-2304619, LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs