3GPP TSG-RAN WG2 Meeting #122	R2-2306481

Incheon, Korea, May 22nd-26th, 2023

Agenda Item:	7.5.2

Source: 	China Unicom

Title:  	Discussion on XR awareness

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, there are some agreements on XR awareness as shown below in [1].

UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting. FFS on whether EoDB signalling is needed.

This contribution provides our considerations on XR awareness, especially on XR traffic assistance information from UE to network and the way assistance information is transmitted.

Discussion

RAN2 confirms that there may be jitter in the arrival of traffic from XR to UE, for example, the encoder may introduce jitter when generating different types of frames. Jitter may affect how gNB manages UE scheduling. We believe that UE should send gNB jitter information to achieve better scheduling, at least including average arrival time and jitter range.

Due to the periodicity of XR service, UE can periodically measure the average arrival time of data packets, calculate the average arrival time and jitter range through statistical rules, and transmit them to RAN. This parameter can be used at gNB for UE UL scheduling and further help UE save power.

There is other information about jitter, such as the latest arrival time of jitter, variance of jitter range, etc. Whether other jitter-related parameters should be sent to NW needs to be further discussed.

Proposal 1: UE should provide jitter statistics to gNB, including average arrival time and jitter range. Whether other relevant parameters need to be provided needs further discussion.

At present, many companies provide some solutions to send UL periodic information, the most common is RRC signalling and MAC CE. We believe that sending periodic messages through RRC is a better option because it already supports the framework for reporting UE assistance information (UAI). The method based on RRC has high reliability. Although this method may cause larger transmission delay and affect traffic when reconfigured. However, the UL periodicity usually does not change frequently, thus RRC method is more suitable.

The advantage of MAC CE is that the transmission rate will be transmitted in MAC layer, which can adapt to the dynamic traffic pattern, but too frequent updates will bring greater pressure to the scheduler because of the lack of sufficient time to adapt. For dynamic or semi-static assistant information, MAC CE can be considered.

Proposal 2: XR traffic assistance information is recommended to be sent by RRC, MAC CE can be considered in special cases.

SA2 has agreed that CN provides RAN with burst end indication through GTP-U header in the header of the last PDU in data burst. When the RAN receives the instruction, it represents the end of the data burst, and the DRX activity time of the UE can be terminated immediately to achieve the purpose of power saving.

We think End of Data Burst indication is very useful for XR applications with up-link-centric traffic. The network usually does not know when the data burst on the uplink ends, because the size of the data burst in the XR traffic varies from frame to frame and is unpredictable.

We note some views that BSR = 0 can be used to indicate the end of the data burst. However, this method cannot make the UE notice at the first time. If the UE can provide a burst end indication to the network, the network can use this information to determine whether to terminate the DRX activity time of the UE in advance. This way can provide UE more occasions to turn into sleeping time and save power.

Proposal 3: It is necessary for UE to send the end of data burst indication to gNB.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: UE should provide jitter statistics to gNB, including average arrival time and jitter range. Whether other relevant parameters need to be provided needs further discussion.

Proposal 2: XR traffic assistance information is recommended to be sent by RRC, MAC CE can be considered in special cases.

Proposal 3: It is necessary for UE to send the end of data burst indication to gNB.

References

[1]  R2_121bis-e Chair Notes EOM