3GPP TSG-RAN WG2 #123	R2-2307092

Toulouse, France, August 2023

Agenda Item:	7.15.6

Source:	OPPO, Nokia, Nokia Shanghai Bell, vivo, NEC, MediaTek Inc., Samsung

Title:	Discussion on ’TX Profile’

Document for:	Discussion, Decision

Introduction

In the last RAN2 meeting, one left issue is on Tx Profile.

Agreements on backward compatibility issue in SL CA (for GC/BC)

1: 	Consider a case that a V2X service which needs to be mapped into multiple carriers while there is at least one legacy UE to receive this V2X service. RAN2 further discuss whether TX profile approach can be supported.

Discussion

In last meeting, the “TX profile” issue has been discussed

FFS on backward compatibility issue in SL CA (for GC/BC)

- No issue with service-to-carrier mapping (P3:4668)

- Need TX profile to handle the compatibility issue (P2:5093)

[NEC]: If we rely on service-to-carrier mapping and the service is supported by SL CA, Rel-16/17 UEs cannot have that service. [Apple]: In LTE time, multiple carriers were already supported before V2X CA was introduced. In NR, only single carrier was considered in Rel-16/17. [Vivo, Huawei]: No issue for backward compatibility and no need for TX profile. [Ericsson, ZTE, Xiaomi]: TX profile is already allowed option to solve the backward compatibility issue. [Apple]: Legacy UE and Rel-18 UE can participate in Rel-18 service that requires SL duplicated transmission. [OPPO]: For SL CA w/o duplicated transmission, it seems clear we can rely on service-to-carrier mapping. With duplicated transmission, it’s not clear how TX profile can make differentiation compared to service-to-carrier mapping. [Nokia]: Service-to-carrier mapping seems baseline. And propose to have further discussion whether we need TX profile in addition. [Apple]: Want to have offline discussion to explain their arguments.

The proposed “TX Profile” aims at supporting a legacy service for the Rel-18 RX UEs with the extended coverage. In Rel-16/17, those services are well received by the Rel-16/17 UEs on a single carrier, i.e., with no carrier aggregation. However, in order to extend the coverage of the legacy services for Rel-18 UEs, it is suggested that PDCP duplication should be used and TX profile indicates PDCP duplication per service. It is also argued that PDCP duplication should be used for that service even if there is one Rel-16/17 RX UE as a receiving UE together with Rel-18 UEs.

The proposed “TX profile” is to support a legacy service for Rel-18 RX UEs with extended coverage while it is sufficient to use a single carrier for Rel-16/17 RX UEs.

As mentioned, the original intention of the proposed “Tx profile” is to enable Rel-18 RX UEs to receive the Rel-16/17 services using PDCP duplication for coverage extenstion. Since the Rel-16/17 UEs function without any performance issue without PDCP duplication, this cannot be considered a backward compatibility issue. When PDCP duplication is not used, there is a possibility that Rel-18 RX UEs located at extended coverage may not be able to receive the Rel-16/17 services. However, this is not a Rel-18 UE specific issue, as the same issue holds for legacy Rel-16/17 UEs if they are located at the extended coverage for the reception of Rel-16/17 services. Therefore, the issue is essentially about improving the performance of Rel-18 UEs, and thus, it can be regarded as an optimization. Some may argue that the proposed TX profile for PDCP duplication is similar to the TX profile indicating DRX. However, these two have completely different intentions. In the case of DRX TX profile, it aims to prevent Rel-18 UEs from using new features, i.e., DRX, that may cause issues with the Rel-16/17 UEs, i.e., backward compatibility. On the other hand, PDCP duplication is intended to enforce the use of new features due to the expected performance degradation if the new feature, i.e., PDCP duplication, is not utilized.

In Rel-18, when PDCP duplication is not used for Rel-16/17 services, there is no backward compatibility issue for Rel-16/17 UEs.

In Rel-18, when PDCP duplication is not used for Rel-16/17 services, the same performance issue is faced by Rel-18 UEs as to legacy Rel-16/17 UEs if the extended coverage is required for the Rel-16/17 services. If the coverage is not expected to be increased for Rel-16/17 services, there should be no performance issue even for Rel-18 UEs.

Given that it is backward compatible to support Rel-16/17 services for Rel-16/17 and Rel-18 UEs without PDCP duplication, the baseline solution should be to rely on the service-to-carrier mapping, meaning that the service supported by Rel-16/17 UEs needs to be mapped to the legacy single sidelink carrier, e.g., the one included in the sl-FreqInfoList-r16.

When there is Rel-18 UEs receiving Rel-16/17 services, it is possible to ensure that there is no backward compatibility issue with Rel-16/17 UEs by mapping Rel-16/17 services to the legacy carrier, i.e., via service-to-carrier mapping.

The question is whether to support Rel-16/17 services with the extended coverage in Rel-18 and how, if needed. In our view, if a different requirement is expected in Rel-18 for the Rel-16/17 service, it should be defined as a new and different Rel-18 service so that it is only supported to the Rel-18 UEs and not supported to the Rel-16/17 UEs. If it is critical for Rel-16/17 and Rel-18 UEs to share the same service, it should be offered as a Rel-16/17 service with a consistent coverage expectation. Supporting the Rel-16/17 service with different coverage based on TX/RX pair would only make the implementation of those services more challenging. This is because the distribution of TX/RX UEs is random and makes it difficult to predict the coverage at which a particular service is being provided. Moreover, TX profile for PDCP duplication does not completely resolve the performance issue for Rel-18 UEs because Rel-18 RX UE still may not be able to receive the Rel-16/17 services with PDCP duplication if the TX UE is a Rel-16/17 UE.

TX profile for PDCP duplication cannot resolve the performance issue completely because Rel-18 RX UE at extended range still cannot receive the Rel-16/17 services with PDCP duplication if Rel-16/17 UE not supporting carrier aggregation is a TX UE.

Supporting Rel-16/17 services with extended coverage would complicate the implementation of the service because the coverage of the service is unpredictable due to the random presence of TX and RX UE.

If a service needs to be supported with extended coverage, this service can be defined as a new service in Rel-18.

With all observations, TX profile for PDCP duplication is neither motivated well nor beneficial for stable support of the service. Therefore, it is proposed not to pursue the “TX profile” solution for PDCP duplication.

RAN2 not pursue the ‘TX Profile’ of PDCP duplication.

Conclusion

We have the following observations:

Observation 1	The proposed “TX profile” is to support a legacy service for Rel-18 RX UEs with extended coverage while it is sufficient to use a single carrier for Rel-16/17 RX UEs.

Observation 2	In Rel-18, when PDCP duplication is not used for Rel-16/17 services, there is no backward compatibility issue for Rel-16/17 UEs.

Observation 3	In Rel-18, when PDCP duplication is not used for Rel-16/17 services, the same performance issue is faced by Rel-18 UEs as to legacy Rel-16/17 UEs if the extended coverage is required for the Rel-16/17 services. If the coverage is not expected to be increased for Rel-16/17 services, there should be no performance issue even for Rel-18 UEs.

Observation 4	When there is Rel-18 UEs receiving Rel-16/17 services, it is possible to ensure that there is no backward compatibility issue with Rel-16/17 UEs by mapping Rel-16/17 services to the legacy carrier, i.e., via service-to-carrier mapping.

Observation 5	TX profile for PDCP duplication cannot resolve the performance issue completely because Rel-18 RX UE at extended range still cannot receive the Rel-16/17 services with PDCP duplication if Rel-16/17 UE not supporting carrier aggregation is a TX UE.

Observation 6	Supporting Rel-16/17 services with extended coverage would complicate the implementation of the service because the coverage of the service is unpredictable due to the random presence of TX and RX UE.

Observation 7	If a service needs to be supported with extended coverage, this service can be defined as a new service in Rel-18.

We have the following proposals:

Proposal 1	RAN2 not pursue the ‘TX Profile’ of PDCP duplication.

Reference

[1] R2-2306545	Report from session on LTE V2X and NR SL	Session chair (Samsung)	Report