3GPP TSG-RAN WG2 Meeting #123	R2-2307146

Toulouse, France, August 21- 25, 2023

Agenda Item:	7.11.2.2

Source:	NEC

Title:	UP issues for multicast in RRC Inactive

Document for:	Discussion, Decision

# Introduction

Following the discussions of UP issues for Multicast in RRC Inactive at RAN2#121bis meeting, the following two issues are postponed:

(1)the UP discussion on L2 operation during RRC state transition until the signaling design of PTM configuration in RRCRelease message is concluded.

(2)the discussion on L2 operation during mobility is postponed to next RAN2 meeting.

This paper discusses L2 operation during mobility for multicast reception and HARQ related issues.

# Discussion

## Multicast reception continuity during mobility

It would be very important to clarify the requirement of reception continuity for the UE receiving multicast in RRC Inactive state, during UE movement. Our current understanding is that this should not be supported with unreasonable cost in terms of network implementation.

Proposal-1: RAN2 to clarify the requirement of reception continuity for the UE receiving multicast in RRC Inactive state, during mobility.

## MRB Configuration during mobility

For the UE receiving multicast in RRC Inactive state, during the UE movement, the same UE may need to update its MRB configuration following the new serving cell in terms of both physical parameters and L2 configuration. We assume that in this case, the UE will flush its HARQ buffer and perform MAC reset to apply the new configuration in the new cell, which means the Multicast reception continuity can not be ensured by L1/HARQ during mobility. Meanwhile, RLC reestablishment is expected for the MRB to receive the multicast in the new cell during inactive state.

The RLC re-establishment for the UE when moving into a new serving will result in more packet loss and longer packet delivery latency. In this case, specific to the Multicast MRB (configured with RLC-UM bearer) for the UE in RRC Inactive, the PDCP PDUs which are not successfully received from source cell are discarded, this is the source of the Packet loss as mentioned.

Proposal-2: For the UE receiving multicast in RRC Inactive state, during the UE movement to a new serving cell, UE flushes its HARQ buffer and performs MAC reset and RLC re-establishment.

## PDCP COUNT continuity during mobility

During the pre-meeting email discussion last meeting, there is a discussion on whether the UE needs to re-initialize the PDCP window each times it moves to another cell. As discussed in the previous section, in case of RLC reestablishment, the possible mechanism to avoid the data loss is at PDCP layer to ensure continuous multicast reception for the UE in RRC Inactive state. However, if the PDCP window is re-initialized, it will cause the data loss at PDCP layer, which means the service continuity for multicast reception cannot be ensured.

In Rel-17 MBS, the PDCP COUNT synchronization among gNBs can be achieved by setting the PDCP COUNT value based on the same MBS QoS Flow’s SN. In addition, the gNBs should also have the same QoS flow to MRB, which is based on network implementation. In order to support the continuous PDCP reception window, it can be expected that the same QoS flow to MRB mapping is applied by all of the cells that support the UE multicast reception during its RRC Inactive state, otherwise the continuous PDCP reception window during UE movement is not possible. In practice, different network implementation and configuration should be allowed in order to offer the flexibility at the network side. However, it may be worthwhile for the cell to explicitly indicate such configuration to the UE. This means if the UE reads the configuration, it will know the necessity to re-initialize its PDCP window for the same multicast service when moving to new serving cell.

Proposal-3: The network can explicitly indicate the MRB configuration to the UE, to help the UE to determine the need to re-initialize its PDCP window (i.e., not support PDCP COUNT continuity).

## Handling on PTM related HARQ timer

In RAN#121b meeting, we had an agreement as below:

On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer.

It is agreed that for multicast reception in RRC_INACTIVE, the UE behavior of DRX operation is based on multicast DRX. But whether UE still needs to start HARQ RTT timer and Re-Tx timer should be further studied.

RAN2 had agreement that HARQ feedback mechanism is not supported for multicast reception in RRC_INACTIVE. But even though HARQ feedback in RRC_INACTIVE is disabled, several companies still think it is beneficial for such UEs to start PTM related HARQ timer to receive possible retransmission data if RRC_CONNECTED UEs for the same multicast session has gave the feedback.

To our understanding, it is true that RRC_INACTIVE UE still can receive possible retransmission data if Re-Tx timer is running. However, such kind of UEs are not aware of whether network would send the retransmission data (i.e., not aware of whether there is RRC_CONNECTED UEs giving NACK feedback to network), but still start its Re-Tx timer if its data is decoded unsuccessfully. It seems like UE needs to “blindly” use the Re-Tx timer to monitor possible retransmission data which is not efficient, unnecessary PDCCH monitoring during Re-Tx timer causes more power consumption. Therefore, it is desirable to disable PTM related HARQ timer for multicast reception in RRC_INACTIVE, which is also a simple solution.

For the spec impact on this solution, according to the current MAC spec (TS 38.321 clause 5.7b), it is limited to “if HARQ feedback is enabled”, since the HARQ feedback is disabled when UE is receiving multicast in RRC_INACTIVE. there is no or minor spec impact on this part.

Proposal-4: the UE does not start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for multicast reception during RRC_INACTIVE.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: RAN2 to clarify the requirement of reception continuity for the UE receiving multicast in RRC Inactive state, during mobility.

Proposal-2: For the UE receiving multicast in RRC Inactive state, during the UE movement to a new serving cell, UE flushes its HARQ buffer and performs MAC reset and RLC re-establishment.

Proposal-3: The network can explicitly indicate the MRB configuration to the UE, to help the UE to determine the need to re-initialize its PDCP window (i.e., not support PDCP COUNT continuity).

Proposal-4: the UE does not start PTM related HARQ RTT timer and DRX Retransmission timer for multicast reception during RRC_INACTIVE.

# Reference

- R2-2212521	Details of multicast reception in RRC INACTIVE

- R2-2303969	Remaining UP issues for multicast reception in RRC_INACTIVE