3GPP TSG-RAN WG2 Meeting #123	R2-2307156

Toulouse, August 21-25, 2023

Agenda Item:	7.5.4.1

Source:	Honor

Title: 	Discussion on BSR enhancements for XR

Document for:	Discussion and decision

1	Introduction

In previous RAN2 meetings, the following agreement for BSR enhancements was achieved in RAN2 #121bis[1]:

- Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

The following agreement for new BSR table was made in RAN2 #122[2]:

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases)

According to the above agreement, the remaining issues will be discussed in this document:

- How the new BSR table looks like?

- How to report the new BSR?

- Whether the delay status report is contained in BSR?

- Discussion

## 2.1 How the new BSR table looks like?

The maximum and minimum values of the new BSR table

In TR 38.838-h00, the packet size of a video frame for single-stream is formulized as following:

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M |
| Max | byte | 150% of M | 109% of M |
| Min | byte | 50% of M | 91% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |

According to above table, packet size of a video frame is determined by frame rate (F) and data rate (R). For frame rate, typical values of 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline.

Observation 1: Packet size of a video frame is determined by frame rate and data rate. Values 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline for frame rate.

For the data rate, some reference values in bit rate have been collected in TR 26.928 [3], some values of different cases and scenarios are still FFS (e.g. for 3D mesh, the detail information of UL bitrate for XR in section 6.3).

The bitrates of 2D and 3D in TR 26.928 can be used for calculating UL packet size of a video frame.

- For 2D Compression Technologies

The following table is provided in TR 26.928:

| # Codec | # Coding performance # (Random-Access) |  | # Targeted bitrate # (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| AVC |  |  | 4k: - Statmux: 20-25 Mbps - CBR: 35 - 50 Mbps 8k: - CBR: 80 - 100 Mbps - High quality: 100 - 150 Mbps [33][34][35] |
| HEVC | -40% vs AVC [33][34][35] | -60% vs AVC [33][34][35] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [33][34][35] |

According to the table, regardless the type of codec and resolution, the value range of bit rate is [10, 150] Mbps. Thus, according to formula 150% of (R×1e6 / F / 8), the maximum packet size can be calculated as 1875000 bytes by using maximum data rate R=150Mbps and the minimum frame rate F=15fps. The minimum packet size is 5208 bytes.

- For 3D and XR Visual Formats

In TR 26.928:

“According to clause 4.2.1, 1k by 1k per eye is a minimum for the signal in the viewport, and for stereoscopic rendering this results in basically a signal for 2k by 1k, typically at a frame rate of 50 or 60fps.With current codecs according to clause 4.5.1, the pure viewport data can be represented with around 4-10 Mbit/s. However, a viewport typically only covers around 100 degree horizontal and 60 degree vertical. Hence, to present a full omnidirectional presentation, about 20 times more data may be necessary, leading to 80 – 200 Mbit/s. Viewport-dependent coding and delivery, in particular tiling, can support to reduce the required bitrates.”

The bitrate can be up to 200 Mbit/s, the maximum packet size can be obtained as 2500000 bytes by using the above parameters.

The information obtained by the above analyses can be summarized as following:

Table 1

| Range of bit rate (Mbps) | Frame rate (fps) | Packet size (bytes) |
| --- | --- | --- |
| [10, 200] | 15, 24, 25, 30, 50, 60, 90, and 120 | [5208(10Mbps,120 fps, 50% of M), 2500000(200Mbps,15fps, 150% of M)] |

Due to there is no more detailed information for UL bit rate in TR 26.928, the maximum of UL bit rate for XR is assumed as 200Mbps. Using the frame rate of 15 fps, the maximum value of packet size can be 2500000 bytes, this value can be used as the maximum value of new BSR tables.

Proposal 1: Taking 2500000 bytes (bitrate = 200Mbps, frame rate = 15 fps) as baseline of the maximum value for the new BSR table.

According to [4], the quantization error of the legacy BSR table is 6.5% across all code points, we then have the following requirement for the new BSR table:

step sizeBmin= (Bmax-Bmin)/256Bmin≤6.5% → Bmin ≥ Bmax1+256 ×6.5%

The Bmin should be more than 141723 bytes (Bmax = 2500000bytes). Thus, the step size of new table is 9215 bytes if liner distribution is used.

For the legacy 8-bit BSR table, the value 141723 bytes is corresponded to index 152 (as shown in table 2). The step size of index 152 is 8643 bytes. Such value is less than 9215 bytes, therefore the most suitable value of Bmin should be more than 141789 bytes (the maximum BS size of index 152), this value can be used as baseline of the minimum value for the new BSR table.

Table 2 8-bits BSR table in TS38.321

Proposal 2: Taking 141789 bytes as baseline of the minimum value for the new BSR table.

## 2.2 How to report the new BSR?

The condition to use new table

According to the analyses of section 2.1, the minimum value of new table cannot be zero since quantization error should be less than legacy one. The step size of the smaller index in legacy BSR table is less than the step size of new BSR table. Thus, for XR services, if buffer size of the LCG is less than minimum value of new table, the legacy BSR table should be used for this LCG.

Proposal 3: The new BSR table should be used for the LCG which buffer size is more than minimum value of the new table. Otherwise, using the legacy BSR table.

How to differ the new index and legacy index of BSR tables

When reporting BSR, UE may report the index of new BSR table or the index of legacy BSR table. However, based on the current BSR format, it is unclear to the network whether the index is from new BSR table or legacy BSR table. It is simple that the different LCIDs are used to identify the MAC CE with new BSR table and the MAC CE with legacy BSR table without introducing new field(s) in BSR.

Proposal 4: Using different LCIDs to identify the MAC CE with new BSR table and the MAC CE with legacy BSR table.

The BSR format for the new BSR table

At the last meeting, RAN2 supported one static BSR table with 8 bits BS field. If only one LCG with available UL data can use the new BSR table, the short BSR format which contains 8 bits BS field should be reported. In current specification, the extended short BSR format for IAB contains 8 bits BS field, however the LCG field is occupied 8-bits to indicate 256 number of LCGs. Since 8 number of LCGs are supported in XR, 3-bits LCG field is enough. If more than one LCG with available UL data can use the new BSR table, the current long BSR format can be reused.

Proposal 5: A new short BSR format with 8-bits BS field and 3-bits LCG field is introduced.

## 2.2 Whether the delay status report is contained in BSR?

In RAN2#120, RAN2 has agreed to report delay information associated with data volume [5].

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

Tracking back to the reason of introducing delay status report (DSR), it is because that with knowing the remaining time of packet the network can achieve more suitable UL scheduling by avoiding discard some packets nearly reaching transmission delay budget. To be sure, these packets are buffered in the UE.

If a new format containing remaining time and data volume information is introduced, many problems should be discussed, such as: 1) the data volume information in BSR and new format may be redundant, 2) the conditions for triggering BSR and new format, 3) the relationship between DSR procedure and BSR procedure etc.

It is a straightforward and simply way to extending BSR to contain remaining time, the current protocol architecture of BSR can be reused.

Proposal 6:  Extending BSR format to contain remaining time.

If remaining time is contained in BSR format, in some times the remaining time will not be reported while buffer size is reported. Therefore, it is needed to introduce an indication to indicate whether the remaining time is reported.

Proposal 7: An indication is introduced in BSR to indicate whether the remaining time is reported.

In RAN2#122, RAN2 has agreed the following about remaining time [2].

- 1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Since the remaining time reflects the transmission urgency of packet, a remaining time threshold should be introduced to trigger delay status reporting. When the remaining time is smaller than such threshold, the delay status reporting can be triggered. As the BSR for XR contains remaining time, when the delay status reporting is triggered, the BSR is also triggered.

Proposal 8:  A high delay status threshold is introduced for triggering delay status report (i.e., BSR with remaining time).

With the P8, when remaining time is smaller than high delay status threshold, the PDCP entity should sent an indication to trigger DSR to MAC entity for informing that there is little delay budget remains. Then, the MAC entity can determine whether to trigger DSR.

Proposal 9: An indicator to trigger DSR (i.e. BSR with remaining time) is sent from PDCP entity to MAC entity when remaining time is less than the high delay status threshold.

For avoiding frequently reporting DSR, a prohibit DSR timer can be introduced to control the frequency of DSR reporting. If prohibit DSR timer is running in MAC, the MAC will not trigger the DSR. Only when a DSR is triggered in MAC layer, the DSR will be reported.

Proposal 10: Introducing a prohibit DSR timer to avoid frequently reporting DSR (i.e. BSR with remaining time):

- When a DSR containing remaining time is transmitted, start or restart the prohibit DSR timer.

- A DSR can be triggered when prohibit DSR timer is not running.

If a DSR is triggered, the MAC entity could detect whether there are available UL-SCH resources. To guarantee the reference time for the remaining time is determined from the point of the first transmission of the information, the MAC layer can instruct transmission opportunity to PDCP entity if UL-SCH resources are available, the PDCP entity may calculate the remaining time based on the PDCP discard timer and sent the remaining time to MAC when receiving. Remaining time can equal to high delay status threshold minus the running value of discard timer.

Proposal 11: The PDCP entity calculates and sends the remaining time to the MAC entity when a transmission opportunity for DSR (i.e. BSR with remaining time) is received.

In current specification, NW configures the LCH(s) with similar QoS/priority into one LCG, thus reporting buffer status per LCG can satisfy the QoS requirements of services. With same assuming, the MAC entity can report the delay status (i.e remaining time) per LCG. As a LCG may contain one or more LCH(s) and each LCH has associated remaining time, the MAC entity can obtain the minimum remaining time from LCH(s) for the LCG.

Proposal 12: The MAC entity reports minimum remaining time per LCG.

3	Conclusions

In this contribution, we discussed the issues related to BSR for XR-specific capacity improvement. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: Packet size of a video frame is determined by frame rate and data rate. Values 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline for frame rate.

Proposal 1: Taking 2500000 bytes (bitrate = 200Mbps, frame rate = 15 fps) as baseline of the maximum value for the new BSR table.

Proposal 2: Taking 141789 bytes as baseline of the minimum value for the new BSR table.

Proposal 3: The new BSR table should be used for the LCG which buffer size is more than minimum value of the new table. Otherwise, using the legacy BSR table.

Proposal 4: Using different LCIDs to identify the MAC CE with new BSR table and the MAC CE with legacy BSR table.

Proposal 5: A new short BSR format with 8-bits BS field and 3-bits LCG field is introduced.

Proposal 6:  Extending BSR format to contain remaining time.

Proposal 7: An indication is introduced in BSR to indicate whether the remaining time is reported.

Proposal 8:  A high delay status threshold is introduced for triggering delay status report (i.e., BSR with remaining time).

Proposal 9: An indicator to trigger DSR (i.e. BSR with remaining time) is sent from PDCP entity to MAC entity when remaining time is less than the high delay status threshold.

Proposal 10: Introducing a prohibit DSR timer to avoid frequently reporting DSR (i.e. BSR with remaining time):

- When a DSR containing remaining time is transmitted, start or restart the prohibit DSR timer.

- A DSR can be triggered when prohibit DSR timer is not running.

Proposal 11: The PDCP entity calculates and sends the remaining time to the MAC entity when a transmission opportunity for DSR (i.e. BSR with remaining time) is received.

Proposal 12: The MAC entity reports minimum remaining time per LCG.

4	References

- RAN2 #121bis-e meeting report.

- RAN2 #122 meeting report.

- 3GPP TR 26.928 V18.0.0 (2023-03) Extended Reality (XR) in 5G

- R2-2304711 BSR and delay status reporting for XR

- RAN2 #120 meeting report.