3GPP TSG-RAN WG2 Meeting #123	 Tdoc R2-2307193

Toulouse, France Aug21  – Aug25, 2023

Agenda Item:	8.7.4

Souce:	MediaTek Inc.

Title:	On Triggering Unchanged PCI for Handover Enhancement in LEO NTN

Document for:		Discussion and decision

# 1 Introduction

The work item objective (WID) for Rel-18 enhancements in mobility aspects has been discussed in WG2 Meeting #122. This contribution carries on the discussion for potential solution details in enhancing NTN Connected mode handover, specifically in the PCI unchanged scenario, with the following agreements [1]:

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e., no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e., no other IE) if the gap is very short/zero).

# 2 Discussion

## 2.1 PCI Unchanged

It has been agreed that hard switching for PCI unchanged under quasi-Earth fixed and same feeder link scenario is supported in Release 18. In this scenario, both the source and target satellites are connected to the same gNB, and all the UEs switch from serving to target satellite, while using the same PCI, as the serving satellite flies out of coverage. The following cell switching flow is proposed, as below:

Timing for the serving cell service stop time (t-Service) or the target cell service start time (t-Servicestart) can be broadcast to the UE to initiate pre-configuration in preparation for the cell switch. This is for the UE to know when the serving cell will disappear, and when to expect the switch to the target satellite. UE pre-configuration parameters may include neighbour satellite orbital parameters, common TA parameters, K-offset, and K-mac for the target satellite. UL/DL data transmission is then suspended in the serving satellite and the switch is trigger based on time – either t-Service or t-Servicestart can be used, or a timer from suspending data transmission.

Proposal 1: A time-based trigger or value i.e., t-Service or t-ServiceStart can be broadcast to the UE.

Proposal 2: UE prepares for target cell switch via pre-configuration, with parameters that may include neighbour satellite orbit parameters, common TA parameters, K-offset, and K-mac of the target satellite.

In this agreed hard switch scenario, there is no overlap between the service and target satellite cell, i.e., only one satellite will be in service at any one time. Hence time-based triggers will determine the service interruption time.

Once the switch is complete, DL synchronization is to be performed for the UE to synchronise with the new satellite and triggers the start of a DL sync timer. If this timer times out, DL synchronization has failed, and UE goes into radio link failure, and performs RRC Re-establishment, with priority to connect to the original cell.

Proposal 3: Once the switch has occurred, DL synchronization is to be performed to all the UEs to the target satellite. This triggers the start of a DL sync timer, where, if synchronization fails and there is a DL sync timeout, UE will go into Radio link failure and performs RRC Re-establishment, with priority to connect to the original cell.

# 4 Conclusions

Proposal 1: A time-based trigger or value i.e., t-Service or t-ServiceStart can be broadcast to the UE.

Proposal 2: UE prepares for target cell switch via pre-configuration, with parameters that may include neighbour satellite orbit parameters, common TA parameters, K-offset, and K-mac of the target satellite.

Proposal 3: Once the switch has occurred, DL synchronization is to be performed to all the UEs to the target satellite. This triggers the start of a DL sync timer, where, if synchronization fails and there is a DL sync timeout, UE will go into Radio link failure and performs RRC Re-establishment, with priority to connect to the original cell.

# 5 References

[1] R2-2306541 – Report from Break-out session on NR-NTN and IoT NTN, May 2023