3GPP TSG-RAN WG2 Meeting #123                                                                   R2-2307210

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Discussion on Selective Activation of Cell Groups in NR-DC

Agenda Item:	7.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, NR-DC with selective activation cell of groups related issues were discussed and the following agreements were achieved [1].

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

Rapporteur take initiative on naming offline

Terminology is “Subsequent CPAC”

In this contribution, in section 2, we will discuss the open issues. The proposals are summarized in Section 3.

Discussion

Release of the Subsequent CPAC Configuration

In RAN2#121 meeting, whether the Subsequent CPAC configuration should be maintained or released after PCell change was discussed and achieved the agreement below.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

As agreed above (highlighted in yellow), the UE should release the Subsequent CPAC configuration after PCell change at least for inter MN PCell change. The other cases will be discussed in this section.

- Intra MN case

For intra MN PCell change, considering if PCell change occurs, the coverage of the source PCell and the target PCell will be different, and we can assume the candidate SNs of the stored Subsequent CPAC configuration configured via source PCell should have overlapping coverage with the source PCell, but they may not have overlapping coverage with the new target PCell, i.e. it is not ensured that the candidate PSCells (configured before PCell change) selected for UE could be configured with the new target PCell as DC after PCell change, due to no overlapping coverage between the new PCell and the candidate SCG. And, the execution conditions of candidate PSCells are configured based on the measConfig configured by source PCell, after PCell change, the execution conditions of candidate PSCells need to be updated as the measConfig may be changed by target PCell. Therefore, we prefer to release the Subsequent CPAC configuration when intra MN PCell change occurs.

Proposal 1: The Subsequent CPAC configuration should be released after PCell change in intra-MN case.

- Via explicit indication or autonomously

As the agreement highlighted in green, in case of inter-MN PCell change, the UE releases the Subsequent CPAC configuration by receiving explicit indication from network. We understand if P1 is agreed, it seems no need to release the Subsequent CPAC configuration by explicit indication from network. The main considerations are, if the explicit indication is conveyed by the handover command, the source MN needs to inform the target MN the Subsequent CPAC configuration which may introduce the interface impact and, furthermore, for intra-SN PSCell change which scenario has been supported in previous meeting, the MN cannot know whether the SN initiated intra-SN Subsequent CPAC is configured for the UE, it means the source MN couldn’t inform the target MN the configuration of SN initiated intra-SN Subsequent CPAC which will cause the UE will not release the Subsequent CPAC configuration as no explicit indication from network. Considering the potential complexity of UE releases the Subsequent CPAC configuration by explicit indication from network, for simplicity, if P1 is agreed, due to it is a mandatory UE behaviour upon PCell change, we prefer UE to release Subsequent CPAC configuration automatically upon PCell change (no matter inter MN PCell change or intra MN PCell change) occurs. If P1 is not agreed, the explicit indication is needed as the UE cannot get to know the PCell change is inter MN PCell change or intra MN PCell change.

Proposal 2: If P1 is agreed, the Subsequent CPAC configuration is released autonomously by UE after PCell change, i.e., no explicit indication from network is needed.

- Other cases

For the FFS (highlighted in blue), besides PCell change, for which case the Subsequent CPAC configuration needs to be released should be discussed. We understand the following three cases can be considered.

Case 1: RRC re-establishment procedure initiation

Case 2: going to RRC_INACTIVE state

Case 3: going to RRC_IDLE state

For case 1 and case 2, considering that it is with high probability that the target cell for RRC re-establishment and resume is not the previous PCell, as above discussion the candidate SNs may have no overlapping coverage with the target cell, they are not suitable to be as candidate SCG for the new PCell. Furthermore the MN security key will be changed in case 1 and case 2 which causes the SN security key also needs to be updated. If the Subsequent CPAC configuration is kept for case 1 and case 2, inter-node message and RRC message are needed to update the Subsequent CPAC configuration which will lead in complexity and signalling overhead. In addition, for case 2, when UE triggers RRC resume procedure is not predicted by NW, it will bring resource waste if the Subsequent CPAC configuration is kept by the UE, due to the candidate SNs should reserve the resource for the UE. Therefore, it seems to be reasonable to release Subsequent CPAC configuration in case 1 and case 2. For case 3, when UE going to RRC_IDLE state, it is nature to release Subsequent CPAC configuration, which is similar as R16/R17 CPAC configuration is released when UE going to RRC_IDLE.

Proposal 3: The Subsequent CPAC configuration is also released autonomously by UE in the following cases:

- RRC re-establishment

- state transition to RRC_INACTIVE state

- state transition to RRC_IDLE state

In addition, RAN2 also consider whether the Subsequent CPAC configuration needs to be released or not in the following cases.

Case 4: SCG release

RAN2 has agreed/assumed that CPA is supported in Subsequent CPAC and a CPA conditional configuration can be used for CPC (but with different triggering conditions). Based on agreements, if the UE has stored Subsequent CPA configuration for subsequent CPC while connecting to a PSCell, if SCG is released, the UE can continue the execution condition evaluation for PSCell addition using the stored Subsequent CPA configuration after SCG release. Therefore, for case 4, we think the UE can keep the Subsequent CPAC configuration upon SCG release.

Proposal 4: The Subsequent CPAC configuration is not released by UE automatically in case of SCG release.

Case 5: SCG deactivation

In R17 CPAC discussion, it is agreed that UE is not required to support the joint configuration of R17 CPAC and R17 SCG deactivation, and this is captured in spec as follow:

scg-State Indicates that the SCG is in deactivated state. This field is not used - in an RRCReconfiguration message received: - within mrdc-SecondaryCellGroup, or - in an E-UTRA RRCConnectionReconfiguration message, or - in an E-UTRA RRCConnectionResume message or - in an RRCReconfiguration message received via SRB3, except if the RRCReconfiguration message is included in DLInformationTransferMRDC. The field is absent if CPA or CPC is configured for the UE, or if the RRCReconfiguration message is contained in CondRRCReconfig.

Based on yellow highlighted above, it means the network will not configure the SCG deactivation to UE when the UE is configured with CPAC configuration. For R18 Subsequent CPAC, the same principle should be followed, i.e., the network will not configure the SCG deactivation to UE if the UE is configured with Subsequent CPAC, so, for case 5, we think it is invalid case as the network will make sure that the Subsequent CPAC configuration will be released before deactivating SCG.

Proposal 5: The co-existence between legacy SCG deactivation and subsequent CPAC is not supported.

Security aspect

After RAN2#122 meeting, RAN2 received the reply LS from SA3 [2] about the security issue for Subsequent CPAC, it states as follow.

Following is the solution details from security perspective: 1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. 2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

According to the description above, multiple sk-counters configured for each SN is provided for UE, the UE uses the new sk-counter upon every SN change. We understand this solution could solve the security issue in case of the UE goes back to the previous accessed PSCell. However, a question proposed is that the UE may only know the PSCell change but not SN change. For address the issue, how to enable UE to identify SN change needs to be discussed, some efforts need to be done for it in RAN2. Currently, the sk-counter is provided per SCG configuration i.e., per PSCell configuration, if the SN Counter values are provided for each SN as SA3 LS, RAN2 should discuss the signalling structure to support the configuration and the UE behaviour to use the sk-counter. Considering the limited time for R18 mob discussion and the remaining issues for each topic, it is better to avoid the discussion, the simplest solution is that the UE uses the new sk-counter upon every PSCell change and the sk-counters is provided per candidate PSCell, it is also fulfilled SA3 requirement.

Considering the current stage of the Subsequent CPAC discussion, for simplicity, we propose,

Proposal 6: Multiple sk-counters are configured for each candidate PSCell for Subsequent CPAC, and UE uses one unused sk-counter upon PSCell change.

Execution conditions

- Execution conditions update

In [3], it was discussed that for SN initiated inter-SN Subsequent CPAC, whether the execution conditions for the procedure may be updated by the network upon a PSCell change, but is still FFS.

Proposal 6. For SN initiated inter-SN SCG selective activation, it is FFS whether the execution conditions for the procedure may be updated by the network upon a PSCell change, and the network side procedure to update the execution conditions.

Considering that the intention of this objective is to allow the UE perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network, execution conditions update after each SCG change should be avoided. It is optional to network to update the execution conditions after SCG change, if needed.

Proposal 7: Execution conditions update via RRC reconfiguration after each SCG change is not required.

- Execution conditions configuration

In RAN2#121bis-e meeting, how to configure the execution conditions for each candidate SCG for Subsequent CPAC was discussed based on [3] and the agreements below were achieved.

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: <……omit……> - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported.

As descripted above, it was agreed that for MN initiated procedure, execution conditions based on event A4 are supported. For SN initiated procedure, execution conditions based on event A3/A5 are supported.

As A4 only takes the quality of the candidate cell into consideration, some concerns are arouse that, from the movement point view, both the quality of the serving cell and the candidate cell should be taken into consideration, otherwise the unnecessary PSCell change may occur, e.g. the serving PSCell is in good condition, but the candidate cell fulfilled the A4 condition, the UE will execute the PSCell change even though the UE could keep work on the serving cell well.

Observation 1: Only A4 event used may cause unnecessary PSCell change if the signal quality of serving PSCell is still good.

For A3/A5 event, for MN initiated procedure, the serving cell refers to the PCell not the PSCell, which is not logical to determine the PSCell change based on the PCell quality. Additional specification work is needed. In R17 CPAC the similar issue was discussed, which was not agreed, we think the same conclusion should be followed.

Even the A3/A5 is used for serving PSCell, another issue should be discussed. If A3/A5 event is used, if each candidate cell is configured with one execution condition with A3/A5, that means the unified threshold configured for Subsequent CPAC i.e., when any of the candidate PSCell is as serving PSCell, the unified threshold is used for one candidate cell. It is not reasonable to use the unified threshold for different serving PSCell, e.g. for event A3 for candidate SCG1, no matter the serving PSCell is PSCell 0 or PSCell2, the same a3-Offset is configured for candidate SCG1; and for event A5, the same a5-Threshold1 is used for serving PSCell, no matter the serving PSCell is PSCell 0 or PSCell2.

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

Observation 2: If one execution condition associated with A3/A5 is configured for each candidate SCG, the unified threshold will be used for different serving PSCell which is not reasonable.

To solve this problem, one candidate solution is to configure the execution condition list of each candidate PSCell based on each candidate cell as serving PSCell, but it will increase configuration complexity and signaling overhead.

Figure 1

Take figure 1 as example, 3 candidate SCG are configured for UE, considering the serving PSCell change, each candidate SCG should be configured with different execution condition according to the serving PSCell, e.g. take candidate cell 1 as example, condition21 is used when the cell 2 as serving PSCell, condition01 is the condition used when UE is not configured with SCG. From the Figure 1 we can find, if UE is configured with N candidate SCG, N*N execution conditions will be configured for UE for all the candidate cells.

Observation 3: If A3/A5 event is used, for each candidate PSCell an execution condition list may be needed considering each candidate PSCell as serving PSCell, which will increase configuration complexity and signaling overhead.

In addition, if one or multiple new candidate PSCell(s) are considered to be added as candidate SCG configurations, the execution conditions list will need to be updated to add the execution conditions for each configured candidate PSCell considering the new added candidate PSCell(s) as serving PSCell. The update procedure of Subsequent CPAC configuration will be complex and introduce much interface impacts. For example, if 4 candidate SCG are configured for the UE, each candidate SCG should be configured with 4 execution conditions, and if new candidate SCG is added, the NW should reconfigure the UE with new execution condition for each configured candidate SCG (5 execution condition for each candidate SCG). From the NW implementation perspective, complex internode interaction should be avoided.

Observation 4: If each candidate cell is configured with multiple A3/A5 events for each possible serving PSCell, the update procedure of Subsequent CPAC configuration will be complex and introduce much interface impacts which should be avoided.

Proposal 8: Additional inter-node interaction due to execution condition configuration/update leaded in by candidate SCG addition/release should be avoided.

In RAN2#121 meeting, it has been assumed that a CPA conditional configuration can be used for CPC. If A4 event is used for Subsequent CPAC, the execution conditions of conditional PSCell addition can be used for subsequent conditional PSCell change as A4 event can be used for both R17 CPA and R17 MN-initiated inter-SN CPC. However, if A3/A5 event is used for Subsequent CPAC, maybe two execution conditions for one candidate PSCell will be needed, one is for conditional PSCell addition, and another is for subsequent conditional PSCell change.

Observation 5: A3/A5 event is not applicable for Subsequent CPAC when UE is not configured with SCG. Additional A4 execution condition for one candidate PSCell is needed if A3/A5 used as execution condition.

Considering above, one compromise solution can be considered. In addition to the A4 configured for each candidate cell, in order to take the quality of the serving PSCell into consideration, a leave condition (e.g. A2 event like) can be configured for serving PSCell and candidate PSCell. When it configured for candidate PSCell, the UE evaluates it only when this cell is as serving PSCell.  When both the leaving condition for the serving PSCell and execution condition of A4 configured for one candidate cell are fulfilled, the UE could perform PSCell change to the candidate cell. And then continue to evaluate the leaving condition for the new PSCell, and the execution condition of A4 of each candidate cell.

With this compromise approach, in case of adding a SCG, the UE can perform PSCell addition only when the execution condition of A4 of candidate cell is met.

Figure 2 below descripts the framework for this solution,

Figure 2

As Figure 2 shows, each candidate cell is configured with one execution condition event A4, and each serving cell is configured with one leaving condition e.g. event A2 like. Each arrow in the figure represents one potential case. E.g when cell 1 as serving cell (highlighted by green), and the UE may perform PSCell change from cell 1 to cell 2 or cell 3 as the arrow with green. Hence the UE should evaluate the leaving condition 1configured for cell1 and the execution condition 2 and execution condition 3 configured for cell2 and cell3, when the leaving condition 1 is fulfilled, it means the quality of the serving PSCell is bad, UE can trigger PSCell change procedure, when one of the execution condition for cell2 and cell3 is fulfilled, UE could trigger PSCell change to the target cell. E.g., cell2. Then the UE will continue to evaluate leaving condition 2 and execution condition1 and execution condition 3.

If no serving PSCell is configured, the UE may perform PSCell addition to cell1/2/3 (arrows with red), so the UE should evaluate the execution condition for cell1/2/3.

This solution could avoid complex inter-node interaction, due to the leaving condition can be configured by serving cell and the candidate cell itself, it doesn’t need to be updated due to the addition/release of the candidate cells.

Proposal 9a: For Subsequent CPAC, in addition to the execution condition of event A4 for each candidate PSCell, a leaving condition (A2 event like) for serving PSCell is configured.

Proposal 9b: UE triggers the PSCell change if both the leaving condition (A2 event like) for the serving PSCell, and the execution condition A4 for one candidate PSCell are fulfilled; UE should trigger PSCell addition if the execution condition A4 for one candidate PSCell is fulfilled.

Delta configuration

In previous meeting, delta configuration related issues were discussed and achieved some agreements, but there are still some FFS issues are left. In current 38.331 running CR for Subsequent CPAC [4], some agreements about reference configuration and complete configuration were captured, but some companies proposed that we can wait more progress on LTM about reference configuration related issues and corresponding spec impacts. Considering that RAN2 has agreed that for the reference configuration for Subsequent CPAC, aim at following similar design as LTM, so we propose to postpone the reference configuration related issues discussion until the clear solution and spec impact for LTM reference configuration are known.

Proposal 10: RAN2 postpones the discussion about reference configuration for Subsequent CPAC until more progress is made on LTM.

Conclusion

Based on the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: Only A4 event used may cause unnecessary PSCell change if the signal quality of serving PSCell is still good.

Observation 2: If one execution condition associated with A3/A5 is configured for each candidate SCG, the unified threshold will be used for different serving PSCell which is not reasonable.

Observation 3: If A3/A5 event is used, for each candidate PSCell an execution condition list may be needed considering each candidate PSCell as serving PSCell, which will increase configuration complexity and signaling overhead.

Observation 4: If each candidate cell is configured with multiple A3/A5 events for each possible serving PSCell, the update procedure of Subsequent CPAC configuration will be complex and introduce much interface impacts which should be avoided.

Observation 5: A3/A5 event is not applicable for Subsequent CPAC when UE is not configured with SCG. Additional A4 execution condition for one candidate PSCell is needed if A3/A5 used as execution condition.

Release of the Subsequent CPAC Configuration

Proposal 1: The Subsequent CPAC configuration should be released after PCell change in intra-MN case.

Proposal 2: If P1 is agreed, the Subsequent CPAC configuration is released autonomously by UE after PCell change, i.e., no explicit indication from network is needed.

Proposal 3: The Subsequent CPAC configuration is also released autonomously by UE in the following cases:

- RRC re-establishment

- state transition to RRC_INACTIVE state

- state transition to RRC_IDLE state

Proposal 4: The Subsequent CPAC configuration is not released by UE automatically in case of SCG release.

Proposal 5: The co-existence between legacy SCG deactivation and subsequent CPAC is not supported.

Security aspect

Proposal 6: Multiple sk-counters are configured for each candidate PSCell for Subsequent CPAC, and UE uses one unused sk-counter upon PSCell change.

Execution conditions

Proposal 7: Execution conditions update via RRC reconfiguration after each SCG change is not required.

Proposal 8: Additional inter-node interaction due to execution condition configuration/update leaded in by candidate SCG addition/release should be avoided.

Proposal 9a: For Subsequent CPAC, in addition to the execution condition of event A4 for each candidate PSCell, a leaving condition (A2 event like) for serving PSCell is configured.

Proposal 9b: UE triggers the PSCell change if both the leaving condition (A2 event like) for the serving PSCell, and the execution condition A4 for one candidate PSCell are fulfilled; UE should trigger PSCell addition if the execution condition A4 for one candidate PSCell is fulfilled.

Delta configuration

Proposal 10: RAN2 postpones the discussion about reference configuration for Subsequent CPAC until more progress is made on LTM.

Reference

[1] Draft_R2_122_meeting_report_v2,  https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_122/Report

[2] S3-233200, LS on Security Solution for Selective SCG

[3] R2-2302934, Report of [Post121][044][eMob] SCG Selective Activation in NR-DC Signalling interaction,	Qualcomm

[4] [Post122][056][Mob18] 38.331 Running CR for selective activation of SCGs for NR-DC (OPPO)