3GPP TSG-RAN WG2 Meeting #123                                                                    R2-2307222

Toulouse, FR, Aug 21 – 25, 2023

Source:	CATT

Title:	On scenarios for LTM

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

Introduction

In RAN#100, the following agreement on the scope of the R18 LTM is made.

- Revise Objective#1 to prioritize MCG for Rel-18 LTM (for all RAN WGs)

In this contribution, we continue to discuss the supported scenarios for LTM in R18 based on the updated WID [1].

Discussion

On consideration of the supported scenarios for MCG LTM

On configuration of the MCG LTM, it may include the following scenarios:

- Scenario #1: the MCG LTM is configured in NR-DC, and after performing the MCG LTM, it is still in NR-DC. E.g. the LTM configurations may also include or not include the SCG configuration;

- Scenario #2: the MCG LTM is configured in non-DC, and after performing the MCG LTM, it is in NR-DC. E.g. the LTM configurations also include the SCG configuration;

- Scenario #3: the MCG LTM is configured in NR-DC, and after performing the MCG LTM, it is in non-DC. E.g. the LTM configurations include the release indication for the SCG;

- Scenario #4: the MCG LTM is configured in non-DC, and after performing the MCG LTM, it is in non-DC. E.g. the LTM configuration does not include any SCG configuration.

The following description is extracted from the R18 Mob WID [1].

- Standalone, CA and NR-DC case with serving cell change within one CG, prioritizing MCG

For NR-DC case, the intention of the R18 LTM is to do serving cell change within one CG. However, the scenario #2 and scenario#3 involves the other CG.

Besides, supporting the scenario #1, scenario #2 and scenario #3 requires more workload and spec impacts, e.g., for these scenarios, bearer type or the bearer termination may change, which may result in the PDCP re-establishment of MCG and/or SCG. Further, configuration of the LTM in these scenarios also requires the Xn interactions between MN and SN node.

Observation 1: Supporting the LTM in NR-DC case leads to more workload.

Considering of the R18 time budget, it is proposed that only scenario 4 should be supported for R18 LTM. That is, NW should ensure the SCG is released when configuring the LTM, i.e., it can be released before the RRC message including the LTM configuration, or within the same RRC message including the LTM configuration.

Proposal 1: The LTM is only supported to be configured for non-DC scenario, e.g., NW should release the SCG before or within the RRC message including the LTM configuration.

On consideration for SCells configured in the LTM candidate configuration

Similar like legacy mobility scenario, the R18 LTM supports CA scenario. That is at least the SpCell should be changed with or without the SCell change. That's the reason why the following agreements were made in RAN2#119bise [1].

- L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change

Observation 2: Similar like legacy (conditional) handover or PSCell change, the co-existence of LTM and CA is supported.

And recall measurement used for the legacy (conditional) handover or PSCell change, only the measurement quantities of the SpCell should be considered so as to trigger the reconfiguration with sync, e.g., for CHO/CPAC, the execution condition is configured for the SpCell only. As for SCell, it is not that urgent as the SpCell. And the addition /release/ activation /deactivation of the SCell depend on the service and data transmission. So in general, we think the legacy SCell operations within the reconfiguration with sync for PCell or PSCell is sufficient.

Observation 3: Instead of triggered by mobility, the SCell change mainly depends on the service and data transmission.

Based on above analysis, the LTM will not specially consider the SCell change only case, so from the mobility perspective, there is no motivation to perform any LTM specific L1 measurement for LTM candidate SCell. I.e., NW will only consider the SpCell measurement to trigger the LTM cell switch.

Observation 4: For mobility (decision to trigger LTM) perspective, no need to consider the L1 measurement results of SCells within the LTM candidate configurations.

However, except for mobility, it is still FFS whether the LTM candidate SCell should be specially considered from the perspective for the data transmission interruption perspective. But to be noted, this is only restricted in the SCell in activated state, since for deactivated SCell, anyway no data transmission is performed.

In legacy, after handover or PSCell change, UE may performs the DL sync of the SCell, and perform the UL sync with the SCell via the PDCCH ordered RACH procedure triggered by the target SpCell (for SCells in the STAG). UE perform L1 measurement, and then report the corresponding reporting results to NW after the handover or PSCell change if the SCell is configured to be activated state. Only after obtain the provided L1 measurement, the NW can then decide the beam and scheduling format to be used for SCell, so as to continue the data transmission on SCell.

Observation 5: In legacy HO/PSCell change, the possible DL sync/UL sync on SCell (in STAG), and the beam indication for SCell is performed after the successful HO/PSCell change.

But for R18 LTM, it is still FFS on whether to support the early DL/UL sync on SCell (in STAG), and whether the beam indication for SCell contains in cell switch LTM command. From the data transmission perspective, we agree it may be beneficial to support such special consideration on LTM candidate SCell. But this will definitely bring more workload and also impose more complexity to UE/NW, i.e., on L1 measurement RS configuration, L1 measurement reporting, and also the repeated CU-DU interaction (each candidate DU need to generate the corresponding L1 measurement reporting based on the state of the LTM candidate SCell, serving DU need to decide whether to indicate the TCI state for LTM candidate SCell based on state of the LTM candidate SCell).

Observation 6: Even supporting the early DL/UL sync and the TCI state activation/beam indication for LTM candidate SCell before the LTM cell switch may be beneficial from data transmission perspective, but it will also bring more complexity for UE/NW, and also more specification impacts.

Besides, for the early TA acquisition, it is enabled via the early RACH procedure. But performing the early RACH on LTM candidate cell will also cause the interruption for data transmission. Such interruption may be quite large if UE is required to perform the early TA acquisition on both SpCell and SCell.

Observation 7: Early TA acquisition on LTM candidate SCell in STAG will cause interruption on serving cell’s data transmission.

Based on above analysis, especially considering the limited time for R18, it is proposed that the R18 LTM mainly focus on LTM candidate SpCell, that is the LTM candidate SCell can be configured, but no additional enhancement is supported.

Proposal 2: For LTM in CA scenario, no special handling applies to the SCells configured in the LTM candidate configuration, i.e., the following is not applied to the SCells:

- L1 measurement configuration and reporting before LTM cell switch;

- Beam indication upon LTM cell switch;

- TCI activation before LTM cell switch;

- Early DL sync;

- Early TA acquisition.

On consideration for SCG LTM

In the beginning of the R18, it was agreed to support the SCG initiated LTM without involvement of MN in this release. However, given the heavy workload on R18 mobility, the discussion and the agreements on SCG LTM is quite limited. Besides, it is observed that there are still many open issues on supporting the SCG LTM as we listed in the following:

- FFS signaling on SCG LTM configuration;

- FFS the successful completion of the SCG LTM, which may be different with the MCG LTM case, since the RRCReconfigurationComplete is sent to the MCG and then forward to the SCG if the LTM configuration is configured via SRB1;

- FFS on the coexistence of the SCG LTM and deactivated SCG;

- FFS on the failure handling for SCG LTM;

- FFS on the coexistence of SCG LTM and MCG LTM, e.g. the maximum number of candidates coordination between MN and SN;

- FFS on UE handling of the SCG LTM upon handover, PSCell change, SCG release, entering RRC_IDLE/INACTIVE, RRC re-establishment.

Besides, it was agreed to prioritize the MCG discussion in the RAN#100 meeting [1]. However, according to the post email discussion on the open issues for LTM, many open issues are unsolved even for the MCG LTM case. So it is proposed not to support the SCG LTM in R18 mobility.

Proposal 3: SCG LTM is not supported for R18 LTM.

Proposal 4: Remove the support of SCG LTM from the running CR [2].

Conclusion

Based on the previous analysis in section 2, our main contributions are summarized as follows:

Observation 1: Supporting the LTM in NR-DC case leads to more workload.

Observation 2: Similar like legacy (conditional) handover or PSCell change, the co-existence of LTM and CA is supported.

Observation 3: Instead of triggered by mobility, the SCell change mainly depends on the service and data transmission.

Observation 4: For mobility (decision to trigger LTM) perspective, no need to consider the L1 measurement results of SCells within the LTM candidate configurations.

Observation 5: In legacy HO/PSCell change, the possible DL sync/UL sync on SCell (in STAG), and the beam indication for SCell is performed after the successful HO/PSCell change.

Observation 6: Even supporting the early DL/UL sync and the TCI state activation/beam indication for LTM candidate SCell before the LTM cell switch may be beneficial from data transmission perspective, but it will also bring more complexity for UE/NW, and also more specification impacts.

Observation 7: Early TA acquisition on LTM candidate SCell in STAG will cause interruption on serving cell’s data transmission.

On consideration of the supported scenarios for MCG LTM

Proposal 1: The LTM is only supported to be configured for non-DC scenario, e.g., NW should release the SCG before or within the RRC message including the LTM configuration.

On consideration for SCells configured in the LTM candidate configuration

Proposal 2: For LTM in CA scenario, no special handling applies to the SCells configured in the LTM candidate configuration, i.e., the following is not applied to the SCells:

- L1 measurement configuration and reporting before LTM cell switch;

- Beam indication upon LTM cell switch;

- TCI activation before LTM cell switch;

- Early DL sync;

- Early TA acquisition.

On consideration for SCG LTM

Proposal 3: SCG LTM is not supported for R18 LTM.

Proposal 4: Remove the support of SCG LTM from the running CR [2].

Reference

[1] RP-231475 R18_MobEnh_WID_Rev_RAN100.

[2] R2-2308435, RRC running CR for LTM.