3GPP TSG-RAN WG2 Meeting #123                                                                    R2-2307223

Toulouse, FR, Aug 21 – 25, 2023

Source:	CATT

Title:	Discussion on RRC Aspects for LTM

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the remaining issues on RRC aspects for LTM, including:

- On LTM configuration and UE behavior upon LTM cell switch;

- On provision of the RACH configuration for early sync;

- On handling of LTM configuration upon other procedure (e.g., reconfiguration with sync) ;

- On supervision timer for LTM cell switch;

- On failure handling for LTM;

Besides, as what is proposed in our contribution [1], the SCG initiated LTM is not considered in R18. So this contribution only focus on the MCG initiated LTM case.

Discussion

On LTM configuration and UE behavior upon LTM cell switch

In RAN2#121bis [2], it was agreed that the generated complete candidate configuration should be complete configuration. And upon execution of the LTM, it is something like the full configuration procedure, that is, UE release the current configuration, and then applies the corresponding configuration.

- Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

- Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch.

In legacy, the complete configuration means all dedicated configurations should be included within the RRCReconfiguration message, regardless whether the configurations are same or not with the UE’s current serving cell configuration. In general, we think similar rules apply for the R18 LTM.

Besides, in RAN2#120 [3], it was agreed that the RB configuration can be reconfigured for UE, and correspondingly the RLC configurations can also be reconfigured.

- The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

Observation 1: For R18 LTM, based on the agreements, it is feasible that the RadioBearerConfig, and the RLC configuration can be different before and after LTM cell switch.

According to the running CR [4], the RadioBearerConfig and the RLC configuration will not be released, we understand the motivation is to avoid the RLC reestablishment and PDCP data recovery based on NW indication (intra-DU case), since the release of these configuration will lead to the release the RLC/PDCP entities. However, whether this is feasible, depend on what configuration can be reconfigured, e.g., the number of the RLC bear/RB. Take DRB as example, if there is DRB 1/2/3 configured in the serving cell, while there is only DRB 1/2 in the complete candidate LTM configuration. Then after performing cell switch based on the running CR, there is DRB1/2/3 in UE side. While from NW perspective, NW only expects to configure the DRB 1/2, this will cause some misalignment between UE and NW side.

Observation 2: There may be misalignment between UE and NW side if the RadioBearerConfig and the RLC configuration are not released upon performing cell switch.

Besides, there are also some configurations on RLC and PDCP configuration, i.e., SN length, reconfiguring of such configuration will definitely cause the RLC reestablishment.

To solve the issue, the following potential options can be considered:

- Option 1:

NW ensure that the radio bearer related configuration and also the RLC related configuration provided within the LTM configuration is the same as UE’s current configuration;

- Option 2:

NW always releases the RadioBearerConfig and RLC configuration upon performing LTM cell switch;

- Option 3:

NW ensure the configuration, e.g., the SN length of RLC, will not reconfigured for LTM, and add restrictions in the procedure of DRB/RLC addition/modification, i.e., for the part that is in the UE’s current configuration but not in the complete LTM configuration, UE shall release these part of configuration.

For option 2, releasing the configuration will correspondingly lead to RLC establishment, and even PDCP re-establishment, so option 2 should be firstly precluded.

As for option 3, it can work, but this requires RAN2 to check what configurations of the RLC/RB can be reconfigured for LTM, and also additional requirements on UE behavior when performing the DRB/RLC addition/modification.

LTM is triggered for mobility, but not due to the service type/QoS. However the RadioBearerConfig and RLC bearer configuration depends on the service, so when configuring the LTM, there is no motivation for NW to reconfigure the bearer related configuration. From this perspective, option 1 is feasible. Further if we go with option 1, there is only a little spec impact.

Observation 3: LTM is initiated for mobility, i.e., to minimize the mobility interruption latency, but not for service type/QoS change.

Proposal 1: When configuring LTM, NW ensures the radio bearer related configuration and also the RLC related configuration within the complete LTM candidate configuration is the same as UE’s current configuration.

On provision of the RACH configuration for early sync

According to the TS38.331 [5], the frequency location of the RACH occasion is configured with respective to the start frequency of the BWP where the RACH resource is configured.

RACH-ConfigGeneric ::= SEQUENCE { prach-ConfigurationIndex INTEGER (0..255), msg1-FDM ENUMERATED {one, two, four, eight}, msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1), ---------------------skip unrelated part------------------ } msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2)..

Observation 4: The frequency resource of the RACH occasion is configured with respective to the frequency location of the BWP.

For R18 LTM, to support early UL sync, it was already agreed to pre-configure the RACH resource for candidate cells to UE. However, as analyzed above, to enable the UE to decide the frequency resource of the RACH resource without pre-decoding the LTM candidate cell configuration before LTM cell switch, at least the frequency location of the BWP should also be provided.  One may argue that NW can directly provide the IE BWP-Uplink where the RACH resource is configured to UE, but this is a heavy signaling overhead, since only frequency information are needed.

Observation 5: Providing the corresponding BWP configuration of each candidate cell, i.e., BWP-Uplink, along with the RACH resource of each candidate cell, leads to heavy signaling overhead.

The following options can be considered to provide the start frequency location of the RACH-resource:

- Option 1:

NW provides the absolute frequency of lowest RACH occasion in frequency domain, i.e., the ARFCN-ValueNR of the lowest RACH resource, and the SCS;

- Option 2:

Reuse the configuration of RACH resource, NW additionally provides the generic parameters of the BWP where the RACH resource is configured and also the basic parameters of an uplink carrier and transmission corresponding to the BWP thereon, e.g., by including the IE BWP and the IE absoluteFrequencyPointA, offsetToCarrier corresponding to the subcarrierSpacing as indicated within the IE BWP;

- Option 3:

Reuse the configuration of RACH resource, NW additionally provides the absolute frequency of the BWP where the RACH resource is configured, e.g., the ARFCN-ValueNR of lowest BWP in frequency domain, and the SCS

Proposal 2: To enable UE to decide the frequency resource of the RACH occasion, the following candidate options can be considered.

- Option 1: NW provides the absolute frequency of lowest RACH occasion in frequency domain;

- Option 2: NW provides the generic parameters of the BWP where the RACH resource is configured and also the IE absoluteFrequencyPointA and offsetToCarrier corresponding to the subcarrierSpacing as indicated within the IE BWP;

- Option 3: NW provides the absolute frequency of the lowest BWP in frequency domain where the RACH resource is configured, and the SCS.

On handling of LTM configuration upon the execution of other procedure

In this clause, we mainly discuss on how to handle the LTM configuration upon successful legacy PCell handover, PSCell change/addition, SCG release, re-establishment, and upon going to IDLE or going to INACTIVE.

For the case of legacy PCell handover, after the PCell is changed, the serving CU may be changed or not, and the configured LTM configuration may be invalid. Considering the LTM is supported under intra-CU case, if inter-CU PCell change occurs, the LTM configuration will be invalid, but for intra-DU, it is up to the NW decides whether the LTM configuration is supported. Anyway how to handle the LTM configuration can be left to NW implementation, e.g., NW can release or modify the LTM configuration correspondingly.

For the case of PSCell change/addition or the SCG release, in general we think the reconfiguration of the PSCell should not impact the PCell, and also the LTM candidate configuration is a complete configuration. So for this case, the LTM configuration is still valid. Besides, as described above, since UE is still under control by NW, so the handling on the LTM configuration can be left be NW control.

Proposal 3: Upon legacy PCell handover (CHO or legacy HO)/PSCell change/addition/SCG release, it is up to NW implementation on how to handle the LTM configuration.

For the case that RRC re-establishment is initiated, as we analyzed in the clause 2.5, the LTM recovery may be performed, e.g. this depends on whether the LTM recovery is supported and configured by NW. So upon initiating the re-establishment procedure, the LTM configuration should be maintained by UE so as for recovery. While if the LTM recovery is not triggered upon performing cell selection with T311 is running (i.e., the selected cell is not an LTM candidate), since the serving cell will be changed, so the LTM configuration will become invalid and should be deleted.

Observation 6: Upon initiating the re-establishment procedure, whether to release the LTM configuration depend on whether the LTM recovery is supported and configured.

Observation 7: Upon cell selection while T311 is running and the LTM recovery is not triggered, the LTM configuration is invalid.

Another case is upon going to RRC_IDLE/INACTIVE, this performs mainly since there is no data transmission, that is the service changes. And as we analyzed above, the LTM is mainly for the mobility but not for service change. So we think even later UE resuming its connection from RRC_INACTIVE, or setup from RRC_IDLE, the LTM configuration may be not valid, i.e., the bearer configuration may be not proper. Furthermore, when UE goes to IDLE or INACTIVE, when and where the UE will trigger RRC connection resume/setup procedure is not predictable. It is a waste for the candidate cell to reserve resource for the UE. So it is proposed that upon UE going to idle/inactive, the LTM configuration should be released by UE.

Observation 8: When and where the UE trigger RRC resume/setup procedure is not predictable, the LTM configuration may be invalid and it is a waste for candidate cell to reserve resource for the UE.

For the case of re-establishment, NW has no awareness on the procedure before receiving the RRCRestablishmentRequest message. As for the RRC release case, this still requires explicit indication on RRCRelease message if NW to control. So it is proposed to relay on UE autonomously release of these invalid LTM configuration for these cases.

Proposal 3: UE autonomously release the LTM configuration upon the following cases:

- Upon initiating the re-establishment procedure and the LTM recovery is not configured;

- Upon cell selection while T311 is running and the LTM recovery is not triggered;

- Upon UE going to IDLE state or INACTIVE state.

On supervision timer for LTM cell switch

On the timer-supervised LTM upon cell switch, previously we had confirmed that the LTM timer is an RRC maintained timer,

LTM supervisor timer is RRC layer timer.

Regarding how to provide LTM timer to UE and whether to separate RACH-based LTM timer from the RACH-less one, until now we have no consensus on these issues. The former issue is covered by the content of the LTM triggered MAC CE in the post email discussion, with the Rapporteur view that even introducing a RACH-less specific LTM timer, it should be configured by RRC. Generally speaking, taking RRC configuration as baseline seems to be straightforward, and then we further consider whether separate timer for RACH based and RACH-less cell switch is needed.

As the timers used for RACH-based and RACH-less scenarios may require different values due to different delay requirement, the NW should be able to provide UE with the corresponding timer for different scenarios. Companies may concern that even the NW provides the LTM timer for RACH-less scenario, a single timer can be pre-configured by RRC, since both RACH-based and RACH-less scenarios in LTE reuse the same timer T304. But from our view the LTM differs from the LTE RACH-less mobility. In LTE, the NW explicitly indicate UE to perform RACH-less handover by including the IE RACH-Skip in MobilityControlInfo, this means the NW is aware whether to let UE perform RACH-based or RACH-less access upon handover, thus the NW can reconfigure UE with a suitable timer (i.e.T304, ) for handover supervision.

Observation 9: In LTE, the RACH-based and RACH-less handover reuse the timer T304 due to the fact that the NW is aware whether to let UE perform RACH-based or RACH-less access upon handover.

When considering LTM, as mentioned in above, it is provided by RRC pre-configuration. According to the current RAN2 agreement,

The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

But the concern is that upon pre-configuration, the NW cannot determine whether the UE to perform RACH-based or RACH-less access to a certain candidate cell upon LTM execution, because the NW cannot predict whether it can successfully obtain the TA for this candidate cell. Thus only providing one timer in the pre-configuration is not suitable for both RACH-based and RACH-less latency requirement. Consequently, the RRC pre-configuration needs to provide two supervisor timers for LTM, with one for RACH-less scenario and the other for RACH-based scenario. UE can choose the corresponding timer based on whether RACH is performed upon LTM execution.

Proposal 4: Upon pre-configuration for LTM, the RRC provides two supervisor timers, with one for RACH-less scenario and the other for RACH-based scenario.

On failure handling for LTM

Regarding the failure for LTM, in general, the following cases may happen:

- Compliance check failure on candidate cell configuration/reference configuration;

- RLF on PCell;

- Beam failure on serving cell;

- LTM cell switch failure;

- Legacy handover failure;

In the following discussion, we will give brief analysis on the corresponding handling on each of the above failure.

Case 1: on handling for 1) compliance check failure on candidate cell configuration/reference configuration:

In legacy, upon compliance check failure for MCG configuration or SCG configuration configured via the SRB1, the RRC re-establishment will be triggered. Or the compliance check for SCG configuration via SRB3 is failed, and then either the SCG failure information procedure or the RRC re-establishment will be triggered.

Figure 1: legacy handling for compliance check failure

And for CHO/CPAC, if the candidate configuration is compliance check failure, the same mechanism is used. We think the similar procedure can be reused for LTM. One may argue that upon compliance check of the LTM candidate fails, instead to enter the re-establishment procedure, UE can provide some indication on the failed LTM candidate cells to NW, so that NW can update the configuration. But according to the following note extracted from the TS38.331, the partial success/failure of the compliance check is not supported.

NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

Observation 10: As for legacy compliance/validity check, the partial success/failure is not supported.

Further, the compliance check failure is indeed some corner case, since NW know all UE capability information, and smart NW will not configure some unavailable configuration to UE.

Observation 11: The compliance check failure is corner case, since smart NW can avoid it via implementation.

Based on this, it is proposed that:

Proposal 5: Upon compliance check failure on MCG LTM candidate cell configuration/reference configuration, the RRC re-establishment procedure will be initiated.

Case 2: on handling of 2) RLF on SpCell

In legacy, RLM is only supported for SpCell. And upon RLF for MCG happen, if the active SCG transmission is not suspended and also the fast recovery is configured, then UE will perform the MCG fast recovery via SCG. If SCG is deactivated, or SCG is suspended, or fast recovery is not configured, then UE will go to the re-establishment procedure. Then upon initiating the re-establishment procedure, if CHO recovery is configured and the selected cell is one the CHO candidate, then CHO recovery will be triggered. Otherwise, the RRC re-establishment will be executed.

Figure 2: handling for RLF of PCell

For the PCell RLF case, we think similar procedure like legacy can be reused. And also the LTM recovery similar like the CHO recovery can also be supported. That is, NW configures whether to perform the LTM recovery for LTM candidate.

Proposal 6: In case the RLF on PCell is declared, UE shall

- If the MCG LTM recovery is configured and the selected cell is a candidate, then LTM recovery is triggered;

- Otherwise, RRC re-establishment will be executed.

Case 3: on handling for 3) Beam failure on serving cell:

Upon BFD is detected for SCell, the BFR MAC CE will be used to perform the BFR (within which procedure, RACH may also be triggered for CG to transmit the BFR MAC CE). And in case of the BFD is detected for SpCell, RACH procedure will be triggered. In general, we think the legacy procedure can be considered here.

And once the RACH procedure fails in case of the BFR procedure, the RLF will be triggered (case 3). Since this is already covered by the case 3), so no need other special handling.

Case 4/5: on handling for 4) LTM cell switch failure, and 5) legacy handover failure:

In legacy, if the CHO handover or legacy HO fails (T304 expire), the re-establishment procedure will be initiated. Then upon initiating the re-establishment procedure, if CHO recovery is configured and the selected cell is one the CHO candidate, then CHO recovery will be triggered. Otherwise, the RRC re-establishment will be executed.

Figure 3: CHO handover/legacy PCell handover failure

In general, we think the similar procedure can be reused for LTM.

Proposal 7: In case of MCG LTM cell switch failure or the legacy PCell handover failure, UE shall

- If the MCG LTM recovery is configured and the selected cell is a candidate, then LTM recovery is triggered;

- Otherwise, RRC re-establishment will be executed.

Conclusion

Based on the previous analysis in section 2, our main contributions are summarized as follows:

Observation 1: For R18 LTM, based on the agreements, it is feasible that the RadioBearerConfig, and the RLC configuration can be different before and after LTM cell switch.

Observation 2: There may be misalignment between UE and NW side if the RadioBearerConfig and the RLC configuration are not released upon performing cell switch.

Observation 3: LTM is initiated for mobility, i.e., to minimize the mobility interruption latency, but not for service type/QoS change.

Observation 4: The frequency resource of the RACH occasion is configured with respective to the frequency location of the BWP.

Observation 5: Providing the corresponding BWP configuration of each candidate cell, i.e., BWP-Uplink, along with the RACH resource of each candidate cell, leads to heavy signaling overhead.

Observation 6: Upon initiating the re-establishment procedure, whether to release the LTM configuration depend on whether the LTM recovery is supported and configured.

Observation 7: Upon cell selection while T311 is running and the LTM recovery is not triggered, the LTM configuration is invalid.

Observation 8: When and where the UE trigger RRC resume/setup procedure is not predictable, the LTM configuration may be invalid and it is a waste for candidate cell to reserve resource for the UE.

Observation 9: In LTE, the RACH-based and RACH-less handover reuse the timer T304 due to the fact that the NW is aware whether to let UE perform RACH-based or RACH-less access upon handover.

Observation 10: As for legacy compliance/validity check, the partial success/failure is not supported.

Observation 11: The compliance check failure is corner case, since smart NW can avoid it via implementation.

On reference/candidate/generated complete configuration and UE behavior upon LTM cell switch;

Proposal 1: When configuring LTM, NW ensures the radio bearer related configuration and also the RLC related configuration within the complete LTM candidate configuration is the same as UE’s current configuration.

On provision of the RACH configuration for early sync;

Proposal 2: To enable UE to decide the frequency resource of the RACH occasion, the following candidate options can be considered.

- Option 1: NW provides the absolute frequency of lowest RACH occasion in frequency domain;

- Option 2: NW provides the generic parameters of the BWP where the RACH resource is configured and also the IE absoluteFrequencyPointA and offsetToCarrier corresponding to the subcarrierSpacing as indicated within the IE BWP;

- Option 3: NW provides the absolute frequency of the lowest BWP in frequency domain where the RACH resource is configured, and the SCS.

On handling of LTM configuration upon other procedure (e.g., reconfiguration with sync) ;

Proposal 3: Upon legacy PCell handover (CHO or legacy HO)/PSCell change/addition/SCG release, it is up to NW implementation on how to handle the LTM configuration.

Proposal 3: UE autonomously release the LTM configuration upon the following cases:

- Upon initiating the re-establishment procedure and the LTM recovery is not configured;

- Upon cell selection while T311 is running and the LTM recovery is not triggered;

- Upon UE going to IDLE state or INACTIVE state.

On supervision timer for LTM cell switch;

Proposal 4: Upon pre-configuration for LTM, the RRC provides two supervisor timers, with one for RACH-less scenario and the other for RACH-based scenario.

On failure handling for LTM;

Proposal 5: Upon compliance check failure on MCG LTM candidate cell configuration/reference configuration, the RRC re-establishment procedure will be initiated.

Proposal 6: In case the RLF on PCell is declared, UE shall

- If the MCG LTM recovery is configured and the selected cell is a candidate, then LTM recovery is triggered;

- Otherwise, RRC re-establishment will be executed.

Proposal 7: In case of MCG LTM cell switch failure or the legacy PCell handover failure, UE shall

- If the MCG LTM recovery is configured and the selected cell is a candidate, then LTM recovery is triggered;

- Otherwise, RRC re-establishment will be executed.

Reference

[1] R2-2307222, on scenarios for LTM.

[2] R2 121bise Chair Notes EOM.

[3] R2 120 Chair Notes EOM.

[4] R2-2308435, RRC running CR for LTM.

[5] TS38.331.