3GPP TSG-RAN WG2 Meeting #123	 R2- 2307227

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	7.9.4

Source:	Xiaomi

Title:	Discussion on multipath

Document for:	Discussion

Introduction

In multi-path, remote UE can connect to the same gNB via both direct and indirect path. There are two scenarios according to [1]. In scenario 1, indirect path reuses U2N relay. In scenario 2, indirect path uses ideal connection. In this contribution, we discuss open issues in both scenarios and give our proposals.

Discussion

Discussion on scenario 1

RB config

One open issue is whether non-split SRB on indirect path is supported in MP. Remote UE need to fulfil threshold condition to enable relay function. Therefore, remote UE is expected to be at cell edge. Remote UE may move out of coverage on direct path but still in coverage of indirect path. In such case, non-split SRB on indirect can provide more reliability. There are views that duplicated/split SRB can be used to obtain the reliability gain from indirect path. But we understand split/duplicate SRB would cost more resource and non-split SRB on indirect path can reduce the signalling on direct path caused by duplication. Furthermore, the relay UE and remote UE may connect to the same cell. It’s up to gNB implementation to configure the non-split SRB on indirect path if indirect path and direct path are on the same PCell. So, non-split SRB on indirect path doesn’t conflict with the agreement that PCell is always on the direct path. Therefore, we don’t see any problem to support non-split SRB on indirect path. Non-split SRB on indirect path can be configured by gNB implementation if indirect path is more reliable than direct path.

Proposal 1: Non-split SRB on indirect path is supported.

In last meeting, following WA is made,

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

Companies supporting WA believes direct path is always more reliable than indirect path. However, as we discussed above, indirect path may provide more reliability than direct path if remote UE is moving from IC to OOC. Therefore, we understand primary RLC entity could be configured on either direct path or indirect path by gNB implementation.

Proposal 2: Revisit the working assumption from last meeting and agree primary RLC entity of the MP split bearer could be configured on either direct path or indirect path.

How to handle relay UE handover

In last meeting, there is a FFS on how to handle relay UE handover. Two options are observed,

Option 1, rely on network to release configuration of relay UE at remote UE before relay UE handover,

Option 2, rely on remote UE to suspend indirect path upon reception of notification message indicating handover

In option 1, relay UE’s handover may be delayed until remote UE’s multipath is released. In some scenarios, relay UE’s channel quality may degrade rapidly, such as high speed or Hetnet. Such delay may result in handover failure or RLF to relay UE. To avoid the HOF or RLF, relay UE may perform handover before multipath is released at remote UE. In such cases, remote UE shall suspend the indirect path upon reception of notification message. NW can still release the multipath afterwards.

Proposal 3: Rely on remote UE to suspend indirect path upon reception of notification message indicating handover. NW can still release the multipath afterwards.

Path Failure recovery

In last meeting, it’s agreed the remote UE report the path failure on the other path if SRB1 is available. Regarding the indirect path failure, in legacy, relay UE in RRC_CONNECTED would send notification message to remote UE upon RLF, handover or RRC connection failure. Remote UE may consider indirect path failure upon reception of the notification message. In multipath, the notification procedure is still reused by relay UE. If remote UE receives the notification message, remote UE shall also report indirect failure via direct path if SRB1 is available on direct path. Otherwise, remote UE shall trigger RRC reestablishment.

Proposal 4: Relay UE reuse NotificationMessageSidelink in MP to notify its RLF and Uu access failure.

Proposal 5: Upon reception of NotificationMessageSidelink, if SRB1 is available on direct path not suspended, remote UE triggers report to network via direct path to report the failure. Otherwise, remote UE triggers RRC reestablishment.

It’s unclear what info is reported in the failure report. Similar as SCG/MCG failure, remote UE shall indicate the failure cause to NW. NW can choose whether release or switch to another path based on the failure cause. Upon direct path failure, the cause value may include t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, t312-Expiry-r16, lbt-Failure-r16, beamFailureRecoveryFailure-r16, bh-RLF-r16. Upon indirect path failure, the cause value may include SL-RLF, relayUE-Uu-RRC-Failure, relayUE-Uu-RLF.

Additionally, UE can include measurement result of the candidate node to NW. If direct path fails, measurement result of the neighbour cell can be included. If indirect path fails, measurement result of candidate relay can be included. NW can choose whether to switch the path to another cell/relay UE.

Proposal 6: Remote UE shall report failure cause and measurement results of candidate cell/relay UE.

Proposal 7: The failure cause of indirect path includes SL-RLF, relayUE-Uu-RRC-Failure, relayUE-Uu-RLF. Failure cause of direct path can reuse the legacy values in MCGFailureInformation.

One open issue is which message is used to indicate indirect path failure. Several options are on the table, SUI, MCGFailureInformation and new message. SUI require full config report, i.e. UE shall set all applicable IEs in SUI every time. During failure recovery, many IEs in SUI are not useful and may not change compared with last transmission. A lot of signalling would be wasted. It’s agreed MCGFailureInformation is used to report the direct path failure information. Also, MCGFailureInformation doesn’t require full config report. It’s simple and efficient to reuse MCGFailureInformation to report the indirect path failure information.

Proposal 8: Reuse MCGFailureInformation to report the indirect path failure information.

On Uu, T316 is used to determine recovery failure and trigger RRC reestablishment. T316 can be reused. In multipath, NW may choose to perform path switch to recover the failure. Therefore, T316 should be stopped if path switch is received, besides the legacy stop event.

Proposal 9: T316 is reused for indirect failure recovery. Besides the legacy stop event, if UE receives path switch, T316 is stopped.

SI forwarding

It’s agreed PCell is always on the direct path. Therefore, remote UE shall acquire the SI of the PCell on the direct path. However, U2N relay UE can forward SIB1 in unsolicited way, upon SIB1 update or remote UE’s initial access. If U2N relay UE’s serving cell is different from direct path, U2N relay UE shall not forward SIB1 to remote UE in multipath.

Proposal 10: If U2N relay UE’s serving cell is different from remote UE’s PCell, U2N relay UE shall not forward SIB1 to remote UE in multipath.

PC5-RRC message to bring IDLE/INACTIVE relay UE into CONNECTED

It’s agreed for bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured. A PC5-RRC trigger is specified at least for other cases.

In R17, relay UE would set the cause value to emergency if emergency is set as the cause value in remote UE’s RRC setup/resume request message. However, in multipath, remote UE doesn’t sent RRC setup/resume request message to relay UE. Therefore, relay UE can’t determine which cause value should be set in its own RRC setup/resume request message. At least for emergency case, relay UE shall set emergency as cause value to avoid RRC setup/resume request rejected by gNB. One solution is for remote UE to indicate the cause value in the PC5-RRC message, if it’s emergency. Relay UE can set the same cause value as remote UE accordingly.

In legacy, the bearer configuration other than RLC0/1 should be configured by gNB after relay UE enter RRC_CONNECTED. This may introduce additional delay to set up multipath bearer or indirect path bearer. Alternatively, NW can send multipath/indirect bearer configuration to remote UE. Remote UE forwards the configuration via PC5-RRC message to relay UE. Upon reception of the configuration, relay UE can establish the multipath/indirect bearer. The delay can be reduced.

Proposal 11: The PC5-RRC message can include remote UE’s RRC connection setup/resume cause value. Relay UE set the cause value in its own RRC connection setup/resume request accordingly.

Proposal 12: The PC5-RRC message can include multipath/indirect path bearer configuration from gNB. Relay UE establish multipath/indirect path bearer accordingly.

How to avoid inter-gNB multipath

During R17 path switch, target relay UE in RRC_IDLE/RRC_INACTIVE may reselect to another cell before remote UE receives the path switch command. Since the new cell may not be prepared, the path switch would fail. To avoid such path switch failure, remote UE would declare path switch failure and trigger RRC reestablishment if target relay UE changes serving cell. During indirect path addition/change in multipath, similar issue exists. The difference is remote UE is still connected to gNB by direct path. Therefore, remote UE doesn’t need to trigger RRC reestablishment. But the indirect path addition/change would still fail. To avoid such failure, remote UE can suspend the indirect path addition and report the target relay UE’s serving cell change to gNB. gNB can decide to continue the path addition/change or change the target relay UE.

Proposal 13: During indirect path change/addition, if target relay UE’s serving cell changes, remote UE suspend the indirect path change/addition and report the updated target relay UE’s serving cell to gNB.

When to release the indirect path

During the post 122 email discussion #403 Procedures for multi-path relay, one open issue is when the remote UE release the indirect path after NW configures remote UE to release the indirect path. In R17, remote UE can decide whether to keep the unicast connection with relay UE upon reception of RRCRelease or NotificationMessageSidelink. The motivation is to continue receive the data buffered at relay UE. In multipath, non-split RB may be configured on indirect path. It’s still possible there is data buffered at relay UE, which can’t be retransmitted on direct path. Therefore, it’s beneficial for remote UE to decide whether/when to release the PC5 unicast connection with relay UE if NW configures remote UE to release the indirect path.

Proposal 14: It’s up to remote UE to decide whether/when to release the PC5 unicast connection with relay UE if NW configures remote UE to release the indirect path

Discussion on scenario 2

One left FFS for Scenario 2 is as following,

Whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

Case G implies there may be multiple relay UEs which can be connected with remote UE. Remote UE can change the relay UE dynamically. However, in RAN2 119 meeting, RAN2 assumes that the relation between remote UE and relay UE in scenario 2 is pre-configured or static and how the relation is pre-configured or static is out of the 3GPP scope. Therefore, we understand there is no dynamic change on the relay UE in scenario 2.

Proposal 15: Following case is not supported,

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

Another issue is how to configure 1:1 bearer mapping. In current bearer configuration, each RLC can serve either SRB or DRB. In multipath, the RLC entity at relay UE can serve either relay UE’s RB or remote UE’s RB. The RLC configuration can be extended to indicate the RLC entity at relay UE serve which UE’s RB.

Proposal 16: The RLC configuration can be extended to indicate the RLC entity at relay UE serve which UE’s RB.

In last meeting, it’s agreed remote UE can report relay UE’s C-RNTI and serving cell ID. But it’s still not clear when remote UE reports. We understand remote UE report relay UE’s info to request gNB to add indirect path. There is no defined measurement quantity on the ideal connection. It’s up to remote UE to evaluate which relay UE is suitable. Therefore, it’s up to remote UE to decide when to trigger such report. Another question is to use which message. Since it’s up to remote UE to decide the report, UAI may be reused.

Proposal 17: It’s up to remote UE to decide when to report relay UE’s C-RNTI and serving cell ID and report which relay UE. No spec impacts.

Proposal 18: UAI is used to report relay UE’s C-RNTI and serving cell ID.

Conclusion

Based on the discussion in section 2, we have following proposals:

- Scenario 1

Proposal 1: Non-split SRB on indirect path is supported.

Proposal 2: Revisit the working assumption from last meeting and agree primary RLC entity of the MP split bearer could be configured on either direct path or indirect path.

Proposal 3: Rely on remote UE to suspend indirect path upon reception of notification message indicating handover. NW can still release the multipath afterwards.

Proposal 4: Relay UE reuse NotificationMessageSidelink in MP to notify its RLF and Uu access failure.

Proposal 5: Upon reception of NotificationMessageSidelink, if SRB1 is available on direct path not suspended, remote UE triggers report to network via direct path to report the failure. Otherwise, remote UE triggers RRC reestablishment.

Proposal 6: Remote UE shall report failure cause and measurement results of candidate cell/relay UE.

Proposal 7: The failure cause of indirect path includes SL-RLF, relayUE-Uu-RRC-Failure, relayUE-Uu-RLF. Failure cause of direct path can reuse the legacy values in MCGFailureInformation.

Proposal 8: Reuse MCGFailureInformation to report the indirect path failure information.

Proposal 9: T316 is reused for indirect failure recovery. Besides the legacy stop event, if UE receives path switch, T316 is stopped.

Proposal 10: If U2N relay UE’s serving cell is different from remote UE’s PCell, U2N relay UE shall not forward SIB1 to remote UE in multipath.

Proposal 11: The PC5-RRC message can include remote UE’s RRC connection setup/resume cause value. Relay UE set the cause value in its own RRC connection setup/resume request accordingly.

Proposal 12: The PC5-RRC message can include multipath/indirect path bearer configuration from gNB. Relay UE establish multipath/indirect path bearer accordingly.

Proposal 13: During indirect path change/addition, if target relay UE’s serving cell changes, remote UE suspend the indirect path change/addition and report the updated target relay UE’s serving cell to gNB.

Proposal 14: It’s up to remote UE to decide whether/when to release the PC5 unicast connection with relay UE if NW configures remote UE to release the indirect path

- Scenario 2

Proposal 15: Following case is not supported in scenario 2,

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

Proposal 16: The RLC configuration can be extended to indicate the RLC entity at relay UE serve which UE’s RB in scenario 2.

Proposal 17: It’s up to remote UE to decide when to report relay UE’s C-RNTI and serving cell ID and report which relay UE. No spec impacts in scenario 2.

Proposal 18: UAI is used to report relay UE’s C-RNTI and serving cell ID in scenario 2.

Reference

[1] RP-223501 Revised WID on NR sidelink relay enhancements