3GPP TSG-RAN WG2 Meeting #123	R2-2307265

Toulouse, France, August 21-25, 2023

Source:	CATT, CBN

Title:	Remaining Issues on Shared Processing

Agenda Item:	7.11.3

Document for:	Discussion and Decision

1	Introduction

In RAN2#122 meeting, the following agreements have been reached on shared processing for MBS broadcast and unicast reception, according to [1].

The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs. No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed). When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII) No additional information is added to MII on top of what has been already agreed.

In this paper, we discuss the remaining issues for this objective.

The contribution is organized as the following. In section 2, we discussed the following issues,

- Exact parameters of the enhanced MII information

- How to address the case where additional information cannot be read by the UE

- UE handling when the non-serving cell updates the configuration.

Our proposals are summarized in Section 3.

2	Discussion

## 2.1 Exact parameters of the enhanced MII information

In previously meeting it was agreed that the information for shared processing to be signalled includes: broadcast frequency, subcarrier spacing, and bandwidth.

In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case)

Then in last meeting it was confirmed no additional information is added to MII on top of the agreed ones.

No additional information is added to MII on top of what has been already agreed.

Among the agreed parameters (i.e., broadcast frequency, subcarrier spacing, and bandwidth), the meaning of “broadcast frequency” is straightforward but it is still necessary to further clarify the exact meaning of the parameter “subcarrier spacing” and “bandwidth”.

Subcarrier spacing

In R17 MBS, the UE receive MBS broadcast on CFR which has same SCS as CORESET0, according TS 38.300 as below,

A CFR configured by SIB is defined for broadcast scheduling as an 'MBS frequency region' with a number of contiguous PRBs with a bandwidth equal to or larger than CORESET0, with the same numerology as CORESET0, and broadcast scheduling may have specific characteristics (e.g., PDCCH and PDSCH configurations).

So for shared processing, it is natural that the SCS to be reported for non-serving cell should be the SCS of the CORESET0 of the non-serving cell.

Proposal 1: For shared processing, the subcarrier spacing reported in MII is the SCS of the CORESET0 of the non-serving cell.

Bandwidth

For the bandwidth to be reported in MII for shared processing, it is observed in previous meetings that some of the companies think it should be broadcast CFR of the non-serving cell whiles some other companies think it should be channel bandwidth of the non-serving cell. This issue is also captured in the RRC running CR [2] as a FFS.

Editor’s note: FFS what is the exact bandwidth that UE reports in MII.

It is known that UE receives MBS broadcast on the broadcast CFR of the cell. So to achieve shared processing between unicast and broadcast, the unicast gNB only need to know the CFR of non-serving cell when schedules the unicast reception.

Therefore, it is proposed,

Proposal 2: For shared processing, the bandwidth reported in MII is the broadcast CFR of the non-serving cell.

## 2.2 On the case where additional information cannot be read by the UE

It was agreed in RAN2#122 meeting that one step is used for MII reporting and UE always reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. However, an issue was raised by company that there are cases where UE cannot acquire the whole information from the non-serving cell due to UE capability restrictions, a FFS was left on this issue as below,

When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then.

There may be cases where UE cannot sync to the non-serving cell and thus cannot read the corresponding system information due to the hardware capabilities are occupied by the operation on the unicast gNB. But in this case, UE can wait to receive broadcast on non-serving cell later as broadcast reception is not so urgent. So it seems sufficient to up to UE implementation for this case.

Proposal 3: It is up to UE implementation for handling the case where additional information cannot be read by the UE from the non-serving cell.

## 2.3 UE handling when the non-serving cell updates the PTM configuration

An issue was discussed in previous meeting that whether/how to trigger the MII reporting when the PTM configuration of the non-serving cell is changed,

FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII)

The issue is that there may be duplicated MII reporting from UEs when the PTM configuration (e.g., SCS, CFR) related to MII reporting is changed on the non-serving cell. Basically the potential optimization for this issue is try to avoid duplicated reporting among UEs on Uu interface, i.e., when the PTM configuration is changed, the remaining UEs do not need to report the same information to the unicast gNB if they have been reported by other UE.

However, the issue may only exist in the case where many UEs operate on the same broadcast gNB and the same unicast gNB, which is a very rare case. Furthermore, the configuration such as SCS, CFR on the non-serving cell is not supposed to change frequently, so duplicated reporting should not be a problem.

Besides, for the potential optimization for PTM configuration change case, additional network control is needed. But we have agreed in last meeting that no additional control from NW is introduced,

No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed).

Based on above, optimization for PTM configuration change case is not needed.

Therefore, we propose,

Proposal 4: For shared processing, UE reports then MII once the relevant PTM configuration changes on the non-serving cell.

3	Conclusion

Based on the discussions, we have the following proposals for the remaining issues of shared processing for MBS broadcast and Unicast reception,

Proposal 1: For shared processing, the subcarrier spacing reported in MII is the SCS of the CORESET0 of the non-serving cell.

Proposal 2: For shared processing, the bandwidth reported in MII is the the Broadcast CFR of the non-serving cell.

Proposal 3: It is up to UE implementation for handling the case where additional information cannot be read by the UE from the non-serving cell.

Proposal 4: For shared processing, UE reports then MII once the relevant PTM configuration changes on the non-serving cell.

4	Reference

[1] Draft_R2_122_meeting_report_v2, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_122/Report

[2] R2-2306855, RRC running CR for eMBS, Huawei, HiSilicon