3GPP TSG-RAN WG2 Meeting #123	 					     R2-2307274

Toulouse, France, August 21-25, 2023

Source:	Continental Automotive

Title:                       Discussion on LP-WUS in RRC IDLE and INACTIVE

Agenda Item:         7.22.2

Document for:	Discussion and Decision

# Introduction

For LP-WUS in RRC IDLE and INACTIVE, the following agreements were achieved in RAN2 #122 meeting [1]:

- Agreement:

RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g. UE need to stop using LP WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

In this contribution, we discuss the possible further enhancements for LP-WUS in RRC IDLE and INACTIVE.

# Discussion

The motivation of LP-WUS is to enable the UE to place its MR in an ultra-deep sleep state when there is no data or signal exchange required. Instead, a separate LP-WUR is used to monitor a LP-WUS, which serves as an indication to wake up the MR. This approach allows for significant power-saving gains by efficiently managing power consumption during periods of inactivity.

In the RRC_IDLE/INACTIVE, the UE monitors the paging occasion for a paging message to receive DL data from the network. However, when the UE is configured to receive the LP-WUS, the LP-WUS may act as a replacement for the paging message. Consequently, the UE is not required to monitor the paging message when it detects the LP-WUS, and it can trigger the RACH procedure directly to receive the DL data.

Proposal 1: When the LP-WUR of the UE is configured to monitor LP-WUS, the UE upon receiving LP-WUS may trigger RACH procedure directly, hence it may act as a replacement for paging message.

It was agreed in 3GPP RAN1#112bis-e meeting [2] that LP-WUR monitoring LP-WUS may not always be able to meet LP-WUS RSRP/RSRQ threshold configured by the network for LP-WUS. It is possible that the UEs monitoring the LP-WUS have different receiver sensitivities. Thus, a UE which is within the LP-WUS coverage, but which has poor receiver sensitivity, may still miss the LP-WUS, and because of the LP-WUS RSRP/RSRQ being below the threshold, turn on its MR for monitoring the paging message instead. This implies that a single LP-WUS threshold is not the most effective criterion to achieve the energy saving promised by LP-WUS. It would be much more beneficial for UE energy saving if the network configures an LP-WUS threshold that is UE specific in the RRC_INACTIVE state. There are two possibilities for the network to determine the LP-WUR sensitivity of the UE. In the first, the UE can include the LP-WUR sensitivity in the UE assistance information. In the second, the UE can include the LP-WUR sensitivity in the UE capability message. Based on this information, the network can configure a UE specific LP-WUS RSRP/RSRQ threshold in the RRC release message for that UE.

Observation 1: A common LP-WUS RSRP/RSRQ threshold is not suitable for all UEs due to their different receiver sensitivities.

Proposal 2: In RRC_INACTIVE, the UE decides to monitor the LP-WUS based on a UE specific LP-WUS RSRP/RSRQ threshold configured by the network.

In the RRC_INACTIVE state, the UE doesn’t report measurement information to the network. Moreover, a UE in the RRC_INACTIVE state may go out of the LP-WUS coverage due to its mobility, and the network will not be able to track the UE location in the cell. Hence, we think it is beneficial that the UE does not monitor the LP-WUS when it is out of LP-WUS coverage or at the LP-WUS coverage edge, unless repetition of the LP-WUS is enabled by network. The UE can provide a status update on whether it is turning off the LP-WUS monitoring or not, and in cases it is not, request for the LP-WUS repetition (e.g., if the UE is at the LP-WUS coverage edge) when the MO-SDT or MT-SDT procedure is triggered.

Proposal 3: In RRC_INACTIVE, it is beneficial for the UE to inform the LP-WUS monitoring status to the network during SDT procedure.

Proposal 4: In RRC_INACTIVE it is beneficial for the UE to inform the network to enable LP-WUS repetition during the SDT procedure.

# Conclusions

In this contribution, we discussed the possible enhancements for LP-WUS in RRC IDLE and INACTIVE. Additionally, we ask RAN2 to discuss the following observations and proposals:

Proposal 1: When the LP-WUR of the UE is configured to monitor LP-WUS, the UE upon receiving LP-WUS may trigger RACH procedure directly, hence it may act as a replacement for paging message.

Observation 1: A common LP-WUS RSRP/RSRQ threshold is not suitable for all UEs due to their different receiver sensitivities.

Proposal 2: In RRC_INACTIVE, the UE decides to monitor the LP-WUS based on a UE specific LP-WUS RSRP/RSRQ threshold configured by the network.

Proposal 3: In RRC_INACTIVE, it is beneficial for the UE to inform the LP-WUS monitoring status to the network during SDT procedure.

Proposal 4: In RRC_INACTIVE it is beneficial for the UE to inform the network to enable LP-WUS repetition during the SDT procedure.

# References

[1] RAN2#122 Chairman Notes

[2] RAN1#112bis-e Chairman Notes.