3GPP TSG-RAN WG2 #123	R2-2307279

Toulouse, 21st August – 25th August 2023

Source:                    	DENSO CORPORATION

Title:  	Consideration on flight path reporting for NR UAV

Document for:        	Discussion and decision

Agenda Item:         	7.8.3 	Flight path reporting

- Introduction

In the past RAN2 meetings, the following have been agreed about the flight path reporting.

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. - A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity - No requirements are placed on spatial distribution of waypoints - A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. - UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. - UAI message can also be used to indicate the UE has flight path availability. - FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS.

- RAN2#120[1]:

- RAN2#121[2]:

- The granularity of flightpath timestamp is 1s. - Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

- RAN2#121bis-e[3]:

- Flightpath update indication in UAI is configurable by the network - Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE - Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] - As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) - As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling

- RAN2#122[4]:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In accordance with the above agreements and FFS, this paper attempts to investigate about the consideration on the flight path report and further enhancements.

- Discussion

  - Delta flight path reporting

In the previous two meetings (RAN2 #121bis-e, #122), it was not agreed whether delta flight path reporting should be supported or not. During the meetings, there were some opinions to avoid complicated signaling as the flight path update would not be performed frequently. We agree that the partial update for waypoints would not be performed frequently in the typical use case of UAV since only a few obstructions would be expected in the aerial route compared to terrestrial route.

However, we suppose that time difference between reported time and actual time could easily occur due to external condition (e.g. weather condition). In such case, sending the entire flight path would cause inefficient radio resource usage since the waypoint location information will not be updated but the timestamp should be updated.

Additionally, flightpath information might be maintained in long range since flightpath will be included in the handover message in NR. This assumption would raise the possibility of the time difference mentioned above. Therefore, the following could be observed and proposed:

Observation 1: Time difference between reported time and actual time could easily occur due to external conditions (e.g. weather condition)

Observation 2:	In NR, flightpath information might be maintained in long range since flightpath will be included in the handover message

Proposal 1: RAN2 to allow UE to update flight path partially at least for the timestamps

To update partial flight path, introducing index for each waypoint would work. With the index, UE can specify the waypoint to be updated, and UE can send only difference from the last report if the UE intend to update the flight path partially. Therefore, the proposed format of flight path would be like below:

FlightPathInfoReport-r18 ::=		SEQUENCE {

flightPath-r18					SEQUENCE (SIZE (1..maxWayPoint-r18)) OF WayPointLocation-r18,

nonCriticalExtension				SEQUENCE {}

}

WayPointLocation-r18 ::=	SEQUENCE {

index							INTEGER (0..XXXX),

wayPointLocation-r18				LocationInfo-r16             OPTIONAL,

timeStamp-r18						AbsoluteTimeInfo-r16         OPTIONAL

}

Figure 2. Proposed message structure of flight path

Proposal 2: RAN2 to introduce index for each waypoint so that UE can update flight path partially

Proposal 3: Define wayPointLocation as optional field to allow UE to update timeStamp only

  - Triggering conditions for updating flight path

In the last meeting (RAN2#122), it was agreed that UE should update the reported flightpath when network-configured triggering condition is satisfied. With this agreement, effectiveness of the flightpath could be possibly kept up to requirement of the network. However, periodicity of evaluation for the triggering condition was not discussed. To keep the effectiveness of the flightpath information, the periodicity of evaluation would be one of important factor. For example, the flight path information would be outdated if the periodicity of evaluation is very long.

Proposal 4: RAN2 to consider the periodicity of evaluation if the triggering condition is satisfied

Additionally, the mechanism of ReportConfig for measurement report could be used for the triggering condition configuration (D1 event could be referred to as distance-based event).

Proposal 5: The mechanism of ReportConfig for measurement report could be used for the triggering condition configuration by the network

Aside from network-configured triggering condition, there might be some cases where UE needs to update the reported flightpath like below:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

Therefore, we would like to propose to discuss how UE should behave when such exception cases occur.

Proposal 6: RAN2 to discuss about exception cases where UE needs to update flightpath information even though network-configured triggering condition is not satisfied, such as:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

- Summary and proposal

This paper discussed the consideration on flight path reporting of NR support for UAV. In summary, the followings were observed and proposed:

Observation 1: Time difference between reported time and actual time could easily occur due to external conditions (e.g. weather condition)

Observation 2:	In NR, flightpath information might be maintained in long range since flightpath will be included in the handover message

Proposal 1: RAN2 to allow UE to update flight path partially at least for the timestamps

Proposal 2: RAN2 to introduce index for each waypoint so that UE can update flight path partially

Proposal 3: Define wayPointLocation as optional field to allow UE to update timeStamp only

Proposal 4: RAN2 to consider the periodicity of evaluation if the triggering condition is satisfied

Proposal 5: The mechanism of ReportConfig for measurement report could be used for the triggering condition configuration by the network

Proposal 6: RAN2 to discuss about exception cases where UE needs to update flightpath information even though network-configured triggering condition is not satisfied, such as:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

- References

[1] R2-2213003, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[2] R2-2301903, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[3] R2-2304203, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[4] R2-2306543, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”