3GPP TSG-RAN WG2 Meeting #123	R2-2307281

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	SL Relay service continuity considerations

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The approved release 18 WI on enhanced NR SL Relay [RP-221262] includes the following objective on enhancement of service continuity:

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

This contribution discusses the issues that impact the service continuity of L2 U2N relay for inter-gNB path switching scenario (i.e., scenario A, B and D) and proposes the solutions to be discussed in RAN2.

# 2	Discussion

## 2.1	Issues related to relay UE’s inter-gNB HO during connected remote UE’s path switching

When the remote UE has already established U2N relay connection via the relay UE with the serving gNB, the remote UE’s UP and CP traffic transmission with the serving gNB are forwarded via the relay UE. In rel-17, when the relay UE is configured by the serving gNB to handover (HO) to another gNB, the relay UE sends a NotificationMessageSidelink message to the remote UE to indicate relay UE’s HO, which triggers the remote UE to initiate RRC connection re-establishment procedure. The RRC connection re-establishment procedure will impact the remote UE’s service continuity. In Rel-17, the remote UE can only be triggered to initiate the RRC connection re-establishment procedure as no inter-gNB path switching is supported for the remote UE in rel-17. As inter-gNB I2I or I2D path switching is supported in rel-18, the remote UE can be configured by the serving gNB to make the path switching. The main benefit of this enhancement is that there will be no break in the service of the remote UE.

Observation 1: The remote UE would benefit from being able to perform path switching instead of initiating RRC connection re-establishment when the connected relay UE performs a HO.

Proposal 1: RAN2 to agree the remote UE can be configured to perform path switching when the connected relay UE performs a HO.

If Proposal 1 is agreed, the serving gNB may configure the remote UE to perform path switching when it configures the relay UE to perform inter-gNB HO. When the relay UE receives RRC reconfiguration message (i.e., HO command message in HO case) from its own serving gNB to configure the relay UE to perform HO, the relay UE will start execution of the configured HO by detaching from the serving cell and synchronize to the target cell. As the remote UE’s RRC reconfiguration message for path switching is transmitted from the serving gNB to the remote UE via the relay UE, it may happen that the relay UE may detach from the serving cell before the relay UE receives the RRC reconfiguration message of the remote UE over Uu interface from the serving gNB. In this case the remote UE will not receive the RRC reconfiguration message from the serving gNB, thus the remote UE’s RRC connection to the current serving gNB will be interrupted and the ongoing services of the remote UE will be impacted.

It may be considered to solve the problem by the gNB triggering the remote UE’s RRC reconfiguration procedure for path switching earlier than the relay UE’s RRC reconfiguration for HO. However, it is not practical or feasible due to the following reasons:

- It may be too late to initiate RRC reconfiguration procedure to the relay UE for HO if the gNB needs to wait for the completion of the remote UE’s RRC reconfiguration procedure. This may impact robustness of relay UE’s HO, especially, in high mobility scenario that is typical scenario for many V2X use cases.

- In normal HO procedure, the RRCReconfigurationComplete message from the UE is not sent to the serving gNB (source gNB), but to the target gNB. In this case with regards to path switching of the remote UE, the serving gNB is not able to ensure the RRC reconfiguration message has been delivered to the remote UE before it initiates the RRC reconfiguration procedure to the relay UE for HO as there is no end-to-end RLC or MAC protocol between the gNB and the remote UE to allow the gNB be aware of the delivery of RRC reconfiguration message by ARQ or HARQ feedback.

Observation 2: The path switch command for the remote UE and the HO command for the relay UE are independent messages, and it is not possible for the gNB to ensure the path switch configuration arrives before the HO command.

To avoid the relay UE detaching from the serving cell before the relay UE receives the RRC Reconfiguration message of the remote UE from the serving gNB over Uu interface, the relay UE should be explicitly informed that the remote UE’s RRC Reconfiguration message has been delivered to the remote UE. For instance, the remote UE may indicate to the relay UE the reception of the RRC Reconfiguration message from the gNB if the relay UE sent the NotificationMessageSidelink message to indicate relay UE’s HO. Upon the indication from the remote UE, the relay UE can detach from the serving cell to perform the configured HO.

Proposal 2: RAN2 to agree that the relay UE should be explicitly informed about the reception of the RRC reconfiguration message by the remote UE and then the relay UE can detach from the serving cell for the HO.

## 2.2	Issue related to prolonged inter-gNB signaling over Xn interface for inter-gNB path switching

The Rel-17 baseline procedure of the intra-gNB direct to indirect path switching, which was agreed to be used as a baseline, is illustrated in Figure 1 (Figure 16.12.6.2-1 in TS 38.300). The gNB first configures the Relay UE for remote UE in step 2 before the gNB sends RRCReconfiguration message to configure the remote UE for switching to indirect path in step 3.

Figure 1: Rel-17 procedure for L2 U2N Remote UE switching to indirect path (TS 38.300, Figure 16.12.6.2-1)

For inter-gNB indirect path switching, the inter-gNB signaling over the Xn interface should be similar to HO preparation procedure (i.e., HO Request and HO Request Acknowledgement) over Xn interface for inter-gNB HO. That is, the serving gNB sends the path switching request to the target gNB and the target gNB responses the path switching request acknowledgement as illustrated in Figure 2. Following the baseline procedure of intra-gNB indirect path switching, the target gNB should first configure the target relay UE for the remote UE using RRC Reconfiguration procedure before it can send Path Switching Request Acknowledgement to the serving gNB of remote UE. The additional RRC reconfiguration procedure between the target gNB and the selected target relay UE will prolong the path switching preparation time before the source gNB can send path switching command to the remote UE.

Observation 3: When inter-gNB path switch to indirect path is performed the Relay UE configuration delays the target gNB to send the acknowledgement for path switching.

During the prolonged path switching preparation time, the probability of missing the path switching command transmitted from the source gNB for configuring path switching becomes higher than in normal inter-gNB HO scenario. For example, during the prolonged path switching preparation procedure over Xn, the remote UE may already move out of coverage of the current serving cell if the remote UE has direct path with the source gNB, or if the remote UE has indirect path with the source gNB, it may happen that the relay UE moves out of coverage of source cell or the SL connection between the remote UE and the currently connected relay UE fails and thus sending the RRCReconfiguration message to the remote UE may fail. Missing of the RRCReconfiguration message by the remote UE as illustrated in Figure 2 will cause path switching failure, which will impact the remote UE’s service continuity.

Observation 4: Missing of the RRCReconfiguration message by the remote UE can easily become a frequent problem due to prolonged path switching preparation time caused by inter-gNB path switching.

Figure 2: Procedure of inter-gNB path switching to indirect path

To enhance the remote UE’s service continuity for inter-gNB path switching, RAN2 should discuss the possible solution to deal with the issue. One of the solutions is to allow the source gNB to send an “early” RRCReconfiguration message to the remote UE for path switching before performing the RRC reconfiguration for the relay connection in the relay UE as depicted in Figure 3. In this early RRCReconfiguration message, the source gNB may provide the RRC reconfiguration parameters in the similar way as the target relay UE is in RRC_Idle/Inactive state according to the configuration parameter provided by the target gNB in step 4 of Figure 3. Upon the successful RRC Reconfiguration procedure between relay UE and the target gNB as illustrated in step 6 of Figure 3, the target relay UE can assist the remote UE’s inter-gNB path switching by forwarding the RRC Reconfiguration message to the remote UE.

Figure 3: Enhanced procedure of inter-gNB path switching to indirect path

Proposal 3: RAN2 to agree to allow early RRCReconfiguration message to the remote UE for path switching and to use the target relay UE to assist the remote UE’s inter-gNB path switching.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The remote UE would benefit from being able to perform path switching instead of initiating RRC connection re-establishment when the connected relay UE performs a HO.

Proposal 1: RAN2 to agree the remote UE can be configured to perform path switching when the connected relay UE performs a HO.

Observation 2: The path switch command for the remote UE and the HO command for the relay UE are independent messages, and it is not possible for the gNB to ensure the path switch configuration arrives before the HO command.

Proposal 2: RAN2 to agree that the relay UE should be explicitly informed about the reception of the RRC reconfiguration message by the remote UE and then the relay UE can detach from the serving cell for the HO.

Observation 3: When inter-gNB path switch to indirect path is performed the Relay UE configuration delays the target gNB to send the acknowledgement for path switching.

Observation 4: Missing of the RRCReconfiguration message by the remote UE can easily become a frequent problem due to prolonged path switching preparation time caused by inter-gNB path switching.

Proposal 3: RAN2 to agree to allow early RRCReconfiguration message to the remote UE for path switching and to use the target relay UE to assist the remote UE’s inter-gNB path switching.