3GPP TSG-RAN WG2 Meeting #123                                              	R2-2307352

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT

Title:	RAN2 Impact of 5GS network timing synchronization status and reporting

Agenda Item:	7.23.2

Document for:	Discussion and Decision

Introduction

In last meeting, multiple agreements have been achieved about objective 1b and 1c of the WID [1]:

Agreements

- Update of event ID is informed to UE by normal SI modification procedure.

- Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED

- For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation

- Event ID is optional.  Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.   FFS if gNB always broadcasts event ID

- If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure.

In this contribution, we discuss the content and the delivery method of the 5GS network timing synchronization in RRC_CONNECTED state and continue to clarify the RRC state change mechanism in RRC_IDLE/RRC_INACTIVE state.

Discussion

  - The Clock quality information sent to UE in RRC_CONNECTED

In WID it is said that the “5G Clock quality information” will be transmitted by the gNB to the UE in RRC_CONNECTED state. Details of the 5G clock quality information will be decided by RAN3, and the gNB will send this information based on the “clock quality reporting control information” from the core network and the gNB capability.

From the current TS23.501 [2] section 5.27.1.12, the “clock quality reporting control information” contains the field of “Clock quality detail level” which indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication.

5.27.1.12 Support for network timing synchronization status monitoring [Omitted part] For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, or AF may include CQRCI in its request. CQRCI contains the following fields: - Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. i.e. one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s). - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon degradation or enable it again upon recovery).

So in the air-interface, the gNB could send one type of the clock quality information to the UE via DLInformationTransfer message. It comprises one of the following values: clock quality metrics or acceptable/not acceptable indication.

For acceptable/not acceptable indication, only 1 bit is needed to indicate whether the clock quality is “acceptable” or “not acceptable” for the UE.

For clock quality metrics, the parameters refer to one or more of the following information: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state. There is a table in TS23.501 [2] to illustrate what these parameters mean:

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional)

| Information Name | Description |
| --- | --- |
| Synchronization state | Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1). |
| Clock quality |  |
| >> Traceable to GNSS | Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. |
| >> Traceable to UTC | Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. |
| >> Frequency stability | Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2). |
| >> Clock Accuracy | Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). |
| Parent time source | Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other". |
| NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process. |  |

Proposal 1:  Define the Clock quality detail level using CHOICE structure to indicate “acceptable/not acceptable indication” or “clock quality metrics” to UE in air-interface.

Proposal 2:  For “acceptable/not acceptable indication”, 1 bit flag should be defined. And for “clock quality metrics”, the following parameters should be defined using SEQUENCE structure:

- Synchronization state: Value of “Locked”, “Holdover”, or “Freerun”;

- Clock quality, including “Traceable to GNSS”, “Traceable to UTC”,  “Frequency stability”, and “Clock Accuracy”;

- Parent time source: Values of "SyncE", “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand_set”, “other”.

  - RRC state change mechanism from RRC_IDLE/RRC_INACTIVE

A leftover issue is whether gNB always broadcasts event ID.

Event ID is optional according to SA2.  It is agreed that under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.

Checking internally and offline with other gNB vendors and operators, it is clear that the on-demand SIB mechanism is not yet implemented or deployed in commercial use. And even so, all implementations always broadcast SIB9. So we could follow the assumption that this event ID can only be changed to different value, but it will always be in the SIB9 if the network supports it.

Note SA2 also seem to now share the same understanding since TS23.501 was updated accordingly (i.e. there is no longer a case where the NG-RAN gNB stops broadcasting the reference report ID in SIB9):

- When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB information a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB broadcasts the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available

Proposal 3: Confirm the event ID is always present in the SIB9 if the network supports it.

  - UE capability

To support the feature of 5GS network timing synchronization status and reporting, it is necessary to define a new AS capability for it. But since the NW has the necessary information to determine whether to send detailed clock quality information to a specific UE, the capability does not need to be indicated to the gNB.

Proposal 4: Define a new capability of 5GS network timing synchronization status and reporting feature in UE, and this capability does not need to be indicated to the gNB.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1:  Define the Clock quality detail level using CHOICE structure to indicate “acceptable/not acceptable indication” or “clock quality metrics” to UE in air-interface.

Proposal 2:  For “acceptable/not acceptable indication”, 1 bit flag should be defined. And for “clock quality metrics”, the following parameters should be defined using SEQUENCE structure:

- Synchronization state: Value of “Locked”, “Holdover”, or “Freerun”;

- Clock quality, including “Traceable to GNSS”, “Traceable to UTC”,  “Frequency stability”, and “Clock Accuracy”;

- Parent time source: Values of "SyncE", “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand_set”, “other”.

Proposal 3: Confirm the event ID is always present in the SIB9 if the network supports it.

Proposal 4: Define a new capability of 5GS network timing synchronization status and reporting feature in UE, and this capability does not need to be indicated to the gNB.

Reference

- RP-230754 New WID on NR Timing Resiliency and URLLC enhancements Nokia, Nokia Shanghai Bell

- TS 23.501-i22