3GPP TSG-RAN WG2 #123	R2-2307387

Toulouse, France, August 2023

Agenda Item:	7.9.4

Source:	NEC

Title:	Discussion on remaining CP issue of U2N multi-path relay

Document for:	Discussion, Decision

Introduction

During the previous two meetings, RAN2 has made the following agreement regarding to SL U2N multi-path relay

Progress made in RAN2#121bis-e - The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition. - In case of duplication, PDCP control PDU only transmits on the primary RLC entity same as legacy. - Non-split SRB1 and 2 over indirect path is not supported in Scenario 2. - Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path. - If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path. Details of the reporting mechanism can be further discussed. - When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1. - When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path - When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2. - When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path - The bearer type configuration is provided per SRB. It is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types (within the bearer types that are supported). - FFS if there are cases where the configuration of non-split SRBs over indirect path is useful. - In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition. Need for reporting in the idle/inactive case can be further discussed. FFS what ID is used. - WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE. - A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment. FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE. This agreement does not imply any conclusion on non-split SRB1 on indirect path. - A remote UE in multipath that is released to RRC_IDLE/RRC_INACTIVE can apply legacy cell/relay selection behaviour, thus moving to single-path operation on either path according to implementation. - LS to SA2 on authorization for multi-path Scenario 2 is approved in [105].

Progress made in RAN2#122 - For Scenario-1/2, PDCP duplication of DRB is controlled by legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE delivered via direct path. - For Scenario-1/2, RRC sets the initial state of PDCP duplication for split SRB/DRB as in legacy. - For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced. - For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery. - For Scenario-1/2, confirm the WA that: for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen. - For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover. - For Scenario-1/2, optionally configure UL data split threshold for split DRB. Usage of the threshold follows legacy behavior. - For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network. - For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior. - For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior. - FFS if these two timers are new or reuse the existing timers. - WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition. - LS to SA3 to check if there is any security concern with the WA is approved in [209]. - WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path. - WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure. - For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2. - For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE. - LS to R3 to notify this conclusion, with “take into account” action is approved in [212].

It can be observed that most of the work has been finished. However, there are still some leftover issues where we will address them in this contribution.

Discussion

During last meeting, it has been agreed that MCGFailureInformation could be used when remote UE reporting direct-path failure via indirect-path. However, companies could not ensure whether additional IE should be introduced for U2N relay scenario.

In details, the message definitions is given in the following:

-- ASN1START -- TAG-MCGFAILUREINFORMATION-START MCGFailureInformation-r16 ::= SEQUENCE { criticalExtensions CHOICE { mcgFailureInformation-r16 MCGFailureInformation-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } MCGFailureInformation-r16-IEs ::= SEQUENCE { failureReportMCG-r16 FailureReportMCG-r16 OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } FailureReportMCG-r16 ::= SEQUENCE { failureType-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, t312-Expiry-r16, lbt-Failure-r16, beamFailureRecoveryFailure-r16, bh-RLF-r16, spare1} OPTIONAL, measResultFreqList-r16 MeasResultList2NR OPTIONAL, measResultFreqListEUTRA-r16 MeasResultList2EUTRA OPTIONAL, measResultSCG-r16 OCTET STRING (CONTAINING MeasResultSCG-Failure) OPTIONAL, measResultSCG-EUTRA-r16 OCTET STRING OPTIONAL, measResultFreqListUTRA-FDD-r16 MeasResultList2UTRA OPTIONAL, ... } MeasResultList2UTRA ::= SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2UTRA-FDD-r16 MeasResult2UTRA-FDD-r16 ::= SEQUENCE { carrierFreq-r16 ARFCN-ValueUTRA-FDD-r16, measResultNeighCellList-r16 MeasResultListUTRA-FDD-r16 } MeasResultList2EUTRA ::= SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16 -- TAG-MCGFAILUREINFORMATION-STOP -- ASN1STOP

It could be found that in the traditional NR Uu case, UE needs to report the SCG measurement result via either measResultSCG or measResultSCG-EUTRA. However, for U2N multi-path relay case, there is no such SCG definition, on the other hand, U2N remote UE will only take the measurement of the PC5 link quality of the indirect link with U2N relay UE. Therefore, this IE cannot be fully reused. So RAN2 should consider to include the measurement result of the PC5 link quality of the indirect link into the MCGFailureInformation

RAN2 consider to include the measurement result of the PC5 link quality of the indirect link into MCGFailureInformation.

In addition, in the traditional NR Uu case, UE also needs to report the measurement result of neighbour cells in this message. The purpose of this information is to allow network to have an accurate decision on whether to switch the UE into SCG or other candidate cells. However, for U2N relay case, UE may be not only switched to other candidate cells, but also connect to other candidate U2N relay UEs. So RAN2 should consider whether U2N remote UE’s behaviour of switching to other candidate cells is enough. If not, the measurement result of other candidate relay UEs should also be included into this message.

RAN2 should discuss whether to include the measurement result of other candidate relay UEs into MCGFailureInformation.

During last meeting, there was a pending issue that whether to allow network release U2N relay UE’s configuration at U2N remote UE side when the U2N relay UE performing handover. Or U2N remote UE can suspend the indirect path until the handover procedure is finished.

In details, since currently we are discussing multi-path scenario, so whether the indirect link is suspended or not will not affect U2N remote UE’s service continuity. Furthermore, the temporary suspension of indirect link will not request U2N remote UE to perform measurement for either the U2N relay UE of indirect link, or other candidate U2N relay UEs, where it could be kind of power saving for U2N remote UE. Therefore, it is suggested that U2N remote UE suspends the indirect path upon reception of notification message indicating relay UE handover.

Suspension of indirect link will not cause service interruption of U2N remote UE under multi-path scenario.

Suspension of indirect link will not request U2N remote UE to perform measurement for either the U2N relay UE of indirect link, or other candidate U2N relay UEs, where it could be kind of power saving for U2N remote UE.

U2N remote UE suspends the indirect path upon reception of notification message indicating relay UE handover.

In addition, during last meeting, one remaining issue is about the UE behaviour upon the T304-like timer is expired. If one recalling the current T304 expiry behaviour, which is shown below:

| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync for the MCG which does not include sl-PathSwitchConfig, or upon reception of RRCReconfiguration message including reconfigurationWithSync for the SCG not indicated as deactivated in the NR or E-UTRA message containing the RRCReconfiguration message or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| --- | --- | --- | --- |

It clarifies that UE would initiate RRC re-establishment procedure when UE under DC case performing intra-NR handover or U2N remote UE performing indirect to direct path switch. To us, the current UE behaviour could be reused for Rel-18 direct path addition/change scenario.

U2N remote UE would initiate RRC re-establishment procedure upon the T304-like timer expired, when it performing direct path addition/change.

On the other hand, when U2N remote UE performing indirect path addition/change, it has been agreed that T420-like timer could be used. So one recall the current definition of T420 timer, which is shown below:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |

In general, T420 timer would be used when Rel-17 U2N remote UE performing direct to indirect path switch. Rel-17 U2N remote UE needs to perform RRC re-establishment procedure when T420 timer expired. However, in the multi-path scenario, it is not necessary for U2N remote UE to perform RRC re-establishment procedure since the direct link is still maintained. So the more efficient UE behaviour is to report the measurement result of other candidate relay UEs via direct link. While the stop condition could be reused.

For U2N multi-path scenario, U2N remote UE could report the measurement result of other candidate relay UEs via direct link when T420-like timer is expired.

The stop condition of T420 timer could be reused for T420-like timer.

Last but not least, there is no observed blocking issue to reuse both T304 timer and T420 timer, so the current timer could be reused.

Reuse T304 timer for direct path addition/change.

Reuse T420 timer for indirect path addition/change.

Conclusion and Proposal

In this paper, we have discussed the remaining issue of U2N relay multi-path, a brunch of observations and proposals are listed below:

Observation 1	Suspension of indirect link will not cause service interruption of U2N remote UE under multi-path scenario.

Observation 2	Suspension of indirect link will not request U2N remote UE to perform measurement for either the U2N relay UE of indirect link, or other candidate U2N relay UEs, where it could be kind of power saving for U2N remote UE.

Proposal 1	RAN2 consider to include the measurement result of the PC5 link quality of the indirect link into MCGFailureInformation.

Proposal 2	RAN2 should discuss whether to include the measurement result of other candidate relay UEs into MCGFailureInformation.

Proposal 3	U2N remote UE suspends the indirect path upon reception of notification message indicating relay UE handover.

Proposal 4	U2N remote UE would initiate RRC re-establishment procedure upon the T304-like timer expired, when it performing direct path addition/change.

Proposal 5	For U2N multi-path scenario, U2N remote UE could report the measurement result of other candidate relay UEs via direct link when T420-like timer is expired.

Proposal 6	The stop condition of T420 timer could be reused for T420-like timer.

Proposal 7	Reuse T304 timer for direct path addition/change.

Proposal 8	Reuse T420 timer for indirect path addition/change.

Reference

[1] R2-2306692		[AT122][419][Relay] Short ID in U2U relay (Ericsson)	Ericsson