3GPP TSG-RAN WG2 Meeting #123	R2-2307398

Toulouse, France, August 21-25, 2023

Agenda item:	7.4.2.3

Source: 	Fujitsu

Title: 	LTM cell switch execution and completion

Document for:	Discussion and decision

1. Introduction

This contribution will discuss the LTM cell switch execution and completion, involving:

- Content of the LTM cell switch command MAC CE

- UE processing after receiving the LTM cell switch command MAC CE

- LTM completion in case of RACH-less LTM

- Modifications to the LTM procedure

2. Discussion

2.1 Content of the LTM cell switch command MAC CE

RAN2 has no agreement on BWP information while the information about active DL and UL BWPs for target cell can be included in the LTM cell switch command MAC CE from RAN1 point of view. Considering that the first activate BWP indicated by RRC configuration is provided during LTM preparation phase and subsequent LTM is supported, load status of the BWP may be changed. The LTM cell switch fails if the first activate BWP is overloaded. In this case, BWP switching after LTM does not work. At least for intra-DU LTM, source DU has knowledge of load status of the BWP and can include BWP information in the MAC CE, e.g. if the first BWP is overloaded. RAN2 can confirm that the information about active DL and UL BWPs for target cells can be included in the LTM cell switch command MAC CE.

Proposal 1: Active DL and UL BWP(s) for the target cell(s) can be included in the LTM cell switch command MAC CE.

With similar reasons, we propose to indicate the SCell activation/deactivation state in the LTM cell switch MAC CE.

Proposal 2: In addition to existing SCell activation/deactivation mechanisms, the SCell activation/ deactivation state can be included in the LTM cell switch command MAC CE for intra-DU case.

In addition, we think that the UL carrier indicator can be included in the MAC CE. For example, in case of RACH-less LTM, this indicator is used to indicate the UL carrier for Configured Grant to transmit the first UL data in the target cell if CG is configured for both UL carriers.

Proposal 3: The UL carrier indicator can be included in the LTM cell switch command MAC CE.

2.2 UE processing after receiving cell switch command

The handling of activated SCells which are unchanged after the LTM cell switch needs to be discussed. For example, regarding UL synchronization, whether the UE can apply the same TA as the one before the cell switch in order to skip the RACH procedure and how to handle the TA timer.

Proposal 4: RAN2 to discuss how to handle the activated SCells which are unchanged after the LTM cell switch.

2.3 LTM completion in case of RACH-less LTM

UL grant for the first UL transmission in target cell

In LTM procedure, the gNB transmits a MAC CE triggering cell switch and the UE switches to the target cell. The UE completes the LTM cell switch procedure by sending its first UL transmission in target cell. For the first UL transmission, RAN2 agreed that Configured grant (CG) or dynamic grant (DG) can be used [1]. If DG is used, the target DU needs to be informed about the selected beam, so that the target DU can start scheduling dynamic UL grant. It means that DG is available for intra-DU LTM and whether it can be used for inter-DU LTM depends on RAN3 decision. So, for inter-DU LTM, CG configuration is required, and beam association should be included for CG configuration. With this, at least CG resource is available in case of inter-DU LTM.

Proposal 5: CG configuration including beam association should be configured for candidate cell(s) for inter-DU LTM.

In case of Intra-DU LTM, nothing is required for RAN3 to support DG option. So, at least DG is available for intra-DU LTM. Based on these, CG configuration is up to gNB implementation. In this case, beam association is not needed.

Proposal 5bis: CG configuration without beam association can be configured for candidate cell(s) for intra-DU LTM.

For Configured grant, there are CG type 1 and CG type 2. For CG type 1, CG resource is configured by RRC and the network will monitor CG resource upon configuration. The UE can consider CG resource as valid upon configuration or upon reception of LTM cell switch MAC CE. For CG type 2, in legacy, CG resource is configured by RRC and activated by DCI. For LTM, CG resource can be activated by cell switch command MAC CE or by DCI at the same time with the MAC CE transmission. If CG type 2 is used for the first UL transmission in target cell, reduced resource overhead is implemented, e.g. by common resource shared by multiple UEs. For CG type 2, the network and the UE will monitor CG resource upon CG activation and thus the target DU needs to be informed, like DG option. In this way, beam association is not required in CG type 2 configuration for both intra-DU and inter-DU.

Proposal 6: RAN2 to discuss which is used for the first UL transmission in target cell, CG type 1, CG type 2 or either.

The next issue for configured grand is which UL carrier is used for the first UL transmission in target cell, i.e. NUL or SUL. CG resource can be configured on either SUL or NUL by RRC and thus the UL carrier configured with CG resource will be used for the first UL transmission in target cell. Or, UL carrier for CG can be signalled explicitly, e.g. in RRC configuration or in LTM cell switch command. Alternatively, UL carrier is selected according to the RSRP of DL path loss reference.

Proposal 7: RAN2 to discuss which UL carrier is used for the first UL transmission in target cell for CG.

It is possible that the initial transmission for CG fails and the question is whether and how to perform the retransmission. In case CG transmission fails, retransmission can be CG or DG. The simple method is CG retransmission.

Proposal 8: CG retransmission for the first UL transmission in target cell is allowed.

According to RAN2 agreement, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE [1]. In Rel-17, unified TCI state is used for beam indication. The beam indicated in the LTM MAC CE, e.g. a pair of UL and DL unified TCI state index or joint unified TCI state index for the target cell can be used for the beam association. If joint TCI state index is included in the MAC CE, the UE selects the CG occasions associated to the joint TCI state index for the first UL data and the network knows the DL beam based on the CG occasion where the first UL data is received. If a separate UL/DL unified TCI state is used, the UE can select the CG occasions associated to the DL TCI state index and transmit the first UL data by the beam associated to the indicated UL unified TCI state. Then the network knows the DL beam according to the CG occasion and the UL beam according to the beam where the first UL data is received. Alternatively, SSB information associated to TCI state is used for the association, like CG-SDT.

Proposal 9: RAN2 to discuss how the UE determines the CG occasion used for the first UL transmission in target cell.

Successful reception of the first UL transmission in target cell

For LTM completion, the following options on how UE to determine the successful reception of its first UL data by the network are on the table:

- Option 1: RLC ACK of RRCReconfigurationComplete message

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution identify MAC CE

Option 1 incurs more latency. Option 3 is used for NTN but we don’t see the motivation to use it for LTM. So, Option 2 is preferred.

If Configured Grant is used for initial transmission or retransmission of the first UL transmission, C-RNTI addressed PDCCH can identify the successful reception of the first UL transmission in target cell. If Dynamic Grant scheduling via C-RNTI addressed PDCCH is used for initial transmission or retransmission of the first UL transmission, C-RNTI addressed PDCCH cannot identify the successful reception since it can be used to schedule UL grant for the first UL transmission. In this case, C-RNTI addressed PDCCH scheduling a new transmission is used to identify the successful reception at gNB.

Proposal 10: For LTM completion in case of RACH-less LTM, if CG is used for (re)transmission of its first UL data, the UE determines the successful reception of its first UL data by the network when C-RNTI addressed PDCCH is received.

Proposal 10bis: For LTM completion in case of RACH-less LTM, if DG is used for (re)transmission of its first UL data, the UE determines the successful reception of its first UL data by the network when C-RNTI addressed PDCCH scheduling a new transmission is received.

Considering mobility latency, a new MAC CE indicating the successful reception of the first UL data at gNB can be introduced so that there is DL data and PDCCH scheduling the new MAC CE can be transmitted immediately.

Proposal 11: A new MAC CE is introduced to indicate the successful reception of the first UL data to the UE.

2.4 Modifications to the LTM procedure

At RAN2#122 meeting, Figure 1 is endorsed as baseline for the procedure of LTM:

Figure 1: Signalling procedure for LTM [2]

According to RAN2 agreements, in RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA or on the reception of Msg3/MsgA in CBRA while the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. So, in case of RACH-based LTM, some steps in RACH procedure, e.g. the reception of preamble in CFRA or the reception of Msg3/MsgA in CBRA, are a part of LTM completion phase rather than LTM execution phase. However, RACH procedure is included in the LTM execution phase in the signaling procedure for LTM in the running CR. So, we propose to modify this to align with RAN2 agreements.

Proposal 12: RAN2 to discuss which one of the following changes will be applied in the signaling procedure for LTM:

- Alt.1: to remove the box of LTM completion

- Alt.2: to include RACH Procedure in the box of LTM completion

3. Conclusion

In this contribution, we have some discussions on the LTM cell switch execution and completion of the L1/L2 triggered mobility procedure and the following proposals are made:

Content of the LTM cell switch command MAC CE

Proposal 1: Active DL and UL BWP(s) for the target cell(s) can be included in the LTM cell switch command MAC CE.

Proposal 2: In addition to existing SCell activation/deactivation mechanisms, the SCell activation/ deactivation state can be included in the LTM cell switch command MAC CE for intra-DU case.

Proposal 3: The UL carrier indicator can be included in the LTM cell switch command MAC CE.

UE processing after receiving cell switch command

Proposal 4: RAN2 to discuss how to handle the activated SCells which are unchanged after the LTM cell switch.

UL grant for the first UL transmission in target cell

Proposal 5: CG configuration including beam association should be configured for candidate cell(s) for inter-DU LTM.

Proposal 5bis: CG configuration without beam association can be configured for candidate cell(s) for intra-DU LTM.

Proposal 6: RAN2 to discuss which is used for the first UL transmission in target cell, CG type 1, CG type 2 or either.

Proposal 7: RAN2 to discuss which UL carrier is used for the first UL transmission in target cell for CG.

Proposal 8: CG retransmission for the first UL transmission in target cell is allowed.

Proposal 9: RAN2 to discuss how the UE determines the CG occasion used for the first UL transmission in target cell.

Successful reception of the first UL transmission in target cell

Proposal 10: For LTM completion in case of RACH-less LTM, if CG is used for (re)transmission of its first UL data, the UE determines the successful reception of its first UL data by the network when C-RNTI addressed PDCCH is received.

Proposal 10bis: For LTM completion in case of RACH-less LTM, if DG is used for (re)transmission of its first UL data, the UE determines the successful reception of its first UL data by the network when C-RNTI addressed PDCCH scheduling a new transmission is received.

Proposal 11: A new MAC CE is introduced to indicate the successful reception of the first UL data to the UE.

Modifications to the LTM procedure

Proposal 12: RAN2 to discuss which one of the following changes will be applied in the signaling procedure for LTM:

- Alt.1: to remove the box of LTM completion

- Alt.2: to include RACH Procedure in the box of LTM completion

References

- Chairman’s notes for RAN2#122

- R2-2306957, ‘38.300 running CR for introduction of NR further mobility enhancements’, MediaTek Inc., vivo, RAN2#122