3GPP TSG-RAN WG2 Meeting #123	R2-2307403

August 21st – 25th, 2023, Toulouse, France

Agenda item:	7.9.4

Source: 	Fujitsu

Title: 	Discussions on multi-path

Document for:	Discussion and decision

Introduction

In RAN2#121 meeting, there are following agreements on Multi-path relaying [1].

Agreements:

As a baseline, direct path addition for multi-path is a path switch procedure in which the target configuration contains both paths.

Upon direct path addition for multi-path, one of the serving cells of the added direct path is configured as PCell for the remote UE.

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

In RAN2#121bis-e meeting, there are following agreements [2]:

Agreements:

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

In RAN2#122 meeting, the following agreements are made on multi-path procedure [3]:

Agreements:

For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

FFS if these two timers are new or reuse the existing timers.

In this contribution, we would like to discuss some further issues in multi-path relaying for Scenario-1.

Discussion

Indirect path addition

In the RAN2#121 meeting, for indirect path addition or change in scenario 1, it is agreed that a PC5-RRC trigger is specified to bring the idle/inactive relay UE to RRC_CONNECTED at least for the cases where SRB1/split SRB1 is in the indirect path.

An issue to be considered is the condition that the remote UE sends the PC5-RRC trigger to the relay UE. We assume that there could be the following options:

- Option 1: If the relay UE is in RRC_Idle/ RRC_Inactive state, the remote UE sends a PC5-RRC trigger to make relay UE enter RRC_Connected state.

In this option, the relay UE may need to inform the remote UE of the relay UE’s RRC state, i.e., whether the relay UE is in RRC_Idle/ RRC_Inactive state.

- Option 2: If the remote UE receives an indication from network, it sends a PC5-RRC trigger to make relay UE enter RRC_Connected state.

In this option, in the indirect path addition message, an indication can be included to make the remote UE send the PC5-RRC trigger.

- Option 3: The remote UE always sends a PC5-RRC trigger to make relay UE enter RRC_Connected state regardless of relay UE’s RRC state if the RRCReconfigurationComplete message is transmitted only via the direct path.

In this option, on receiving the PC5-RRC trigger from the remote UE, the relay UE in RRC_Connected state can ignore the trigger.

In these three options, Option 1 needs new indication of the relay UE’s RRC state in PC5 and Option 2 needs new indication in Uu RRC messages for indirect path addition/change. Option 3 does not introduce new signalling, however it could have signalling overhead on the PC5 interface between the remote UE and the relay UE, since the PC5-RRC trigger is always transmitted by the remote UE if the RRCReconfigurationComplete message is transmitted only via the direct path. Besides, Option3 requires the new behavior for the relay UE.

We assume that Option 2 is simple and has relatively small impact on the standard. So, we prefer Option 2.

Proposal 1: For bringing the idle/inactive relay UE to RRC_CONNECTED, if the remote UE receives an indication from network, it sends a PC5-RRC trigger to make relay UE enter RRC_Connected state.

The following Figure 1 shows an example of indirect path addition.

Figure 1 Example of indirect path addition

As agreed in the RAN2#121bis-e meeting, the remote UE sends the RRC reconfiguration complete message to gNB via the direct path for Scenario 1 if one of the conditions is met: when split SRB1 with duplication is configured, when primary RLC entity of split SRB1 is on direct path, or when non-split SRB1 is configured on direct path.

An issue to be considered is when the RRC reconfiguration complete message via the direct path should be transmitted by the remote UE. In our opinion, the RRC reconfiguration complete message can be transmitted once the remote UE performs the configurations in the RRC reconfiguration to add the indirect path in step 3 in the figure above. This is simple and similar to SCG addition in NR-DC.

Proposal 2: RRC reconfiguration complete message is transmitted on the direct path once the remote UE performs addition of the indirect path when split SRB1 with duplication is configured or when primary RLC entity of split SRB1 is on direct path or when non-split SRB1 is configured on direct path.

RAN2 has agreed to use T420-like timer for indirect path addition and change in the last meeting, and the T420-like timer can be used to determine the success/failure of indirect path addition procedure. On the stop condition of this timer, there may be some options:

- Option 1: the RRC reconfiguration complete message is transmitted successfully in the indirect path at least in case that split SRB1 with duplication is configured on the indirect path. This option is similar to the stop condition of the current T420 in direct-to-indirect path switch.

- Option 2: the remote UE has established PC5-RRC connection with the relay UE.

- Option 3: the remote UE receives an indication from the relay UE that the relay UE has successfully connected with the gNB.

In our opinion, option 1 may not be applicable to the case that the RRC reconfiguration complete message is transmitted only via the direct path. For option 2, since the idle/inactive relay UE may fail to enter RRC_CONNECTED or the relay UE may change its serving gNB during path addition, this option cannot ensure that the relay UE can serve the remote UE successfully. So, we prefer option 3. When the relay UE has successfully connected with the gNB of the direct path, the relay UE can transmit an indication to the remote UE, and the remote UE would stop the T420-like timer and consider that the indirect path addition is successful.

Proposal 3: When the remote UE receives an indication from the relay UE that the relay UE has successfully connected with the gNB, the remote UE stops the T420-like timer.

When the T420-like timer expires, the remote UE would consider that the indirect path addition fails. Another issue to be discussed is the remote UE behavior when the indirect path addition fails. In Release 17 direct-to-indirect path switch, the remote UE would initiate RRC re-establishment in these cases. From our perspective, in case of indirect path addition failure, RRC re-establishment might not be needed since the direct path can still work. An option is that the remote UE can send a failure indication to the gNB via the direct path to indicate the failure of indirect path addition. This failure indication can be an RRC message, such as SCG failure information.

Proposal 4: The remote UE sends a failure indication to the gNB via the direct path when the T420-like timer expires.

For sending the failure indication, the remote UE can fall back to the original single path configuration.

Proposal 5: The remote UE falls back to the original single path configuration if indirect path addition fails.

Direct path addition

In RAN2#121 meeting, RAN2 agrees that direct path addition for multi-path is a path switch procedure in which the target configuration contains both paths as a baseline. The following Figure 2 shows an example of direct path addition for the case where SRB1/split SRB1 is still configured in the indirect path. In this case, the RRC reconfiguration complete message from the remote UE would be transmitted via the indirect path. RAN2 has agreed that a T304-like timer is used for direct path addition.

Figure 2 Example of direct path addition

An FFS in the last meeting is about the remote UE behavior when the T304-like timer expires (i.e., direct path addition failure). In Release 17 indirect-to-direct path switch procedure, the remote UE will initiate RRC re-establishment when T304 expires. For direct path addition in multi-path, the remote UE can send a failure indication to the network, instead of initiating RRC re-establishment, in order to save the time of service interruption. This failure indication can be an RRC message, such as MCG failure information.

Proposal 6: The remote UE sends a failure indication to the gNB via the indirect path when the T304-like timer expires.

Similarly, the remote UE can fall back to the original single path configuration, i.e., indirect path configuration when the T304-like timer expires.

Proposal 7: The remote UE falls back to the original single path configuration when the T304-like timer expires.

Direct path release

The remote UE’s direct path and indirect path may be in the same cell or in different cells under the same gNB. In Multi-path relaying, RAN2 has agreed that PCell is on the direct path of the remote UE.

For the direct path release procedure, we assume that the PCell of the remote UE will be changed to the PCell of the relay UE, since the PCell in the direct path will be released after the direct path release. Thus, the direct path release procedure should also be a path switch procedure with only indirect path configuration in the RRC reconfiguration message.

Proposal 8: The PCell of the remote UE is changed to the PCell of the relay UE after direct path release.

Proposal 9: Path switch procedure can be used for direct path release.

RAN2 has agreed that Mode 1 resource allocation can be supported for the remote UE configured with multi-path in Scenario 1 in principle. In addition, in Release 17 SL relay, only Mode 2 resource allocation can be used by the remote UE which has been connected to network via a relay UE. If the remote UE is configured with Mode 1 resource allocation in multi-path, it can be discussed how the remote UE works with Mode 2 resource allocation after direct path release. We think that the Mode 2 configuration should be included in the direct path release message in this case.

Proposal 10: Mode 2 configuration should be included in the direct path release message if the remote UE is configured with Mode 1 in multi-path.

Path activation/deactivation

The operations of path addition, path release or path change are typically performed via RRC procedures. If the remote UE and/or the relay UE are moving fast, these operations may be performed frequently, which would be relatively slow and may cause higher signalling overhead. Therefore, it is proposed to use MAC CE to activate/deactivate the path more efficiently, which is similar to SCell activation/deactivation or duplication activation/deactivation.

Proposal 11: Path activation/deactivation is supported in multi-path.

Since the MAC entity in the indirect path is hop-by-hop, the MAC CE can be transmitted via the direct path and used for path activation/deactivation of the indirect path.

Proposal 12: MAC CE via the direct path is used for path activation/deactivation of the indirect path.

Conclusion

In this contribution, we have discussed some multi-path related issues. We have the following proposals:

Proposal 1: For bringing the idle/inactive relay UE to RRC_CONNECTED, if the remote UE receives an indication from network, it sends a PC5-RRC trigger to make relay UE enter RRC_Connected state.

Proposal 2: RRC reconfiguration complete message is transmitted on the direct path once the remote UE performs addition of the indirect path when split SRB1 with duplication is configured or when primary RLC entity of split SRB1 is on direct path or when non-split SRB1 is configured on direct path.

Proposal 3: When the remote UE receives an indication from the relay UE that the relay UE has successfully connected with the gNB, the remote UE stops the T420-like timer.

Proposal 4: The remote UE sends a failure indication to the gNB via the direct path when the T420-like timer expires.

Proposal 5: The remote UE falls back to the original single path configuration if indirect path addition fails.

Proposal 6: The remote UE sends a failure indication to the gNB via the indirect path when the T304-like timer expires.

Proposal 7: The remote UE falls back to the original single path configuration when the T304-like timer expires.

Proposal 8: The PCell of the remote UE is changed to the PCell of the relay UE after direct path release.

Proposal 9: Path switch procedure can be used for direct path release.

Proposal 10: Mode 2 configuration should be included in the direct path release message if the remote UE is configured with Mode 1 in multi-path.

Proposal 11: Path activation/deactivation is supported in multi-path.

Proposal 12: MAC CE via the direct path is used for path activation/deactivation of the indirect path.

References

- Report from session on positioning and sidelink relay, Session Chair (MediaTek), RAN2#121

- Report from session on positioning and sidelink relay, Session Chair (MediaTek), RAN2#121bis-e

- Report from session on positioning and sidelink relay, Session Chair (MediaTek), RAN2#122