3GPP TSG-RAN WG2 Meeting #123	R2-2307407

21st – 25th Aug 2023

Toulouse, France

Source:                    	DENSO CORPORATION

Title:  	Discussion on NR-DC with subsequent CPAC.

Document for:        	Discussion and decision

Agenda Item:         	7.4.3	NR-DC with selective activation cell of groups

- Introduction

At RAN2 #122[1], the following was agreed.

- For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPAC.

- FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPAC

- The UE shall skip the condition evaluation for a candidate which is a current PScell.

- The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

- Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

- Rapporteur take initiative on naming offline

- In this paper, we discussed about more detail of NR-DC with selective activation of the cell groups.

- Terminology is “Subsequent CPAC”

In this paper, we discussed about more detail of NR-DC with selective activation of the cell groups.

- Discussion

2.1 Management of candidate PSCell configurations

In RAN2-121bis-e meeting[2], RAN2 agreed that source MN generates the execution conditions for the initial CPAC in MN-initiated inter-SN SCG selective activation. However, the execution configuration for subsequent CPAC is FFS on the following options.

- Option 1: Source MN generates the execution conditions for all subsequent CPAC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPAC.

However, in last meeting, RAN2 agreed that the who generate the execution condition for MN-initiated inter-SN SCG selective activation is FFS. Therefore, we think RAN2 to discuss more detail of the execution condition generation procedure for MN-initiated inter-SN SCG selective activation.

In our opinions, in subsequent CPAC, since RAN2 agreed that the UE keeps all execution configurations after CPC, the UE also keeps the execution condition for initial CPC. In Option 1, since source MN generates the execution condition for initial CPC and all subsequent CPAC, the UE can evaluate the execution condition with the legacy MN-initiate inter-SN CPC procedure even after initial CPC. However, in subsequent CPAC, all execution condition is generated by source MN based on event A4 and it may lead to unsuitable PSCell change. In Option 2, since the candidate SNs generate execution condition for subsequent CPAC, the candidate SN can decide the suitable condition that the UE should leave this PSCell after initial CPC. However, the UE has both execution condition generated by MN for initial CPC and execution condition generated by SN for subsequent CPAC and then, the UE may not evaluate the condition with legacy procedure. In our opinion, Option 2 has the more complexity and may require many legacy procedure changes, and then as the simplest solution, RAN2 to adopt Option 1 for that the UE evaluate only the execution condition generated by the source MN.

Table.1 Comparison of Option 1 and Option 2

| Option | Pros. | Cons. |
| --- | --- | --- |
| Option 1 | the UE can evaluate the execution condition with the legacy MN-initiate inter-SN CPC | all execution condition is generated by source MN based on event A4 and it may lead to unsuitable PSCell change |
| Option 2 | the candidate SN can decide the suitable condition that the UE should leave this PSCell after initial CPC | the UE has both execution condition generated by MN for initial CPC and execution condition generated by SN for subsequent CPAC |

Proposal 1: RAN2 to adopt Option 1 for MN-initiated inter-SN SCG selective activation.

Furthermore, in RAN2-121bis-e meeting, RAN2 agreed to support the execution condition based on event A4 for MN-initiated procedure, however it was FFS whether RAN2 support event A3/A5. If RAN2 support execution condition based on event A3/A5 for MN-initiated procedure, the source MN can generate more suitable condition for subsequent CPAC than execution condition based on event A4. Therefore, RAN2 to support the execution condition based on event A3/A5 for MN-initiated procedure.

Proposal 2: RAN2 to support the execution condition based on event A3/A5 for MN-initiated procedure.

In RAN2 #119bis-e meeting [3], it was agreed that the UE keeps the conditional reconfiguration of the candidate PSCells, however, it was not discussed how to release the configuration.

In RAN2-121bis-e meeting, RAN2 discussed about keep/release procedure for the execution conditions, and agreed that it should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

- UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

According to the current procedure, the NW can indicate removal or modification of the configuration to the UE. Thus, the additional RRC signaling would be necessary for release of the configuration and it may lead signaling overhead. Therefore, it is worth to discuss releasing method of the configuration in subsequent CPAC. The one way is the UE trigger-based method in which if some conditions (e.g. timer, measurement result) are fulfilled at the UE side, the UE assumes that the configuration is invalid. We propose RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell.

Proposal 3: RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell.

Furthermore, if the UE fail the random access to target-SN, the UE sends SCG Failure Information to NW for indication of random access problems. However, in selective activation of PSCell scenario, it may occur some problems since the UE maintains conditional reconfiguration for subsequent PSCell change. Therefore, we proposed RAN2 to discuss the procedure for Random Access Failure.

Proposal 4: RAN2 to discuss the procedure for Random Access Failure.

2.2 Procedure of Subsequent CPAC

In RAN2#119bis-e meeting, RAN2 agreed the UE continues the evaluating of the execution condition of candidate cell, however, the timing when the UE starts evaluating of the execution condition was not discussed.

- Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPAC, the UE continues evaluating the execution conditions of other candidate PScells. - Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

In Rel-17, the UE starts the evaluation of execution condition upon receiving conditional reconfiguration from MN. However, in subsequent CPAC, the UE needs to start evaluation of execution condition without indication from MN after PSCell change for subsequent PSCell change. Therefore, we think RAN2 should clarify when to start condition evaluation for subsequent PSCell change.

Proposal 5: RAN2 to clarify when to start condition evaluation for subsequent PSCell change.

2.3 Security Issue

In SA3-111 meeting, SA3 agreed the maintenance of security key for SNs as below[5]:

Following is the solution details from security perspective:

1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC.

2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

SA3 is offering a joint conference call that can be used to discuss and to enhance the above described solution details.

To RAN2

ACTION: 	SA3 requests RAN2 to take a note of the above status of security analysis and to provide feedback on the above solution details.

In SA3 discussion, the MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC, and then, the UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. However, in Rel-17, the MN can set only one sk-counter value in RRCReconfiguration message for each SN and the UE does not have the variable to store the sk-counter values for each SN. Therefore, the list of sk-counter values for each SN is needed for the UE and RRCReconfiguration message for this security solution.

Proposal 6: The list of sk-counter values for each SN is needed for the UE and RRCReconfiguration message.

In Rel-17, the UE derive the new KSN when sk-counter value is included in RRCReconfiguration received from MN (or applying the RRCReconfiguration stored in condRRCReconfig in a case of CPC). However, in subsequent CPAC, the UE needs to select unused sk-counter value from the list of sk-counter value upon every SN change. Furthermore, in subsequent CPC, since the sk-counter value may not be included in RRCReconfiguration stored in condRRCReconfig, UE cannot start to derive the KSN upon applying the RRCReconfiguration stored in condRRCReconfig. From these reasons, RAN2 need to the procedure of KSN derivation for subsequent CPAC.

Proposal 7: RAN2 to discuss the procedure of KSN derivation for subsequent CPAC, such as selection of unused sk-counter in UE and, the start timing of KSN derivation for subsequent CPC

In this scenario, for ensuring the security communication between the UE and the SN, the SN needs to set the same security key with UE’s one. Therefore, security key update mechanism is needed for the SN after every SN Changes same as the UE.

Observation 1: SN needs to set the same security key with UE when the UE updates the own security key.

However, in our understanding, the security key for the SN is indicated by SN Addition Request in Rel-17 inter-SN CPC. Therefore, if the UE updates own security key upon SN changes, the SN cannot detect the security key change and may set the different key from UE’s one. This may cause communication error between the UE and the SN. As the simplest solution is that the MN send the new key to SN for ensuring the security communication. However, in this solution, MN need to know the latest sk-counter which selected by UE. If the UE has two or more unused sk-counter values for target-SN, the MN (or SN) cannot identify the security key used in UE.  From these reasons, the UE need to send the using sk-counter value (or KSN) to MN for security key update on SN. Therefore, we think the sharing mechanism of security key between the UE and the SN is needed for selective activation of PSCell.

Proposal 8: Sharing mechanism of security key between the UE and SN is needed for subsequent CPC. For example, UE sends the using sk-counter value (or KSN) to MN for security key update on SN.

- Summary and proposal

This paper discussed the procedure of the subsequent conditional PSCell change after a cell group change. In summary, the followings were proposed:

Observation 1: SN needs to set the same security key with UE when the UE updates the own security key.

Proposal 1: RAN2 to adopt Option 1 for MN-initiated inter-SN SCG selective activation.

Proposal 2: RAN2 to support the execution condition based on event A3/A5 for MN-initiated procedure.

Proposal 3: RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell.

Proposal 4: RAN2 to discuss the procedure for Random Access Failure.

Proposal 5: RAN2 to clarify when to start condition evaluation for subsequent PSCell change.

Proposal 6: The list of sk-counter values for each SN is needed for the UE and RRCReconfiguration message

Proposal 7: RAN2 to discuss the procedure of KSN derivation for subsequent CPAC, such as selection of unused sk-counter in UE and, the start timing of KSN derivation for subsequent CPC

Proposal 8: Sharing mechanism of security key between the UE and SN is needed for subsequent CPC. For example, UE sends the using sk-counter value (or KSN) to MN for security key update on SN.

- References

[1] R2-2307001, “RAN2#122 Meeting Report,” ETSI MCC.

[2] R2-2304601, “RAN2#121bis-e Meeting Report,” ETSI MCC.

[3] R2-2211101, “RAN2#119bis-e Meeting Report,” ETSI MCC.

[4] RP-213565	“New WID on Further NR mobility enhancements,” MediaTek.

[5] S3-233200,	“LS on Security Solution for Selective SCG,” Nokia, Nokia Shanghai Bell