3GPP TSG-RAN WG2 Meeting #123		R2-2307419

Toulouse, France, August 21–25, 2023

Source:	CATT

Title:	Discussion on RACH-less and common (C)HO configuration

Agenda Item:	7.7.4.2

Document for:	Discussion and Decision

Introduction

In RAN2 previous meeting, preliminary agreements were made on HO enhancements for NTN [1]. In this document, we focus on the RACH-less and common (C)HO for HO enhancements in NTN. Our observations as well as proposals are provided.

Discussion

2.1 RACH-less

In RAN2#122 meeting, various agreements were achieved for RACH-less. Based on the agreement, there are some FFS left to further discussion.

- 2.1.1	RACH-less failure handling

Some companies think when RACH-less fails the UE can switch to RACH-based HO considering the RACH-less HO failure may be caused by UL synchronization failure. So, it may be beneficial on falling back to RACH-based HO to achieve high HO successful rate.

However, if following LTE mechanism, the UE will perform RRC-reestablishment when T304 expires. That is, the UE will perform RACH procedure anyway. So, there is no need to define one fallback mechanism which brings complexities to the procedure.

Proposal 1: For RACH-less HO, if the UE doesn’t receive the UE Contention Resolution Identity MAC CE until the T304 expiry, UE will initiate the RRC re-establishment procedure like LTE.

- 2.1.2	Pre-allocated UL grant handling

It has been agreed that pre-allocated UL grant can be applied in RACH-less HO. But in NR, some other features, for example, autonomous (re)transmission is supported in NR-U and CG-SDT. In the following, we discuss the autonomous retransmission.

In CG-SDT, it may happen that the network does not receive the initial transmission of CG-SDT and no feedback is sent to the UE from the network. So, one timer cg-SDT-RetransmissionTimer was defined. When cg-SDT-RetransmissionTimer expires, the UE will perform autonomous retransmission for the initial transmission of CG-SDT.

The similar case also exists in RACH-less. That is the network does not receive the initial transmission for RRCReconfigurationComplete message and no retransmission scheduling from the network can be performed. Therefore, in order to improve the transmission robustness, autonomous retransmission is needed. So it is suggested define one retransmission Timer in pre-allocated uplink grant for RACH-less HO.

Proposal 2: For pre-allocated UL grant in RACH-less HO, define one retransmission timer.

Proposal 3: When the retransmission timer in RACH-less expires, the UE performs autonomous retransmission for RRCReconfigurationComplete on the pre-allocated UL grant.

- 2.1.3	HARQ mode

In Rel-17 NTN, HARQ mode B was defined. In HARQ mode B, the UE does not expect retransmisison scheduling for one uplink HARQ process. In the following we will discuss the applicable HARQ mode for  dynamicUL grant and pre-allocated UL grant in RACH-less.

For dynamic UL grant, retransmission can only be performed based on network scheduling. Hence, HARQ mode B is not suitable for UL grant scheduled by PDCCH.

For pre-allocated UL grant, only one shot transmission, i.e. RRCReconfigurationComplete, is needed. Per Proposal 3, the UE can perform retransmission based on the new defined retransmission timer even there is no network scheduled retransmission. And the UE assumes successful RACH-less HO when the UE receives UE Contention Resolution Identity MAC CE. It seems HARQ mode B is applicable for pre-allocated UL grant. However, it can’t be excluded that the network schedules retransmission for the initial transmission of the UE to guarantee the reliability of RRCReconfigurationComplete message. Hence, the UE is expecting retransmission in this case, i.e. HARQ A mode should be applied.

There are two solutions to ensure that the UE works in HARQ mode A during RACH-less.

- Solution 1: The network to make sure that the HARQ process(es) used during the RACH-less is(are) configured with HARQ mode A, e.g. via HO command.

- Solution 2: The UE ignores the configured HARQ mode and assumes the HARQ process(es) is(are) HARQ mode A during RACH-less procedure.

Compared with Solution 1, Solution 2 is simple and does not bring extra complexities and signalling overhead to the network. So we slightly prefer Solution 2.

Hence, it is proposed that:

Proposal 4 In RACH-less HO, the UE ignores the configured HARQ mode and assumes the HARQ process(es) is(are) HARQ mode A during RACH-less procedure for both pre-allocated UL grant and PDCCH scheduled UL grant.

- 2.1.4	Combining with PCI unchanged scenario

In RAN2#121bis-e meeting [2], it was agreed that:

- In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1.

It can be seen that satellite switching without PCI changing (not requiring L3 mobility) is supported at least for hard satellite switch.

In PCI unchanged scenario, the UE can be switched to the target satellite with the same cell configuration except the satellite specific information. So, the UE can keep the UE specific configuration unchanged, i.e. the UE does not need to perform reconfiguration, except TA acquisition as well as DL/UL synchronization in PCI unchanged scenario. Thus, it can be seen that PCI unchanged scenario is one specific case of inter-satellite handover with same gateway/gNB.

Observation 1: PCI unchanged scenario is one specific case of inter-satellite handover with same gateway/gNB.

And it has been agreed in RAN2#121bis-e meeting that:

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

Based on the agreements above, it can be seen that RACH-less can be supported for inter-satellite HO with same gateway/gNB. Hence, taking Observation 1 into account, RACH-less can also be combined with PCI unchanged scenario. In this case, the network could indicate implicitly or explicitly NTA of the target cell to the UE like RACH-less handover, e.g. using other signalling instead of handover command. If RACH-less is combined with PCI unchanged scenario, the UE does not need to perform RACH procedure to acquire TA but just DL synchronization procedure. This simplifies the whole procedure in inter-satellite HO with the same gateway/gNB and is beneficial to latency reduction for PCI unchanged scenario.

Observation 2: It simplifies the switch procedure and is beneficial to latency reduction to combine the RACH-less with the unchanged PCI scenario.

However, considering the limited time for the R18 discussion, if the RACH-less applied for unchanged PCI scenario, how to indicate the NTA needs to be discussed, so even though it is feasible from the technical point of view, we propose the combination of the RACH-less and unchanged PCI scenario is not discussed in R18. It could be discussed in R19.

It is proposed that:

Proposal 5: RAN2 doesn’t discuss the combination of RACH-less with unchanged PCI scenario further in R18 due to the limited time.

2.2	 Common (C)HO configuration delivery

In RAN2#122 meeting, the following agreements were agreed for common (C)HO configuration delivery, which are shown below:

- Come back to the proposal to broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling) after feedback from RAN3

- Send al LS to RAN3 asking whether, in case target cell’s servingCellConfigCommon is broadcast in the source cell (as common (C)HO signalling), the target cell’s servingCellConfigCommon can be transferred to the source cell in the inter-gNB HO case in R18

Last meeting, some companies thought there were impacts on RAN3 for common configuration for target cells, so RAN2 drafted LS to RAN3 to ask for the feasibility for inter-gNB scenario [3]. Considering it is more popular that the NTN is deployed via intra-gNB in sight of the vast area covered by the satellite, we can focus on the scenario of intra-gNB in RAN2.

For intra-gNB scenario, one solution, i.e. unchanged PCI scenario, has been agreed considering it brings more benefits on overhead reduction with less L2 impact. And some companies think common (C)HO configuration is similar to the unchanged PCI scenario and one unified solution is enough. But, unchanged PCI scenario is only applicable for earth-fixed cell, which is one specific case of intra-gNB. Hence, some requirements are necessary for network and UE implementation for unchanged PCI scenario. This is not needed for common (C)HO. To leave more flexibility to UE and network, solutions can be studied for common (C)HO configuration.

Hence, it is proposed that:

Proposal 6: RAN2 first focuses on intra-gNB scenario for common (C)HO configuration.

If proposal 6 is agreed, the following issues can be further discussed in RAN2.

- 2.2.1	Parameters transmitted in the common configuration for neighbor cells

During current HO procedure, the target cell will transfer common information, i.e. field of spCellConfigCommon, to UE via dedicated RRC signaling. Actually, this part is the same for all the UEs which perform HO to the same target cell.

Besides, considering this common information is broadcasted by the target cell via SIB1, there is no security issue if it is provided via broadcast by the serving cell.

So, it is obvious that sending servingCellConfigCommon of the target cells via common signaling brings signaling reduction from the network perspective.

Proposal 7: The serving cell broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling).

- 2.2.2	Mechanism for transmission of common (C)HO configuration

If the common (C)HO configuration is broadcasted in SIB, two options can be further studied:

- Option 1: Using existing SIB to transfer the common (C)HO configuration;

- Option 2: Defining one new SIB to transfer the common (C)HO configuration.

In fact, the common (C)HO configuration is only used to transfer the common configuration of the target cell for the HO procedure. If the information is included in existing SIBs, both inactive/idle and connected UEs have to receive the common (C)HO configuration, even if it is not necessary for these UEs. This introduces negative impacts to the UEs which does not support or have no interests on common (C)HO configuration. Hence, it is not suitable to include this information in the existing SIB.

On the other hand, when new SIB is defined to transfer the common (C)HO configuration, only the UE which is interested in the information will acquire the information. There will no influence to the UE which does not support or has no interest on the common (C)HO configuration mechanism. And if new SIB is defined, the legacy mechanism can be reused, i.e. the SIB scheduling information is included in SIB1 and the UE which is interested in the information can acquire the information based on the scheduling information in SIB1.

Proposal 8: Define one new SIB, i.e. SIBx, to transfer the common (C)HO configuration.

Considering the trajectory of the satellite can be predicted, most UEs will perform handover to only one or two adjacent target cell(s) in the upcoming direction. Hence, the network needs to broadcast the common configuration of these adjacent target cell(s) considering it is high probability the UE will be switched to these cells. And common configuration of the other neighbour cells is not needed to be broadcasted.

Proposal 9: The network broadcasts the common configuration of the adjacent target cell(s) based on the trajectory of the satellite, the number is FFS.

- 2.2.3	Procedure for transmission of common (C)HO configuration

In Figure 1, we illustrate the procedure for how common (C)HO configuration works. The procedure can be summarized as following:

Step 1: The UE acquires the scheduling information for common (C)HO configuration from SIB1 upon particular time, e.g. based network indication or one duration before t-Service.

The scheduling information for (C)HO may include the window length where SIBx is broadcasted, the periodicity of SIBx, wherein SIBx indicates the common (C)HO configuration per Proposal 8.

Considering the common (C)HO configuration is only applied for handover execution, so it only requires to be provided before the time of the handover execution. So it doesn’t need to be broadcast periodically continuingly as other SIBs.

For quasi-Earth Fixed Cell, considering the UEs starting HO at the time close to t-Service, one option is to introduce one time offset offsetHO in SIB1. SIBx can be broadcasted after the time [t-Service - offsetHO] instead of during the whole serving time. And the UE only monitors SIBx during the time [t-Service - offsetHO, t-Service]. Another solution is network could broadcast one indication via Paging to inform the UE to acquire the SIBX.

Step 2: The UE starts to monitor SIBx according to the scheduling information in SIB1 if the UE supports common (C)HO configuration.

Based on the above analysis, the UE could start to monitor SIBx at indicated time.

Step 3: The network sends (C)HO command without common (C)HO configuration to the UE supporting common (C)HO configuration.

The order of the step 2 and step 3 may change and this is up to network implementation.

Step 4: the UE starts HO using the target cell common configuration in SIBx.

It may happen that  the UE does not receive common configuration in SIBx. In order to solve this issue, the UE could indicate to the source cell or just listen to the SIB1 of the target cell to acquire servingCellConfigCommon of the target cell.

Figure 1: Illustration on how (C)HO configuration works

Based on the procedure shown in Figure 1, it is proposed that:

Proposal 10： The procedure in Figure 1 can be adopted as the baseline for common (C)HO configuration mechanism:

- Step 1: The UE acquires the scheduling information for common (C)HO configuration from SIB1 upon the specified time e.g. upon t-service-offsetHO, or upon reception of indication from network.

- Step 2: The UE starts to monitor SIBx according to the scheduling information in SIB1 if the UE supports common (C)HO configuration.

- Step 3: The network send (C)HO command without common (C)HO configuration.

- Step 4: the UE starts HO using the target cell common configuration in SIBx.

Notes: The order of step 2 and step 3 is up to network implementation.

Conclusion

In this document, we analyse issues for HO enhancement in NTN, and we find the observations and proposls as following:

RACH-less:

Proposal 1: For RACH-less HO, if the UE doesn’t receive the UE Contention Resolution Identity MAC CE until the T304 expiry, UE will initiate the RRC re-establishment procedure like LTE.

Proposal 2: For pre-allocated UL grant in RACH-less HO, define one retransmission timer.

Proposal 3: When the retransmission timer in RACH-less expires, the UE performs autonomous retransmission for RRCReconfigurationComplete on the pre-allocated UL grant.

Proposal 4 In RACH-less HO, the UE ignores the HARQ mode configured previously and assumes the HARQ process(es) is(are) HARQ mode A during RACH-less procedure for both pre-allocated UL grant and PDCCH scheduled UL grant.

Observation 1: PCI unchanged scenario is one specific case of inter-satellite handover with same gateway/gNB.

Observation 2: It simplifies the switch procedure and is beneficial to latency reduction to combine the RACH-less with the unchanged PCI scenario.

Proposal 5: RAN2 doesn’t discuss the combination of RACH-less with unchanged PCI scenario further in R18 due to the limited time.

Common (C)HO configuration delivery:

Proposal 6: RAN2 first focuses on intra-gNB scenario for common configuration.

Proposal 7: The serving cell broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling)

Proposal 8: Define one new SIB, i.e. SIBx, to transfer the common (C)HO configuration.

Proposal 9: The network broadcasts the common configuration of the adjacent target cell(s) based on the trajectory of the satellite, the number is FFS.

Proposal 10： The procedure in Figure 1 can be adopted as the baseline for common (C)HO configuration mechanism:

- Step 1: The UE acquires the scheduling information for common (C)HO configuration from SIB1 upon the specified time e.g. upon t-service-offsetHO, or upon reception of indication from network.

- Step 2: The UE starts to monitor SIBx according to the scheduling information in SIB1 if the UE supports common (C)HO configuration.

- Step 3: The network send (C)HO command without common (C)HO configuration.

- Step 4: the UE starts HO using the target cell common configuration in SIBx.

Notes: The order of step 2 and step 3 is up to network implementation.

Reference

- RAN2#122 meeting report;

- RAN2#121bis-e meeting report;

- R2-2306922, LS on common signalling in (C)HO (Oppo) LS out, Rel-18;