3GPP TSG-RAN WG2 #123	Tdoc R2-2307437

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.21.2

Source: 	Ericsson

Title:  	Further NR Coverage Enhancements CP Discussion

Document for:	Discussion and Decision

# Introduction

Previous RAN2 meetings for this work item have settled some important agreements related to the configuration of Multiple PRACH Transmissions.

For RACH Partitioning Framework, we propose some methods to further agree on configuration procedures. The possibility to support some CFRA cases and the possible impact on other working groups is discussed, as well as some suggestions on using individual threshold values to enhance the success rate when doing random access with Multiple PRACH Transmissions.

# Discussion

## 2.1 RACH Partitioning Framework

The random access procedure initialises a number of parameters at the start of the procedure as can be seen in 3GPP Reference 1:

3GPP Reference 1; TS 38.321 17.4.0 Chapter 5.1.1

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources: -- omitted -- - FeatureCombination: feature or a combination of features associated with a set of Random Access resources; - featurePriorities: priorities for features, such as RedCap, Slicing, etc. (see clause 5.1.1d); -- omitted --

Since the FeatureCombination parameter is initialised at the start of the procedure, it would be complex to define a procedure that considers other FeatureCombinations in the MAC procedure for random access.

It appears after RAN2#122 that the related agreement from previous meeting in this area could be interpreted in two ways:

RAN2#121bis-e: Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE)

Either that msg1 repetition is one feature, or that the repetition number are each one feature.

There is currently no concise agreement on how the RACH Partitioning Framework should be used for multiple PRACH Transmissions.

Fallback procedures would benefit from msg-1 repetitions being expressed without number of transmissions in FeatureCombination.

Based on these observations, we suggest that the agreement is rewritten to make it clear how msg1-repetitions should be implemented as part of the RACH Partitioning Framework. It is our understandings that most companies prefer a solution that only uses one of the spare values, as described in the following ASN.1 example:

#### FeatureCombination

The IE FeatureCombination indicates a feature or a combination of features to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles).

- FeatureCombination information element

-- ASN1START

-- TAG-FEATURECOMBINATION-START

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                               OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                               OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                   OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                               OPTIONAL,  -- Need R

spare4                     ENUMERATED {true}                               OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                               OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                               OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                               OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                               OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

-- TAG-FEATURECOMBINATION-STOP

-- ASN1STOP

Multiple PRACH transmissions are defined as one bit (supported or not supported) in FeatureCombination IE, i.e. regardless of number of PRACH transmissions (2, 4, or 8).

Given earlier RAN1 agreements, it is understood that gNB needs to be aware of which ROs are containing a specific number of multiple PRACH transmissions. For this reason, a preamble partition needs to be specified with a specific number of repetitions. It is our suggestion, as proposed by other companies in previous meetings, that this is done through e.g. the FeatureCombinationPreambles information element.

A preamble partition with a FeatureCombination that support msg1-repetitions is also configured with number of Multiple PRACH transmissions.

## 2.2 CFRA

In RAN2#122 the following agreement was made:

RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases.

Other cases that could be considered is Beam Failure Recovery (BFR), which is a valid use case since a BFR could easily happen when Multiple PRACH transmissions would be useful. Arguments have been presented that this would impact RAN1 specifications beyond what is reasonable since RAN1 have no TUs allocated after RAN2#123/RAN1#114.

The impact on RAN1 specs would in our view mainly come from that a UE and network has to determine number of PRACH transmissions based on radio conditions during BFR. We have earlier provided solutions to this, but would like to summarize them again, in order to illustrate that the RAN1 impact could be kept low.

By assigning a CFRA preamble to a specific set(s) of Multiple PRACH Transmissions (MuPT) [2, 4 or 8] the gNB can order the MuPT in advance, since it can be chosen by the gNB for CFRA in RRC_CONNECTED where the knowledge of the radio link is very good. The selection of a number of Multiple PRACH Transmissions could be done in different ways:

Option A) One number of repetitions applicable

Option B) A set of repetitions supported, which the UE can select between based on specified criteria.

At least for option A, the impact to RAN1 seems small and should not cause any controversy.

Different strategies for assigning CFRA preambles towards Multiple PRACH Transmissions could be adopted, with different complexity levels.

CFRA is supported for Multiple PRACH Transmissions for Beam Failure Recovery.

## 2.3 Thresholds

RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers

The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP

It is our view that a sequence type RSRP threshold configuration is both readable and effective:

rsrp-ThresholdMsg1-r18    SEQUENCE (SIZE (1..3)) OF RSRP-Range             OPTIONAL,  -- Cond Msg1Rep

Furthermore, if the RSRP thresholds are the only instruments to steer the number of Multiple PRACH transmissions (we have proposed additional methods in RAN1), it might be necessary to include some options to individually steer UEs depending on success rate of the access. Since all UEs in the case of RSRP thresholds will use a set of thresholds reported in the System Information, these will be the same for all UEs. Since the RSRP only measures the downlink signal strength, it would not reflect the actual quality in the UE radio characteristics. For this reason, two UEs with the same RSRP, might not have the same preconditions for random access success rate.

Two UEs with the same RSRP might not have the same conditions for success of a RACH attempt.

For this reason, we propose that the network can assign an individual RSRP threshold to a UE, by means of RRC signaling after a successful RACH attempt. The individual threshold could be used by the network to adapt to the specific radio characteristics of a single UE, and could be conditionally on that the UE has been static (TA value has not changed).

Individual RSRP thresholds could be configured for a UE.

## 2.4 UE Capabilities

For the UE Capabilities, it seems to be no reason for a UE to support a subset of the repetitions factors (2, 4, or 8). Rather, the UE should support msg1 repetitions completely, or not at all.

A UE that supports msg1 repetitions support all repetitions factors (2, 4, 8), indicated in UE Capabilities.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There is currently no concise agreement on how the RACH Partitioning Framework should be used for multiple PRACH Transmissions.

Observation 2	Fallback procedures would benefit from msg-1 repetitions being expressed without number of transmissions in FeatureCombination.

Observation 3	Different strategies for assigning CFRA preambles towards Multiple PRACH Transmissions could be adopted, with different complexity levels.

Observation 4	Two UEs with the same RSRP might not have the same conditions for success of a RACH attempt.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Multiple PRACH transmissions are defined as one bit (supported or not supported) in FeatureCombination IE, i.e. regardless of number of PRACH transmissions (2, 4, or 8).

Proposal 2	A preamble partition with a FeatureCombination that support msg1-repetitions is also configured with number of Multiple PRACH transmissions.

Proposal 3	CFRA is supported for Multiple PRACH Transmissions for Beam Failure Recovery.

Proposal 4	Individual RSRP thresholds could be configured for a UE.

Proposal 5	A UE that supports msg1 repetitions support all repetitions factors (2, 4, 8), indicated in UE Capabilities.

# References

3GPP 38.331, 17.4.0, NR; Radio Resource Control (RRC); Protocol specification

- [3]	3GPP 38.212, 17.4.0, NR; Multiplexing and channel coding

- [4]	3GPP 38.213, NR; Physical layer procedures for control