3GPP TSG RAN WG2 #123												  R2-2307442

Toulouse, France, August 21-25, 2023

Agenda Item:	7.8.3

Source: 	NEC

Title:	Discussion on Flight Path Reporting

Document for:	Discussion

- Introduction

In RAN2 122 the below agreements were made [1]:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This contribution provides our considerations on flight path information and reporting.

- Discussion

2.1Triggering conditions of flightpath update notification

In previous meeting, companies agreed to introduce delta time or distance conditions to trigger flightpath update notifications and several options were proposed. We see the main reason to have triggers is to make flightpath update notifications be controlled by the network, hence periodically triggering seems not so flexible and may lead to signaling overhead. To ensure that every triggering of notification indicates meaningful change, both how long (for time-based) and how far (for distance-based) the UE deviates and how many way point are deviate make sense. From this perspective, we think the triggering conditions can be (M and N are also configurable):

- More than M waypoints’ timestamps have changed from the reported FPP, and the change is more than a configurable time threshold

- More than N waypoints’ locations have changed from the reported FPP, and the change is more than a configurable distance threshold

Consider following conditions to trigger flightpath update notification:

- More than M waypoints’ timestamps have changed from the reported FPP, and the change is more than a configurable time threshold

- More than N waypoints’ locations have changed from the reported FPP, and the change is more than a configurable distance threshold

2.2 Fight path plan reporting in RRC_INACTIVE

Fight path reporting mechanism introduced in LTE Rel-15 contains two steps:

- Step 1: Inform the network “flight path info is available”

- Step 2: Include flightPathInfoReport and set it to include the list of waypoints along the flight path upon reception of UEInformationRequest message (with flightPathInfoReq field present);

In previous meeting, RAN2 agreed to allow the flight path to be updated. Our understanding is that the flight path update mechanism itself is out of RAN2 scope, therefore RAN2 discussion should focus on how to report the flight path update to the network.

In previous meeting, RAN2 also agreed that UAV UEs need to inform the network “flight path info is available” firstly. The availability indication can be included in RRCConnectionSetupComplete message, RRCConnectionResumeComplete message, RRCConnectionReconfigurationComplete message or RRCConnectionReestablishmentComplete message，upon reception of RRCConnectionSetup,  RRCConnectionResume or RRCConnectionReconfiguration including mobilityControlInfo or RRCConnectionReestablishment by the UE. In other words, only RRC_CONNECTED UAV UEs can report flight path plan.

Only RRC_CONNECTED UAV UEs can report flight path.

For the NR, when flight path is updated, the above legacy LTE procedure can be reused by informing the network “flight path info is updated and available”. However, when the UAV UEs is in RRC_INACTIVE and merely have flight path update to report, they still have to enter RRC_CONNECTED state firstly and send the report according to specified procedures. In such a case, RRC connection setup and subsequently release to RRC_IDLE/INACTIVE state happens every time reporting flight path update.

In previous meeting, RAN2 agreed to adopt location list of waypoints (3D location information) and timestamp as the basic content of flight path report. As summarized in TS22.125, typical message size of waypoint report ranges from 84-140 bytes.

When small size flight path plan is frequently reported, LTE-like mechanism may cause unnecessary power consumption and signaling overhead. In general, we observe that enabling flight path plan report in RRC_INACTIVE with Rel-17 SDT procedure, which is a procedure allowing data and/or signaling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state), is beneficial from network performance/efficiency and UE battery performance perspectives.

RAN2 to consider enabling flight path update report in RRC_INACTIVE.

- Conclusion

Observation 1:	Only RRC_CONNECTED UAV UEs can report flight path.

Proposal 1:	Consider following conditions to trigger flightpath update notification:

-	More than M waypoints’ timestamps have changed from the reported FPP, and the change is more than a configurable time threshold

-	More than N waypoints’ locations have changed from the reported FPP, and the change is more than a configurable distance threshold

Proposal 2:	RAN2 to consider enabling flight path update report in RRC_INACTIVE.

- References

[1] R2-23xxxxx, Report of 3GPP TSG RAN WG2 meeting #122, ETSI MCC.