3GPP TSG-RAN WG2 #123				Tdoc R2-2307465

Toulouse, France, August 21-25, 2023

Agenda Item:	7.20.2

Source:	Ericsson

Title:	On 2TA operation

Document for:	Discussion

# 1	Multi-DCI Multi-TRP with two TAs

RAN2 has received the L1 parameters in R1-2306271. The parameters related to “mTRP-2TA” are as follows:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| ServingCellConfig | tag-Id2 | New | This parameter provides the second TAG ID configured for the servingcell. | (0..maxNrofTAGs-1) |  | Exact design is up to RAN2 |
| TCI-State | tag-Id-ptr | New | This parameter is used to associate a TAG ID with a joint TCI state. This covers the case when joint TCI state is used for UL transmission. | 0, 1 | Note that this parameter either points to either tag-ID or tag-ID2 configured per serving cell depending on the value it indicates. This is only a suggestion/example and the final RRC signaling detail is up to RAN2 to decide. | pointer to original or second TAG ID. |
| TCI-UL-State | tag-Id-ptr | New | "This parameter is used to associate a TAG ID with an UL TCI state. This | 0, 1 | Note that this parameter either points to either tag-ID or tag-ID2 configured per serving cell depending on the value it indicates. This is only a suggestion/example and the final RRC signaling detail is up to RAN2 to decide. |  |
| ServingCellConfig | n-TimingAdvanceOffset2-r18 | New | covers the case when UL TCI state is used for UL transmission." | { n0, n25600, n39936 } | Note that the first n-TimingAdvanceOffset is the legacy parameter given in ServingCellConfigCommon/ServingCellConfigCommonSIB | does it work to have seocnd one? Especially broadcasted? |

The existing TS 38.331 has the following parameters and definitions:

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

..

}

tag-Id

Timing Advance Group ID, as specified in TS 38.321 [3], which this cell belongs to.

The IE TAG-Config is used to configure parameters for a time-alignment group.

TAG-Config information element

-- ASN1START

-- TAG-TAG-CONFIG-START

TAG-Config ::=                      SEQUENCE {

tag-ToReleaseList                   SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id                          OPTIONAL,   -- Need N

tag-ToAddModList                    SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG                             OPTIONAL    -- Need N

}

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                  TimeAlignmentTimer,

...

}

TAG-Id ::=                          INTEGER (0..maxNrofTAGs-1)

-- TAG-TAG-CONFIG-STOP

-- ASN1STOP

TAG field descriptions

tag-Id Indicates the TAG of the SpCell or an SCell, see TS 38.321 [3]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG).

timeAlignmentTimer The timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

…

}

n-TimingAdvanceOffset

The N_TA-Offset to be applied for all uplink transmissions on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

In TS 38.300, the following is specified for current TAG operation:

### 9.2.9	Timing Advance

In RRC_CONNECTED, the gNB is responsible for maintaining the timing advance to keep the L1 synchronised. Serving cells having UL to which the same timing advance applies and using the same timing reference cell are grouped in a TAG. Each TAG contains at least one serving cell with configured uplink, and the mapping of each serving cell to a TAG is configured by RRC.

For the primary TAG the UE uses the PCell as timing reference. In a secondary TAG, the UE may use any of the activated SCells of this TAG as a timing reference cell, but should not change it unless necessary.

Timing advance updates are signalled by the gNB to the UE via MAC CE commands. Such commands restart a TAG-specific timer which indicates whether the L1 can be synchronised or not: when the timer is running, the L1 is considered synchronised, otherwise, the L1 is considered non-synchronised (in which case uplink transmission can only take place on PRACH).

RAN2 has discussed for couple of meetings if the legacy definitions are still valid when the second TAG id and related TA timer is added to the serving cell and TCI states grouped to follow one of these. A TRP is essentially a spatial direction hence it is essentially a TCI state or group of those. From RAN121bis LS was sent to RAN1in R2-2304342 to clarify certain assumption. RAN2 received response in R1-2306249 with the following content:

1. Overall Description:

RAN1 thanks RAN2 for the LS on 2TA for multi-DCI multi-TRP and would like to provide the following response.

Question 1 on TAG grouping

RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs.

Q1a:   For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs?

Answer:  Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2.

Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group?

Answer:  RAN1 has not reached consensus to increase the current number of TAGs per cell group.

Question 2 on operation

Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

Answer: RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide.

Which UL or DL operation is impacted have not been discussed in RAN1.

The answer to the question 2 confirms that the operation of the new TAG is different from the legacy TAG which is defined for cell group and has e.g. the timer associated to all UL channels of all serving cells associated to the TAG.

Current procedures in TS 38.321 for PTAG are as follows:

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

Current procedures in TS 38.321 for STAG are as follows:

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

Given the confirmation from Ran1 that “when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide.” Note that RAN1 also does not mandate a definition for the new added TAG.

The MAC procedures are discussed in Report of [Post122][852][MIMOevo] 2TAs for multi-DCI multi-TRP. However, RAN2 needs to discuss the definitions of PTAG and STAG when second TAG is added to the serving cell. Also, related to the definition, RAN2 needs to discuss if a UE can be configured with one PTAG or two PTAGs.

If one PTAG is allowed then if the PTAG expires, all serving cells of the UE (and all TRPs) follow the legacy procedures of TS 38.321 as listed above. This allows to follow the legacy definition of PTAG but it contradicts the RAN1 response that only UL associated to expired TAG are impacted and it limits the functionality of the two TA.

- Assuming one PTAG  allows to follow the legacy definition of PTAG but it contradicts the RAN1 response that only UL associated to expired TAG are impacted and it limits the functionality of the two TA.

One option is to update the definition of legacy PTAG such that if two TAGs are configured for a PsCell, both of these are PTAGs but only if both expire the legacy PTAG procedures are followed.

RAN2 to discuss if two PTAGs per serving cell are supported.

If RAN2 concludes that two PTAGs per serving cell can be configured, RAN2 needs to discuss the relation and definition of these PTAGs. The definition of the PTAG in this case needs to be updated such one PTAG does not apply to all UL channels of all serving cells associated to that PTAG.

If RAN2 concludes that two PTAGs per serving cell can be configured, the definition of the PTAG in this case needs to be updated such one PTAG does not apply to all UL channels of all serving cells associated to that PTAG.

If RAN2 concludes that there is only one PTAG per serving cell, the functionality of the second TAG of that serving cell needs to be discussed. The definition of the second TAG to PTAG should be associated only to those UL/DL operations which are linked to that TAG id.

If RAN2 concludes that only one PTAGs per serving cell can be configured, the definition of the second TAG to the PTAG should be associated only to those UL/DL operations which are linked to that TAG id.

For STAG, there seems to be consensus that coincided with RAN1 design that if STAG expires then all TRPs of all sCells belonging to that TAG follow certain MAC level procedures. But the TRPs of these sCells belonging to the other TAG should not be impacted. However, this requires that the definition of STAG is revised for the case when serving cell is configured with two TAGs.

RAN2 to update the definition of STAG such that it is associated only to those UL/DL operations which are linked to that TAG id.

RAN2 also asked whether there are configuration restrictions on how the two TAGs can be configured to different serving TCI states of different serving cells. RAN1 did not indicate restriction but indicated it may do so later. Meanwhile, RAN2 can continue discussing restrictions from RAN2 perspective. One question is what happens in the following case if no restrictions to configure are defined:

Cell A has TAG1 and TAG2

Cell B has TAG1 and TAG3

TAG1 and TAG2 expire. It is clear all TRPs of cellA should have the legacy procedures. What happens to cell B? Only procedures related to TRPs under TAG1 will expire? This is aligned with current understanding of the intention of having two TAGs. This may be different if the TAG1 is PTAG or STAG and if RAN2 concludes there may be one or two PTAGs associated to a serving cell. Our view is that specification support in TS 38.321 becomes simpler if it is not allowed to configure different second TAG to one first TAG.

RAN2 discuss if it is allowed to configure different second TAG to one first TAG.

RAN1 further indicated that a second TA timer offset n-TimingAdvanceOffset2-r18 would be configured. However, instead of common configuration, it would be placed in dedicated configuration. The excel also has the following questions: “does it work to have second one? Especially broadcasted?” It is also unclear to us how this is intended to be supported in TS 38.331. Definition of the existing offset is “The N_TA-Offset to be applied for all uplink transmissions on this serving cell.” It is clear that if second offset is added and intended to work with the legacy offset, the definition of the legacy offset needs to be changed. This may be tricky from backwards compatible perspective.

RAN2 discuss how the second TA timer offset is defined and whether the definition of the existing offset is changed or updated.

# 2	PRACH configuration “per additional PCI”

RAN1 had the following agreement:

Agreement

Confirm the following working assumption:

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Note that RAN1 supports only CFRA for 2TA operation and it is up to RAN2 to decide on CBRA. Hence this discussion is related to CFRA and it is assumed RAN2 can decide later whether CBRA needs to be supported or not.

In TS 38.331, PRACH configuration is given to the UE in IE The CellGroupConfig, which is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

Within that IE the PRACH configuration is given with IE ReconfigurationWithSync:

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                             OPTIONAL -- Cond Occasions

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGenericTwoStepRA-r16,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Resource-r16,

msgA-TransMax-r16                       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}    OPTIONAL, -- Need S

resourcesTwoStep-r16                    SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...,

[[

msgA-PUSCH-Resource-Index-r16   INTEGER (0..3071)     OPTIONAL  -- Cond 2StepCFRA

]]

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

There is also related RRC procedure specified in Clause 5.3.5.8.3 which states that at T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) UE release preambles provided in rach-ConfigDedicated if configured and so on. The added RACH configuration could be something similar to IE BeamFailureRecoveryConfig, however, RAN2 should wait for RAN1 input on the configuration and operation.

RAN2 to wait for RAN1 input on the configuration and operation.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss if two PTAGs per serving cell are supported.

Proposal 2	If RAN2 concludes that two PTAGs per serving cell can be configured, the definition of the PTAG in this case needs to be updated such one PTAG does not apply to all UL channels of all serving cells associated to that PTAG.

Proposal 3	If RAN2 concludes that only one PTAGs per serving cell can be configured, the definition of the second TAG to the PTAG should be associated only to those UL/DL operations which are linked to that TAG id.

Proposal 4	RAN2 to update the definition of STAG such that it is associated only to those UL/DL operations which are linked to that TAG id.

Proposal 5	RAN2 discuss if it is allowed to configure different second TAG to one first TAG.

Proposal 6	RAN2 discuss how the second TA timer offset is defined and whether the definition of the existing offset is changed or updated.

Proposal 7	RAN2 to wait for RAN1 input on the configuration and operation.