3GPP TSG-RAN WG2 Meeting #123	R2-2307495

Toulouse, France 21st – 25th August, 2023

Agenda Item:	7.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on shared processing for MBS broadcast and unicast reception

Document for:   Discussion and decision

Introduction

In the RAN2#122 meeting, we made the agreements for shared processing as below [1]:

- The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs. - No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed). - When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. - FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII) - No additional information is added to MII on top of what has been already agreed.

In this contribution, we further discuss the FFS issues from the last meeting.

Discussion

MII reporting procedure

In the last meeting, it was agreed that the whole information (i.e. frequency, bandwidth, SCS) is included in the MII for non-serving cell if allowed by SIB1 of its serving cell. However, it is still FFS whether there are cases where the whole information is not available at the UE.

In our understanding, as the processing resources of the UE are shared between the unicast/multicast transmission from serving cells and the broadcast reception from non-serving cell, it is possible that the UE is unable to read the SIB1 of the non-serving cell during unicast/multicast transmission. For example, when the configured CA/DC transmission for unicast occupies all the processing resources of the UE, the UE cannot read the SIB1 from the non-serving cell. In this case, only the frequency of the non-serving cell is available based on SIB21 from the serving cell or based on USD, but the additional information (e.g. SCS, bandwidth) of the non-serving cell is not available.

Observation 1: It is possible that the UE cannot read the SIB1 from the non-serving cell so that the SCS/bandwidth from the non-serving cell is not available at the UE.

To avoid interruption of the on-going services, the additional information cannot be reported by the UE without proper configuration from the serving gNB. In legacy NR, to report the CGI for inter-frequency or inter-RAT, the UE can be configured by the network to use the autonomous gap. Similar mechanism (e.g. the existing autonomous gap) can be reused to acquire the information from the non-serving cell in case they cannot be acquired while performing the communication with the serving cell(s).

To be specific, the whole information (i.e., frequency, SCS, bandwidth) can be reported in the MII if available. But if any information (e.g., SCS, bandwidth) is not available, the UE has no choice but only reports the interested broadcast frequency in the MII. In order for the UE to report whole information to the NW, after receiving the MII, the NW can choose to configure an autonomous gap for the UE, to enable the UE to acquire the SCS/bandwidth information from the non-serving cell. The whole information can be reported once available.

Proposal 1：When SCS/bandwidth of the non-serving cell is not available (i.e. the UE is not able to acquire it from the non-serving cell), only frequency information is reported in the MII. The whole information is reported once available.

In the last meeting, it is also FFS whether special handling for MII reporting is needed when the non-serving cell updates the configuration. According to the current spec in Rel-17, the MII will be initiated when the content of MII changes as follows.

1> if SIB21 is provided by the PCell: 2> ensure having a valid version of SIB21 for the PCell; 2> if the UE did not transmit MBS Interest Indication since last entering RRC_CONNECTED state; or 2> if since the last time the UE transmitted an MBS Interest Indication, the UE connected to a PCell not providing SIB21: 3> if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is not empty: 4> set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message; 2> else: 3> if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is different from mbs-FreqList included in the last transmission of the MBS Interest Indication; or 3> if the prioritisation of reception of all indicated MBS broadcast frequencies compared to reception of any of the established unicast bearers and multicast MRBs has changed since the last transmission of the MBS Interest Indication: 4> set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

In our view, the initiation of the MII should follow the similar mechanism as Rel-17, i.e. the UE initiates transmission of the MII message when the frequency, SCS or bandwidth for non-serving cell broadcast reception has changed. In the last meeting, some companies have concern on the signalling overhead, and it was proposed to avoid repeated MII reporting on the condition that multiple UEs are receiving broadcast from the same non-serving cell. However, it is hard for the NW to manage the UEs receiving broadcast from the same non-servings, and dynamically decide which UE to report or not to report the MII considering UE’s mobility. Besides, different non-serving cells on the same frequency may not share the same configuration. What’s more, the change of SCS or bandwidth of the non-serving cell does not happen frequently. Considering the above reasons, it is better not to make the procedure further complex.

Proposal 2: No special handling is needed on MII reporting when the non-serving cell updates the configuration.

MII content

In our understanding, the MBS broadcast configuration in the non-serving cell should be aligned with the MBS broadcast configuration used for serving cell in Rel-17. Therefore, if SCS is included in the MII for shared processing, it should be the SCS of the initial BWP (i.e. CORESET#0) of the non-serving cell. Besides, for the bandwidth included in the MII, it was proposed to be either the cell bandwidth or the CFR bandwidth of the non-serving cell. However, it should be noted that when the UE camps on a cell, the UE only needs to support a bandwidth which is larger than initial BWP and smaller than or equal to the cell bandwidth, according to the current RRC spec:

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which - is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which - is wider than or equal to the bandwidth of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured, and …… 4> apply a supported downlink channel bandwidth with a maximum transmission bandwidth which - is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which - is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a RedCap UE, of the RedCap-specific initial downlink BWP if configured; 4> select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present; 4> forward the cellIdentity to upper layers; ……

Based on the 38.331 text, it is clear that the channel bandwidth the UE actually uses may NOT be the bandwidth of initial BWP or CFR. In MBS scenario, the serving gNB should decide the available unicast capability based on the bandwidth the UE actually uses for broadcast reception in non-serving cell. For example, the CFR configured by the non-serving cell for broadcast reception is 30MHz, but the UE may actually use 40MHz channel bandwidth to receive broadcast if 30MHz is not supported by the UE.

Observation 2: When the UE camps on a cell, the actual channel bandwidth used by the UE may NOT be the bandwidth of initial BWP or CFR.

We understand the channel bandwidth reported in the MII should be aligned with the signalled bandwidth capability in UE capability information. This bandwidth can give the serving gNB more precise information about how much processing capability can be left to unicast/multicast scheduling, with less implementation complexity. In our view, when the UE performs broadcast reception from the non-serving cell, the indicated bandwidth in the MII should be the channel bandwidth actually used by the UE, which covers the broadcast CFR bandwidth but can be smaller than the carrier bandwidth of the non-serving cell.

Proposal 3: The SCS indicated in the MII is the SCS of the initial BWP of the non-serving cell where the UE intends to receive MBS broadcast.

Proposal 4: The bandwidth indicated in the MII is the channel bandwidth used by the UE for broadcast reception in the non-serving cell, which covers the CFR bandwidth.

Conclusion

In this contribution, we provide the following proposals:

MII reporting procedure

Observation 1: It is possible that the UE cannot read the SIB1 from the non-serving cell so that the SCS/bandwidth from the non-serving cell is not available at the UE.

Proposal 1：When SCS/bandwidth of the non-serving cell is not available (i.e. the UE is not able to acquire it from the non-serving cell), only frequency information is reported in the MII. The whole information is reported once available.

Proposal 2: No special handling is needed on MII reporting when the non-serving cell updates the configuration.

MII content

Observation 2: When the UE camps on a cell, the actual channel bandwidth used by the UE may NOT be the bandwidth of initial BWP or CFR.

Proposal 3: The SCS indicated in the MII is the SCS of the initial BWP of the non-serving cell where the UE intends to receive MBS broadcast.

Proposal 4: The bandwidth indicated in the MII is the channel bandwidth used by the UE for broadcast reception in the non-serving cell, which covers the CFR bandwidth.

References

[1] RAN2#122 chairman’s note.

[2] R2-2304821, Discussion on shared processing for MBS broadcast and unicast reception, Huawei, HiSilicon.