3GPP TSG-RAN2#123										     R2-2307537

Toulouse, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Extended periodicity for CG-SDT

Agenda item:	7.24.2

Document for: Discussion and Decision

# Introduction

At RAN2#122, RAN2 agreed to specify CG-SDT periodicities higher than the values supported in Rel-18. RAN2 made this agreement under the assumption that this extension would have no or low impact on RAN1 specifications. The actual values of the CG-SDT periodicities have been left as FFS for now. In this contribution we take a look at the impacts to RAN2 specifications to configure these higher periodicities for CG-SDT.

# Discussion

Whilst the decision made in RAN2 was conditional upon RAN1 assessing this to be of low impact, it is worth noting that when the agreement on the CG period was made in RAN1, RAN1 already anticipated that RAN2 may at some point specify larger periodicities. The corresponding RAN1 agreement is copied below for reference.

Agreement for SDT in RAN1#108-e Agreement - For CG-SDT, the starting time of association period is SFN0. - Regarding the candidate value set of association period, adopt the Table 2.4-1 - FFS CG period smaller than 5ms - Note: It does not mean to RAN2 the maximum CG period is 640ms. The table will be updated if RAN2 introduces other CG period values. - Note: The potential impact on PDCCH monitoring periodicity should be considered if larger CG period value is introduced in RAN2 Table 2.4-1: Mapping between CG period and SS/PBCH block to CG PUSCH resource association period CG period (msec); Association period (number of CG periods except when CG period is less than 5 ms). 5; {1, 2, 4, 8,16, 32, 64, 128}. 8; {1, 2, 4, 5, 8, 10, 16, 20, 40, 80}. 10; {1, 2, 4, 8,16, 32, 64}. 16; {1, 2, 4, 5, 8,10,20,40}. 20; {1, 2, 4, 8,16, 32}. 32; {1, 2, 4, 5, 10, 20}. 40; {1, 2, 4, 8, 16}. 64; {1, 2, 5, 10}. 80; {1, 2, 4, 8}. 128; {1, 5}. 160; {1, 2, 4}. 320; {1, 2}. 640; {1 }.

Observation 1: RAN1 already anticipated that RAN2 may specify larger periodicities for CG-SDT and left the decision on this to RAN2.

RAN1 agreement also notes potential impact to PDCCH monitoring (specifically to avoid any adverse impact on the UE power consumption) with the larger periodicity for CG-SDT. However, RAN2 agreed to start the T319a only after the first transmission happens on the CG-SDT resource and hence the actual SDT session starts only after the first transmission happens and only lasts at most until T319a expires. Thus, extension of CG-SDT period should have no impact to UE power consumption. It should be noted that RAN2 has no intention to extend T319a as part of this feature.

Observation 2: Extension of CG-SDT periodicity will have no impact to UE power consumption since the UE only monitors the PDCCH after the first message is transmitted over the CG-SDT resources until T319a expiry

Thus, we anticipate no bottlenecks for this feature to be specified under TEI18.

Looking at RAN2 impacts, RAN2 needs to agree on the actual values for extended periodicities. Whilst the extended periodicities would need to be reflected in the mapping table between CG period and SSB to CG PUSCH resource association period in RAN1 specs, RAN2 specs should be updated to support the new extended periodicities in the signalling. Currently only up to 640 ms is supported for CG-SDT.  For the extended periodicities, it would be good to cover up to 10s of sec or minutes. For example, the default keep alive packet timeout interval for TCP is 1800 sec. The default idle time out for TCP traffic is 300 sec. The default keep alive timer for MQTT used for IoT applications is 60 sec. It is proposed to specify CG-SDT periods covering these ranges and along with a few code points around these. So, the following values are proposed:

Proposal 1: For the extended CG-SDT period, specify the following values {10, 60, 120, 300, 600, 1200, 1800, 3600} secs

In addition to this, RAN2 also agreed that for the initial transmission if the CG-SDT occasion is farther than a configured threshold, then the UE is allowed to fallback to RA-SDT. Thus, the specification also needs to consider the actual values for this threshold. In the MT-SDT running CR, the value of this threshold is currently left as FFS since the final values depend on the actual CG-period agreed. We propose to finalise these values to cover the above ranges.

Observation 3: The values of the parameter cg-SDT-MaxDurationToNext-CG-Occasion-r18 depends on the final values for the extended CG-SDT period.

Proposal 2: Adopt the following values for the parameter cg-SDT-MaxDurationToNext-CG-Occasion-r18 {100, 1000, 30000, 100000, 200000, 300000, 1500000, 3000000} ms

# Conclusion

Observation 1: RAN1 already anticipated that RAN2 may specify larger periodicities for CG-SDT and left the decision on this to RAN2..

Observation 2: Extension of CG-SDT periodicity will have no impact to UE power consumption since the UE only monitors the PDCCH after the first message is transmitted over the CG-SDT resources until T319a expiry

Observation 3: The values of the parameter cg-SDT-MaxDurationToNext-CG-Occasion-r18 depends on the final values for the extended CG-SDT period.

Proposal 1: For the extended CG-SDT period, specify the following values {10, 60, 120, 300, 600, 1200, 1800, 3600} secs

Proposal 2: Adopt the following values for the parameter cg-SDT-MaxDurationToNext-CG-Occasion-r18 {100, 1000, 30000, 100000, 200000, 300000, 1500000, 3000000} ms

# References

[1] R2-2306904, LS on longer CG-SDT periodicities, RAN2#122