3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307555

Toulouse, France, 21st – 25th Aug, 2023

Source:              CATT

Title:	             Further Discussion on SL LBT

Agenda Item:	7.15.2

Document for:	Discussion and Decision

Introduction

In past RAN2 meetings, some agreements were reached on SL LBT. But there are still some leftover issues, listed below:

- Issue 1: Whether OOC UE is considered in the SL-U bullet?

- Issue 2: When detecting the SL C-LBT failure, how to consider the PSFCH impact?

- Issue 3:  How to perform the SL C-LBT failure recovery for mode 2 RRC connected UE and OOC UE?

- Issue 4: What are the conditions for cancelling the triggered SL C-LBT failure?

- Issue 5: How to design the SL C-LBT failure MAC CE?

- Issue 6: What are the impacts of introducing SL C-LBT failure MAC CE?

- Issue 7: Whether the working assumption on enhanced LCP can be agreed?

Discussion

SL-U scope clarification

In the RAN2#122 meeting, when discussing the SL C-LBT failure detection and SL C-LBT failure cancellation, only three kinds of UE(mode 1 UE, mode 2 RRC idle/inactive UE and mode 2 RRC connected UE) are mentioned. But OOC UE is not considered.

According to the WID [1], the SL-U corresponding bullet is described as below:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

Based on the above  description, it only restrict that Uu operation for mode 1 is limited to licensed spectrum only, but it does not exclude OOC UE. Since OOC UE has not been discussed till now, it is proposed to discuss whether OOC UE is considered or not in the SL-U bullet.

Proposal 1: Suggest RAN2 to discuss whether OOC UE should also be considered in the SL-U bullet.

PSFCH impact on SL C-LBT failure detection

Regarding to the PSFCH impact on SL C-LBT failure detection, the following agreement was reached in RAN2#122 meeting:

Agreements on SL C-LBT failure and PSFCH

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

There is one FFS left. In our understanding, the multiple PSFCH occasions can be handle same as the single PSFCH occasion when performing LBT failure indication counting because LBT failure only corresponding to each transmission resource.

Proposal 2: Revise the agreement reached on the last RAN2 meeting as below: “Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured. ”

SL C-LBT failure recovery for mode 2 RRC connected UE and OOC UE

Mode 2 RRC connected UE

In RAN2#122 meeting, regarding to the SL C-LBT failure recovery for mode 2 connected UE, only one agreement was reached as below:

Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE)

1: 	RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE.

Based on the agreement, it only stated that UE should report the SL C-LBT failure indication to gNB, but it still unclear how the SL C-LBT failure is recovered.

- Option 1: Follow mode-1 UE behavior.

- Option 2: Follow mode-2 RRC idle/inactive UE behavior.

- Option 3: Combination of Option 1 and Option 2.

For Option 1, if mode 2 is used, the SL resource is selected by the UE itself, it is unclear what gNB can do to help the SL C-LBT failure recovery.

For Option 2, UE will first perform SL C-LBT failure recovery like mode 2 idle/inactive UE. If the SL C-LBT failure cannot be recovered, it can report the SL RLF to gNB via SUI to indicate the SL C-LBT failure.

For Option 3, if it is supported, it is unclear when to report the SL C-LBT failure indication and when to perform UE autonomous SL C-LBT failure recovery. This will need more discussions and specification efforts.

Compared with the above three options, from our perspective, Option 2 is more attractive.

Proposal 3: For mode-2 RRC connected UE, once SL C-LBT failure occurs, it can perform SL C-LBT failure recovery like mode-2 RRC idle/inactive UE.

Proposal 4: For mode-2 RRC connected UE, if the SL C-LBT failure cannot be recovered, UE can report SL RLF to gNB to indicate the SL C-LBT failure.

OOC UE

As discussed in Proposal 1, if OOC UE is also considered in SL-U bullet, the SL C-LBT failure recovery procedure for OOC can be same as the mode-2 RRC idle/inactive UE.

Proposal 5: For OOC UE, once SL C-LBT failure occurs, the SL C-LBT failure recovery procedure is same as mode 2 RRC idle/inactive UE.

Conditions for cancelling the triggered SL C-LBT failure

In RAN2#122 meeting, the conditions for cancelling the triggered SL C-LBT failure were initially discussed for mode 1 UE, mode 2 UE RRC idle/inactive UE and mode 2 RRC connected UE. And only one agreement was reached for mode 1 UE.

Agreements on SL C-LBT cancellation

1: 	For mode 1, SL C-LBT is cancelled upon SL C-LBT failure MAC CE transmission

Besides the agreed condition, there are also other conditions can be considered. In the following, we give further analysis.

In NR-U, the triggered consistent LBT failure can be cancelled in the following cases listed below:

- 5.21.2 LBT failure detection and recovery procedure …… 1> if a MAC PDU is transmitted and LBT failure indication is not received from lower layers and this PDU includes the LBT failure MAC CE: 2> cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE. 1> if consistent LBT failure is triggered and not cancelled in the SpCell; and 1> if the Random Access procedure is considered successfully completed (see clause 5.1) in the SpCell: 2> cancel all the triggered consistent LBT failure(s) in the SpCell. 1> if lbt-FailureRecoveryConfig is reconfigured by upper layers for a Serving Cell: 2> cancel all the triggered consistent LBT failure(s) in this Serving Cell.

## 5.12 MAC Reset If a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: …… 1> cancel, if any, triggered Power Headroom Reporting procedure; 1> cancel, if any, triggered consistent LBT failure;

Based on the above description, in NR-U, the conditions for cancelling the trigger C-LBT failure include:

- MAC reset;

- LBT failure recovery parameters are reconfigured;

- Random access in SpCell is successful;

- C-LBT failure MAC CE transmission.

In SL-U, considering UE may in different RRC state and using different resource allocation mode, the analysis will be performed based on RRC state and resource allocation mode.

Mode 1 (RRC connected)

For Mode 1 UE, besides the agreed SL C-LBT failure cancellation condition, similar as NR-U, the following conditions should also be considered:

- PC5 MAC  reset;

- SL-specific LBT failure recovery parameters are reconfigured.

Proposal 6: For mode 1 UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- When the SL-specific LBT failure detection related parameter(s) is reconfigured;

- SL MAC reset.

Mode 2 (RRC connected)

For Mode 2 RRC idle/inactive UE, similar as NR-U, the following conditions can be considered:

- PC5 MAC  reset;

- SL-specific LBT failure recovery parameters are reconfigured.

In addition, some more conditions can be considered, e.g., resource pool or RB set reselection or reconfiguration.

Proposal 7: For mode 2 RRC connected UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- When the SL-specific LBT failure detection related parameter(s) is reconfigured;

- SL MAC reset;

- Upon resource pool or RB set(s) reselection;

- Upon reconfiguration of the resource pool(s) which includes the SL RB set(s) with triggered but not cancelled SL C-LBT.

Mode 2 (RRC idle/inactive/OOC)

For Mode 2 RRC idle/inactive/OOC UE, similar as NR-U, the following condition can be considered:

- PC5 MAC reset.

In addition, some more conditions can be considered, e.g., resource pool or RB set reselection.

Proposal 8: For mode 2 RRC idle/inactive/OOC UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- SL MAC reset;

- Upon resource pool or RB set(s) reselection.

SL C-LBT failure MAC CE format

In RAN2, it was agreed that the granularity of LBT failure indication is RB set. Hence, it is obvious that RB set index(s) should be included in the SL C-LBT failure MAC CE. But how to represent the RB set index should be further discussed since it is unclear how RB set is indexed in RAN1, whether it is indexed per SL BWP or indexed per resource pool. Furthermore, it is also unclear whether different resource pool can share the same RB set(s). Hence, before discussing the detailed SL C-LBT failure MAC CE format, it had better send LS to check how the RB set is indexed in PHY.

Proposal 9: At least RB set index(s) should be included in the SL C-LBT failure MAC CE.

Proposal 10: Send LS to RAN1 to check how the RB set is indexed.

Impacts of introducing SL C-LBT failure MAC CE

In RAN2#121bis-e meeting, it was agreed to introduce SL-specific LBT failure MAC CE. Once SL C-LBT failure MAC CE is triggered and there is no UL grant, SR should be triggered. The SR resource configured for Sidelink CSI Reporting MAC CE can be reused for SL C-LBT failure MAC CE.

Proposal 11: When SL C-LBT failure MAC CE is triggered and there is no UL grant, SR can be triggered.

Proposal 12: The SR resource configured for Sidelink CSI Reporting MAC CE can be reused for SL C-LBT failure MAC CE.

If SL-specific LBT failure MAC CE is introduced, it should discuss the impact on LCP in Uu. Currently, the LCP priority in Uu is listed below:

- MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for Timing Advance Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding.

Regarding to the SL-specific LBT failure MAC CE, it should have similar priority as the LBT failure MAC CE of NR-U. Hence, its priority can be between MAC CE for LBT failure and the MAC CE for Timing Advance Report.

Proposal 13: If SL-specific LBT failure MAC CE is introduced, its priority should between the MAC CE for LBT failure and the MAC CE for Timing Advance Report.

Working assumption on enhanced LCP

In the RAN2#122 meeting, the following working assumption was agreed:

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

In RAN1#109 meeting, the following agreements were reached:

Agreement: - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions

Since it was RAN1 agreement, there is no motivation to not follow it. Hence, it had better confirm the working assumption as agreement.

Proposal 14: Confirm the following working assumption as agreement: “For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.”

In addition, besides the enhanced LCP, there are two other cases which are not relative to the LCP procedure:

- Case 1: PDU has been generated before the COT sharing information reception and does not satisfies the COT requirement;

- Case 2: PDU has been generated before the COT sharing information reception and satisfy the COT requirement;

In Case 1, if the PDU needs to be transmitted, only type 1 can be used; and in Case 2, type 2 LBT can be used.

Proposal 15: For the MAC PDU(s) which has been generated before the reception of the COT sharing information, if it satisfies the COT requirement, type 2 LBT can be used; otherwise, type 1 LBT can be used.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Suggest RAN2 to discuss whether OOC UE should also be considered in the SL-U bullet.

Proposal 2: Revise the agreement reached on the last RAN2 meeting as below: “Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured. ”

Proposal 3: For mode-2 RRC connected UE, once SL C-LBT failure occurs, it can firstly perform SL C-LBT failure recovery like mode-2 RRC idle/inactive UE. Only when the SL C-LBT failure cannot be recovered, it can report SL C-LBT failure indication to gNB.

Proposal 4: For mode-2 RRC connected UE, if the SL C-LBT failure cannot be recovered, UE can report SL RLF to gNB to indicate the SL C-LBT failure.

Proposal 5: For OOC UE, once SL C-LBT failure occurs, the SL C-LBT failure recovery procedure is same as mode 2 RRC idle/inactive UE.

Proposal 6: For mode 1 UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- When the SL-specific LBT failure detection related parameter(s) is reconfigured;

- SL MAC reset.

Proposal 7: For mode 2 RRC connected UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- When the SL-specific LBT failure detection related parameter(s) is reconfigured;

- SL MAC reset;

- Upon resource pool or RB set(s) reselection;

- Upon reconfiguration of the resource pool(s) which includes the SL RB set(s) with triggered but not cancelled SL C-LBT.

Proposal 8: For mode 2 RRC idle/inactive/OOC UE, all the triggered SL consistent LBT failure should be cancelled when any of the following condition is met:

- SL MAC reset;

- Upon resource pool or RB set(s) reselection.

Proposal 9: At least RB set index should be included in the SL C-LBT failure MAC CE.

Proposal 10: Send LS to RAN1 to check how the RB set is indexed.

Proposal 11: When SL C-LBT failure MAC CE is triggered and there is no UL grant, SR can be triggered.

Proposal 12: The SR resource configured for Sidelink CSI Reporting MAC CE can be reused for SL C-LBT failure MAC CE.

Proposal 13: If SL-specific LBT failure MAC CE is introduced, its priority should between the MAC CE for LBT failure and the MAC CE for Timing Advance Report.

Proposal 14: Confirm the following working assumption as agreement: “For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.”

Proposal 15: For the MAC PDU(s) which has been generated before the reception of the COT sharing information, if it satisfies the COT requirement, type 2 LBT can be used; otherwise, type 1 LBT can be used.

Reference

- RP-230077  WID revision: NR sidelink evolution

- TS38.331  Radio Resource Control (RRC) protocol specification (Release 17)