3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307556

Toulouse, France, 21st – 25th Aug, 2023

Source:              CATT

Title:	             Discussion on Sidelink Resource Reselection

Agenda Item:	7.15.3

Document for:	Discussion and Decision

Introduction

In RAN2#121 meeting, regarding to the resource reselection, it was agreed that:

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

In the RAN2#122 meeting, the FFS issue was initially discussed, but no conclusion was reached. In this contribution, this FFS issue will be further discussed.

Discussion

The purpose of introducing MCSt is to reduce the Type 1 LBT overhead and once UE acquired the COT, it can use the COT to transmit as much data as possible. In the last RAN2#122 meeting, the MCSt discussion was divided into single TB case and multiple TB case. Hence, in the following, we will discuss whether resource (re-)selection should be triggered for each case.

Single TB case

For the single TB case, if MCSt is used, the UE will use consecutive slots to transmit the initial transmission and retransmission(s) of the same TB. For this case, in the RAN2#122 meeting, some companies do not support resource reselection when PSSCH transmission was not performed due to an LBT failure indication from L1. The main reason is that if resource (re)selection is triggered, it will increase the type 1 LBT overhead.

But we have different understanding. In case of no LBT failure, if MCSt is used, only type 1 LBT is used for the initial transmission. And for the other retransmission(s), type 2 LBT can be used. But if the initial PSSCH transmission was not performed due to an LBT failure indication from L1, even if resource reselection is not performed, the next retransmission still needs to perform type 1 LBT. Hence, triggering resource (re-)selection will not increase the Type 1 LBT overhead. On the contrary, if resource reselection is supported, it can increase the LBT success probability and type 1 LBT can be avoided for the consecutive retransmission(s).

Multiple TB case

For the multiple TB cases, in the RAN2#122 meeting, three approaches were mentioned in RAN1 LS. After RAN2 discussion, the following agreements were reached:

Agreements on MCSt

1: 	For Qustion-1 from RAN1 (Q1 in R1-2304257), R2 replies that it is feasible to select the resource for a single TB in MAC layer and concatenate across separate resource selection triggers across TBs in a best-effort manner.

2:	For Qustion-2 from RAN1 (Q2 in R1-2304257), R2 replies that the approach 3 is not compatible with the current specification and it may bring big specification impacts.

3:	For Question-3 from RAN1 (Q3 in R1-2304257), R2 replies that it is feasible to provide a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt.

Based on the above agreements, it is obvious that approach 3 has big specification impacts, and not preferred by RAN2. For approach 1 and approach 2, the MCSt for multiple TB is best effort.  If approach 1 or approach 2 is adopted, the MCSt of multiple TBs are implemented based on concatenation of the MCSt resources of a single TB. Hence, there is no much difference between single TB and multiple TB. It had better adopt the same solution.

Proposal: For MCSt case, UE can trigger resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal: For MCSt case, UE can trigger resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1.