3GPP TSG-RAN WG2 Meeting #123	R2-2307580

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Resolving Open Points on RACH-less HO in Rel-18 NTN

WID/SID:	NR_NTN_enh -Core

Document for:		Discussion and Decision

# Introduction

In WID [1], one of the objectives for Rel-18 NR NTN is to continue enhancements for NTN mobility. The work will consider Rel-17 NR NTN as the baseline for further enhancements including:

- Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3].

As part of this objective, handover enhancements have been discussed in RAN2 during the past meetings. Among those, the RACH-less handover (HO) has been proposed as an effective solution to reduce the signalling overhead.

At RAN2#122 (May 2023) the following agreements on RACH-less HO were made:

Agreements: - In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0. Agreements: - From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO. - Release pre-allocated UL grant after RACH-less HO completion. - LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI. - 4. Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any).

This paper discusses remaining aspects on the RACH-less HO for NTN topic.

# Discussion

## UL synchronization to target cell

Regarding the UL synchronization to the target cell during the RACH-less HO procedure, the following was agreed in RAN2#122 meeting:

Agreement Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any).

According to TS 38.331, the UL synchronization depends on the validity of satellite assistance information in SIB19, which is governed by the timer T430. Upon timer expiration, the UE considers that UL synchronization is lost and is required to acquire SIB19 according to its implementation. This procedure should be the same whether the UE is configured with a RACH-based HO or a RACH-less HO.

Observation 1: According to TS 38.331, the UL synchronization depends on the validity of SIB19 and it does not change whether the UE is configured with a RACH-based HO or a RACH-less HO.

Another aspect is how the UE acquires NTA parameter from the NW, which is required for the UE to estimate the full TA and transmit in the UL. As pointed out in [4], the only difference for the RACH-less HO is the calculation of the TA (TTA) for the target cell is that NTA is configured in the RACH-less HO command.

In RAN2 we adopted LTE RACH-less HO as baseline for NTN. The acquisition of NTA in the RACH-less HO command is already specified in TS 36.331 with targetTA field. The UE can be configured with an NTA value equal 0 or with the same NTA as in the source cell.

RACH-Skip-r14 ::= SEQUENCE { targetTA-r14 CHOICE { ta0-r14 NULL, mcg-PTAG-r14 NULL, scg-PTAG-r14 NULL, mcg-STAG-r14 STAG-Id-r11, scg-STAG-r14 STAG-Id-r11 }, ul-ConfigInfo-r14 SEQUENCE { numberOfConfUL-Processes-r14 INTEGER (1..8), ul-SchedInterval-r14 ENUMERATED {sf2, sf5, sf10}, ul-StartSubframe-r14 INTEGER (0..9), ul-Grant-r14 BIT STRING (SIZE (16)) } OPTIONAL -- Need OR }

targetTA This field refers to the timing adjustment indication, see TS 36.213 [23], indicating the NTA value which the UE shall use for the target PTAG of handover or the target PSTAG of SCG change. ta0 corresponds to NTA=0. mcg-PTAG corresponds to the latest NTA value of the PTAG associated with MCG. scg-PTAG corresponds to the latest NTA value of the PTAG associated with SCG. mcg-STAG corresponds to the latest NTA value of a MCG STAG indicated by the STAG-Id. scg-STAG corresponds to the latest NTA value of a SCG STAG indicated by the STAG-Id.

Proposal 1: There are no further specification impact to discuss regarding the UL synchronization and the NTA acquisition in RACH-less HO.

## Confirmation of RACH-less HO completion

During RAN2#122 meeting, it was discussed how successful RACH-less HO completion should be indicated. Later, the following agreement was taken:

Agreement LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI.

The agreement considers reusing the LTE’s mechanism by confirming HO completion via “UE contention resolution identity MAC CE”. That is captured in the specification TS 36.300 as follows:

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

Observation 2: TS 36.300 specifies that the RACH-less HO is completed when the UE receives the UE contention resolution identity MAC CE from the target cell.

1> if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured: 2> stop timer T304; 2> release rach-Skip;

Once the UE receives the RRC Reconfiguration message from the serving cell, the UE starts timer T304, which controls the HO procedure and determines when HO Failure might be declared. As captured in [3], during RAN2 discussions on RACH-less HO, it was agreed that timer T304 is required to avoid various error cases. Thus, the UE requires an indication to know when to stop the timer T304 and release RACH-less resources. The text in TS36.331 captures this condition as follows:

Note that the specifications considers that timer T304 is running at RRC level and stops after a successful reception indication at MAC level.

Observation 3: Legacy LTE behaviour defines that MAC layer indicates to upper RRC layer to stop the timer T304 and release RACH-less resources.

As indicated above, the RACH-less HO is completed when the UE receives the UE contention resolution identity MAC CE from the target cell. Note that the UE contention resolution identity MAC CE message involves a PDSCH scheduling that will be indicated via PDCCH transmission to the UE. However, according to the specification in TS 36.331, the NW could confirm the HO completion via anything in the PDSCH addressed to C-RNTI.

Observation 4: The NW could confirm the HO completion via anything in the PDSCH.

One option could be to use as indication a PDSCH transmission with DL data, but there may be cases where the NW have nothing to transmit. For that reason, the agreed indication for HO completion should be something valid to all UEs.

The current approach of using the UE contention resolution identity MAC CE, may not be the most optimal approach. The UE uses a MAC CE defined for contention resolution when there is no contention in RACH procedure and the UE contention resolution identity MAC CE takes 48 bits to indicate something that could be done with a shorter number of bits (it is a simple message to convey: RACH-less HO is completed, so theoretically a single bit could suffice). Further, the NW could have ready DL data to send, which will have to wait until the HO has been completed.

Using the MAC CE approach is a compromised solution that does not require further specification changes. However, there is room for further optimization with low specification efforts. A simple enhancement could be that the NW decides whether to schedule DL data (if available) or a MAC CE in the PDSCH.

Proposal 2: The NW confirms RACH-less HO completion scheduling DL data (if available). Otherwise, the NW schedules a UE contention resolution identity MAC CE.

## Combination of CHO and RACH-less procedure

The combination of RACH-less HO with conditional HO (CHO) procedure was briefly discussed during the last RAN2 meeting (see [2]).

In Rel-17, the time-based CHO procedure was agreed for Earth-fixed cells, as the baseline HO procedure since it allows an earlier preparation of the target cell, while the UEs can access the target cell in a distributed manner. However, the exact time for the CHO execution is unknown. This arises potential issues when combining with RACH-less HO.

The CHO preparation occurs earlier as compared with the conventional HO. If the UE is configured with a pre-allocated UL grant, the UL resources can be reserved for a longer period. While in case the UE is configured with a dynamic UL grant, the NW does not know when the UE will start monitoring the PDCCH because the exact CHO execution time is unknown (although the time window may be configured for NTN CHO).

Observation 5: In case of RACH-less CHO and pre-allocated UL grant, the NW might reserve UL resources for a longer period as compared with LTE RACH-less HO (i.e. not combined with CHO).

Observation 6: Dynamic UL grant can be less expensive configuration in terms of resources as compared with pre-allocated UL grant, but the NW does know when exactly the UE will start monitoring the PDCCH.

The combination of time-based CHO and RACH-less HO requires further discussion on the details, even though it may be feasible. However, in our view, such combination does not show major differences as compared with the LTE RACH-less HO and, hence, it does not provide significant gains.

Observation 7: In order to work, RACH-less time-based CHO must be configured and triggered in a similar manner as LTE’s RACH-less HO. Therefore, there is no relevant gain combining RACH-less with time-based CHO.

Proposal 3: RAN2 to further discuss whether there is a significant gain of CHO+RACH-less in comparison to the use of only RACH-less HO.

# Conclusion

The following observations and proposals are made in this document:

Observation 1: According to TS 38.331, the UL synchronization depends on the validity of SIB19 and it does not change whether the UE is configured with a RACH-based HO or a RACH-less HO.

Observation 2: TS 36.300 specifies that the RACH-less HO is completed when the UE receives the UE contention resolution identity MAC CE from the target cell.

Observation 3: Legacy LTE behaviour defines that MAC layer indicates to upper RRC layer to stop the timer T304 and release RACH-less resources.

Observation 4: The NW could confirm the HO completion via anything in the PDSCH.

Observation 5: In case of RACH-less CHO and pre-allocated UL grant, the NW might reserve UL resources for a longer period as compared with LTE RACH-less HO (i.e. not combined with CHO).

Observation 6: Dynamic UL grant can be less expensive configuration in terms of resources as compared with pre-allocated UL grant, but the NW does know when exactly the UE will start monitoring the PDCCH.

Observation 7: In order to work, RACH-less time-based CHO must be configured and triggered in a similar manner as LTE’s RACH-less HO. Therefore, there is no relevant gain combining RACH-less with time-based CHO.

Proposal 1: There are no further specification impact to discuss regarding the UL synchronization and the NTA acquisition in RACH-less HO.

Proposal 2: The NW confirms RACH-less HO completion scheduling DL data (if available). Otherwise, the NW schedules a UE contention resolution identity MAC CE.

Proposal 3: RAN2 to further discuss whether there is a significant gain of CHO+RACH-less in comparison to the use of only RACH-less HO.

# References

- RP-230809, "Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, 3GPP TSG RAN Meeting #99, Rotterdam/The Netherlands, March 20-23, 2023.

- R2-2306541, “Report from Break-out session on NR-NTN and IoT-NTN” 3GPP TSG RAN2 Meeting #122 Meeting, May 22th-26th 2023.

- R2-167495, “On stopping the T304 timer with the RACH-less handover”, 3GPP TSG RAN2 Meeting #96, Reno, US, November 2016.

- R2-2306646, “Summary of [AT122][106][NR-NTN Enh] RACH-less HO (Huawei)”, 3GPP TSG RAN2 Meeting #122, Incheon, Korea, 22-26 May, 2023.