3GPP TSG-RAN WG2 Meeting #123	R2-2307581

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Unchanged PCI and Satellite Switching without L3 Mobility

WID/SID:	NR_NTN_enh -Core

Document for:		Discussion and Decision

# Introduction

In RAN2#121bis-e and RAN2#122 meetings, the following agreements were made for the topic of unchanged PCI mobility:

Agreements in RAN2#121-bis-e 1. In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18). 2. Remove the part in brackets “as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18” in the LS to RAN1. The action to RAN1 will also ask for feedback for the hard satellite switch (not only the soft satellite switch case), e.g., action to RAN1 is to see if there are any major technical issues (as in the agreement).

Agreements in RAN2#122 1. t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens. 2. In hard switch unchanged PCI scenario (i.e., no handover), the UE needs to know the time the UE attempts to re synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e., no other IE) if the gap is very short/zero).

Furthermore, RAN2 sent an LS to RAN1 (R1-2304323) that was addressed during RAN1#113. In that LS, RAN2 asked about technical feedback on the hard satellite switch without PCI change and the feasibility to support soft satellite switching without PCI change (see response below).

Question 1: For hard satellite switching without PCI change, if RAN1 identifies any major technical issues? Reply: RAN1 discussed the resynchronization of UE when hard switching, given that new common TA, K_mac, ephemeris, and cell-specific K-offset are applied during resynchronization to new satellite. From RAN1 perspective, no feasibility issue is identified for hard satellite switching without PCI change.

There was no consensus during the RAN1 discussions on responding on the soft satellite switch case. Such discussions are expected to continue at RAN1#114.

# Discussion

In WID [1], one of the objectives for Rel-18 NR NTN is to continue enhancements for NTN mobility. The work will consider Rel-17 NR NTN as the baseline for further enhancements including:

- Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3].

As part of this objective, handover enhancements have been discussed in RAN2 during the past meetings. Among those, the unchanged PCI mobility has been proposed as a solution to reduce the signalling overhead.

In Rel-17 NTN, cells routed from different satellites are associated with different PCIs. That means, that a stationary UE in an Earth fixed cells deployment, will experience continuous L3 mobility (i.e., handover) as the serving satellite moves away from the cell area and a new satellite takes over. The principle of the unchanged PCI mobility is that the UE avoids the need for L3 mobility (i.e., no handover), which could reduce the signalling overhead (e.g., by not sending individual HO commands) in case of a large number of UEs within the same cell. For this solution to work, the serving gNB should not change after the satellite switch, while outbound and inbound satellites should provide the same cell; that means the same cell configuration (including the same PCI). Since the cell configuration and the serving gNB remain the same, the satellite switch should be mostly transparent for the UE.

Observation 1: For the satellite switching without L3 mobility to work, serving gNB and cell configuration should remain the same as much as possible after the satellite switch.

As shown in the agreements above, in RAN2#121bis-e meeting, it was decided that hard satellite switching with unchanged PCI was supported in case of quasi-Earth fixed cells, same SSB frequency and same serving gNB (no key change). Furthermore, RAN1 confirmed the feasibility of the hard satellite switching without PCI change given that new assistance information is applied during the re-synchronization to the inbound satellite.

This paper discusses remaining issues related to satellite switch with unchanged PCI that needs to be considered in Rel-18.

## Acquisition of new satellite assistance information

In RAN1#113 meeting, RAN1 confirmed the feasibility of the hard satellite switch given that the UE has valid satellite assistance information, which includes new common TA and new satellite ephemeris. Such information is required because for the satellite switch, even if the serving gNB does not change, the frequency and timing drift will change due to satellite change. Thus, the UE needs to know the new satellite’s ephemeris and the new common TA to pre-compensate full TA and Doppler shift before accessing the new cell originated from the inbound satellite.

Observation 2: Before the satellite switch, the UE needs to acquire inbound satellite assistance information for time and frequency pre-compensation.

For unchanged PCI mobility, it is unclear what will be the UE’s behaviour regarding the acquisition of the inbound satellite assistance information.

Observation 3: there is no RAN agreement and/or UE requirements to guarantee the acquisition of the inbound satellite assistance information before the satellite switching.

One possibility could be that the serving cell includes in SIB19 the neighbour cell information. The IE ntn-NeighCellConfigList in SIB19 allows providing a list of NTN neighbour cell configurations, which includes their NTN-config, carrier frequency and PCI. This option entails that neighbour cell’s PCI and serving cell’s PCI are the same in SIB19. It is unclear what would be the UE’s behaviour in this case. For instance, if a UE implementation sorts out cell assistance information by PCI, the UE could apply the wrong assistance information at the switching time.

Observation 4: The UE’s behaviour is unclear when the serving cell broadcasts neighbour cell’s assistance information with the same PCI as the serving cell.

Even if this option was workable, there is a risk that the UE acquires SIB19 too early and neighbour cell’s validity duration expires before t-Service.

Another disadvantage of this possibility is that the SIB19 would be unnecessarily overloaded. Neighbour cell’s information, which would be only needed shortly before t-Service, would be broadcast during the entire serving cell coverage time. Also, the information in SIB19 will be acquired by all UEs. For those UEs that need to perform L3 handover procedure, like Rel-17 NTN UEs, it could be inefficient since the (C)HO command (i.e., RRC Reconfiguration message) may contain up-to-date target cell assistance information. The unchanged PCI mechanism is meant to reduce the signalling overhead, therefore, by re-using SIB19 with the legacy UE’s behaviour, signalling is not reduced, but actually increased.

Observation 5: By using legacy specification framework, the unchanged PCI mechanism may overload SIB19 and, therefore, increase the signalling overhead.

Another option could be relying on the UE acquisition of SIB19 after UL synchronization is lost. For instance, if T430 expires at t-Service, the UE is mandated to inform lower layers that UL synchronization is lost and acquires SIB19 (see relevant excerpts in TS 38.331). However, the exact time when UL synchronization is obtained after SIB19 acquisition is up to UE implementation, which entails an unpredictable interruption time.

A third option could be to introduce a new information element for Rel-18 UEs in an existing SIB or in a new SIB, which contains the satellite information for a satellite switch without change of PCI and is marked as such. This way only Rel-18 and later releases UEs can read the information and confusion between source and target cell can be avoided.

Proposal 1: RAN2 to discuss how to ensure that the UE acquires valid inbound satellite assistance information prior to the satellite switch with unchanged PCI. Signalling shall not impact the legacy UEs.

## Time indication for overlapping and interruption

In the last meeting (i.e., RAN2#122) the timing for the UE to conduct re-synchronization at the new cell was discussed. It was agreed that t-Service in SIB19 could be interpreted as the time when a satellite switch occurs. That would indicate the time when the outbound satellite will stop covering the cell area, however, additional information on when the inbound satellite is available might be required. In case of hard satellite switch, the UE should know the duration of the interruption gap, while for the soft satellite switch, the UE should know the duration of the coverage overlapping.

Observation 6: For a successful re-synchronization, the UE should have knowledge on satellites’ availability.

Several options can be considered. The network could signal the time instance when the new cell becomes available or, alternatively, the duration of the interruption/overlapping time with respect to t-Service. The agreed parameter should consider the number of bits transmitted and the configurability. For instance, a relative time indication could avoid signalling an absolute UTC time (i.e., larger overhead).

Proposal 2: For soft and hard satellite switch, RAN2 to agree a time indication for the UE to know when the new satellite is available.

## Cell access procedure for DL synchronization

Even though unchanged PCI mobility is meant to be transparent for the UE, there are still synchronization actions that the UE should perform to connect to the inbound satellite. After the cell originated from the outbound satellite disappears, the UE should be able to detect, measure and decode the SSS/PBCH blocks of the new cell. During this process the UE first searches in frequency and time for the SSBs. Once the synchronization signals are found and decoded, the UE is ready to acquire MIB, SIB1 and the other SIBs.

For unchanged PCI mobility, since most of the cell configuration remains the same and the UE should acquire SIB prior to the satellite switch, this process could be optimized. However, the UE still should be able to find the SSBs in time and frequency before acquiring UL synchronization. In case of soft satellite switch, when performing the switch from the old cell to the new cell, the UE would have the same probability of finding the old cell as finding the new cell. Furthermore, the SSB of the new and old cell may overlap each other and interfere each other.

Observation 7: While attempting to access the new cell, the UE would risk to re-synchronize to the outbound satellite in soft satellite switching scenario.

One way to avoid confusion and interference is to ensure from network side that the SSB are not aligned in time and frequency and secondly that the UE is informed with assistance information such it will detect the right SSB. This information can for instance be included in the assistance information of the target cell as an offset to the SSB from the source cell for the Rel-18 UEs.

Thus, we propose the following:

Proposal 3: RAN2 to discuss the re-synchronization process and ways to avoid confusion between the two cells in case of soft satellite switching without PCI change.

## Cell access procedure for UL synchronization

The UL synchronization is the procedure in which the UE acquires the time and frequency to start UL transmissions. This is done via RA procedure and in NR, the UE conducts the RA procedure to obtain UL synchronization, TA and C-RNTI. In NR NTN, the full TA is estimated by the UE using the information in SIB, hence, RA procedure is used only for obtaining the UL grant and the C-RNTI.

The legacy RA procedure could be used for the UE to acquire UL grant in case of unchanged PCI mobility. However, this option involves flushing the HARQ buffers upon UL synchronization is lost and longer interruption gap. Furthermore, contention-based RA can lead to a massive RACH storm (all UEs will attempt RACH access at the same time and using the resources from the same common pool), while contention-free RACH requires to configure all the UEs with dedicated preambles (large signalling overhead and heavy resource usage).

Observation 8: Legacy RA procedure (both CBRA and CFRA) can lead to long interruption gap, RACH storm or large signalling overhead.

Alternatively, the unchanged PCI mobility could be combined with the RACH-less procedure (currently being discussed by this RAN WG). In this case, the UE could acquire UL grant via serving cell (i.e., pre-allocated UL grant) or dynamically acquire it via monitoring PDCCH (i.e., dynamic UL scheduling). The former case entails the overhead of signalling all the UEs within the same cell (UL grant should be allocated in a distributed fashion so the access storm is mitigated), while for the latter it is uncertain when the UE will be able to start PDCCH monitoring. It is still unclear how the RACH-less could be combined with unchanged PCI mobility since both procedures are still under discussion and their final shape is unknown.

Observation 9: RACH-less procedure could be beneficial for unchanged PCI mobility but both procedures are still under discussion in RAN2.

The satellite switching without L3 mobility should ensure seamless continuity for the UE when the switching occurs. This means that given that the cell configuration is the same, the gNB is the same and the UE should keep its connected state for a seamless transition, the UE could be scheduled with the UL resources prior to the switching. Particularly, in case of hard satellite switch, the UE could have the knowledge of its own DL and UL scheduling after the interruption gap.

Observation 10: If cell configuration and gNB are the same, the UE may not need to lose UL synchronization after t-Service. It needs to be clarified if the UE: can maintain the connected state, should flush the control buffers, and how long the interruption gap is expected to be, if the satellite switching is supposed to be transparent to the UE.

Proposal 4: RAN2 to discuss the need for using RA procedure and the feasibility of using RACH-less procedure with unchanged PCI scheme.

Proposal 5: RAN2 to discuss the UE’s behaviour during the interruption gap and answer questions such if the UE can skip RA-related procedures, maintain the UL synchronization, and remain in connected mode during the entire switching process.

Given that some companies foresee a very short gap and considering that the intention is to make the satellite switch as transparent as possible, while the serving gNB will not change, our understanding is that the UE could continue seamlessly transmitting and therefore the scheduling across satellites (with the same gNB) is possible. That means that the source cell could schedule in the target cell (same cell - same gNB). In that case, a problem could arise in case the propagation delay between satellite and UE increases for more than one slot duration. If the new TA becomes larger than the old TA due to a longer distance to the new satellite, the first scheduling in the target cell may need to happen at the same time as the last scheduling in the source cell (which is not possible because the UE cannot make two simultaneous UL transmissions).

Observation 11: Unchanged PCI mobility should target scheduling across cells, where the gNB should be aware of TA changes after the satellite switch.

Proposal 6: RAN2 to discuss if current scheduling means are sufficient to enable seamless transmission across cells without causing UL transmission overlap.

One argument to support mobility with unchanged PCI is that the cell keeps the protocol stack and the security keys. That implies that state of PHY, MAC, RLC, PDCP and RRC entities should be maintained as much as possible. According to legacy UE’s behaviour, once the UE detaches from the serving cell, it losses its UL synchronization, and flushes control buffers. In our understanding, to achieve a short interruption gap, the UE should keep its state, maintain the keys and do not reset HARQ, RLC and PDCP buffers.

Such behaviour must be specified, and the UE needs to know whether to perform cell mobility with or without handover procedure. We propose that the UE should considers the following aspects in case L3 mobility is avoided:

Proposal 7: In order to support mobility without L3 handover, Rel-18 UEs should keep security keys without flushing control buffers from MAC, RLC and PDCP layers.

## Cell measurements

Another important aspect to consider is the change of propagation conditions after the satellite switch. During the handover process, the UE re-configures its power control parameters to adjust to the new radio conditions. With unchanged PCI mobility, the UE does not perform L3 mobility, which means that measurement filters are not reset for instance. The filtering average-out could consider past samples that were measured from the old satellite while the new sample belongs to the new satellite - this will lead to erroneous cell measurements.

Observation 12: If the UE does not perform L3 mobility, the UE’s measurement filtering might jointly average out samples from the old satellite and from the new satellite.

Proposal 8: RAN2 to discuss ways for the UE to avoid that measurement configuration and filtering after satellite switch considers samples from the outbound satellite.

## Backwards compatibility

One of the major drawbacks of introducing the unchanged PCI mobility is that it will not be supported by Rel-17 NTN UEs which will co-exist with Rel-18 NTN UEs. Unless Rel-17 UEs are commanded to perform a handover, these UEs will fail to make a transition to the new cell. If no RRC Reconfiguration message is received by the UE, the terminal will declare RLF and will attempt to reconnect via RRC Reestablishment procedure. This procedure can be costly in terms of UE’s power consumption, signalling overhead and user experience. Even if these UEs received an RRC Reconfiguration command, the target cell will share the same PCI as the serving cell, which may cause implementation-related problems.

The contribution in [2] proposed to use a similar approach where the UE does not conduct L3 mobility, but the new cell has a different PCI. That solution could potentially solve the backwards compatibility with Rel-17 because these could conduct a handover to a cell with a different PCI, while the remainder of cell configuration is the same. In our view, the specification work should focus on the UE’s behaviour during the satellite switch without the L3 mobility where “changed PCI” and “unchanged PCI” could be specification options, up to NW configuration.

Proposal 9: RAN2 to clarify whether Rel-17 NTN UE can conduct a handover procedure towards the same cell (with same PCI) or whether this may cause some implementation issues.

# Conclusion

The following observations and proposals are made in this document:

Observation 1: For the satellite switching without L3 mobility to work, serving gNB and cell configuration should remain the same as much as possible after the satellite switch.

Observation 2: Before the satellite switch, the UE needs to acquire inbound satellite assistance information for time and frequency pre-compensation.

Observation 3: there is no RAN agreement and/or UE requirements to guarantee the acquisition of the inbound satellite assistance information before the satellite switching.

Observation 4: The UE’s behaviour is unclear when the serving cell broadcasts neighbour cell’s assistance information with the same PCI as the serving cell.

Observation 5: By using legacy specification framework, the unchanged PCI mechanism may overload SIB19 and, therefore, increase the signalling overhead.

Observation 6: For a successful re-synchronization, the UE should have knowledge on satellites’ availability.

Observation 7: While attempting to access the new cell, the UE would risk to re-synchronize to the outbound satellite in soft satellite switching scenario.

Observation 8: Legacy RA procedure (both CBRA and CFRA) can lead to long interruption gap, RACH storm or large signalling overhead.

Observation 9: RACH-less procedure could be beneficial for unchanged PCI mobility but both procedures are still under discussion in RAN2.

Observation 10: If cell configuration and gNB are the same, the UE may not need to lose UL synchronization after t-Service. It needs to be clarified if the UE: can maintain the connected state, should flush the control buffers, and how long the interruption gap is expected to be, if the satellite switching is supposed to be transparent to the UE.

Observation 11: Unchanged PCI mobility should target scheduling across cells, where the gNB should be aware of TA changes after the satellite switch.

Observation 12: If the UE does not perform L3 mobility, the UE’s measurement filtering might jointly average out samples from the old satellite and from the new satellite.

Proposal 1: RAN2 to discuss how to ensure that the UE acquires valid inbound satellite assistance information prior to the satellite switch with unchanged PCI. Signalling shall not impact the legacy UEs.

Proposal 2: For soft and hard satellite switch, RAN2 to agree a time indication for the UE to know when the new satellite is available.

Proposal 3: RAN2 to discuss the re-synchronization process and ways to avoid confusion between the two cells in case of soft satellite switching without PCI change.

Proposal 4: RAN2 to discuss the need for using RA procedure and the feasibility of using RACH-less procedure with unchanged PCI scheme.

Proposal 5: RAN2 to discuss the UE’s behaviour during the interruption gap and answer questions such if the UE can skip RA-related procedures, maintain the UL synchronization, and remain in connected mode during the entire switching process.

Proposal 6: RAN2 to discuss if current scheduling means are sufficient to enable seamless transmission across cells without causing UL transmission overlap.

Proposal 7: In order to support mobility without L3 handover, Rel-18 UEs should keep security keys without flushing control buffers from MAC, RLC and PDCP layers.

Proposal 8: RAN2 to discuss ways for the UE to avoid that measurement configuration and filtering after satellite switch considers samples from the outbound satellite.

Proposal 9: RAN2 to clarify whether Rel-17 NTN UE can conduct a handover procedure towards the same cell (with same PCI) or whether this may cause some implementation issues.

# References

- RP-230809, "Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, 3GPP TSG RAN Meeting #99, Rotterdam/The Netherlands, March 20-23, 2023.

- R2-2306517, ”Unchanged PCI solution vs PCI change only solution”, Sequans Communications, 3GPP RAN2#122, South Korea, May 2023.