3GPP TSG-RAN WG2 Meeting #123	R2-2307584

Toulouse, France, 21 - 25 of August 2023

Agenda item:	7.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Even Further Details on Flight Path Plan (FPP)

WID/SID:	NR_UAV - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#119bis (October 2022) the following UAV-related agreements have been made [1]:

Agreements: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

At RAN2#120 (November 2022) the following has been agreed on flight path plan [2]:

Agreements: - A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. - A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity - No requirements are placed on spatial distribution of waypoints - A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. - UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. - UAI message can also be used to indicate the UE has flight path availability. - FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS.

At RAN2#121 (February 2023) the following agreements were made on flight path plan [3]:

Agreements: 1. The granularity of flightpath timestamp is 1s. 2. Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Additionally, an email discussion [POST121][314][UAV] discussed aspects of the flight path plan  and the following agreements were made on flight path plan [4]:

- Flightpath update indication in UAI is configurable by the network - Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE - Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] - As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) - As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling

At RAN2#122 (May 2023) the following additional agreement has been made on flight path plan [5]:

Agreements: 1. The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In this paper we focus on UAV’s flight path plan (FPP) related open issues which were discussed during RAN2 #122 and captured in [5] and also partly addressed in our earlier contributions [6][7].

# 2	Flight Path Plan

## 2.1	The role of flight path plan

We begin with the short background why flight path plan is actually needed and was introduced. LTE Rel-15 introduced the support of flight path reporting. For many UAVs the flight route may be predefined and known in advance. If that is the case, then 3GPP-compliant network can take advantage and ask the UAV UE to report such details to gNB. In LTE this is done via UEInformationRequest/UEInformationResponse (see details in [9]) and the UE may report up to 20 waypoints. Each waypoint consists of location information and timestamp, indicating the expected time window within which the UAV UE will visit indicated geo-location. These principles are shown in Fig. 1. In the RAN2#119bis meeting it was agreed to make the time information an optional component of the FPP [1]. It was also discussed what is the actual role of the flight path plan and how the NW uses it (e.g., which protocol layers are involved).

Fig. 1 Flight path plan information reporting by LTE UAV UE [10]

The GSMA (ACJA and GUTMA) have been working on identifying the enablers for BVLOS (Beyond Visual Line of Sight) operations at scale [11][12]. One of the key aspects which has been addressed is how the cellular coverage, and service reliability can be ensured for BVLOS flights. Coverage and flight path data sharing/exchange between MNOs and USS (UAS Service Suppliers) has been marked as a key enabler. The exchange of this information is primarily necessary in the flight planning phase, to determine an optimal flight route/trajectory, whilst during the flight – the context of the RAN2 discussions – the adherence to the flight plan should be monitored at the USS.

The MNO is still responsible to monitor and provide reliable connectivity along the UAV planned route, even in the situations when radio conditions might change in the cellular network due e.g., traffic load variations. This is necessary such that the USS can take the appropriate mitigation actions, such as replanning of the flight route. In this context, we note that for the monitoring of service quality before and after route (re)planning by the USS, the MNO would greatly benefit from awareness of the actual UAV flight plan. This awareness must be, as much as possible, available in ‘real-time’ to the NG-RAN but does not have to cover the entire flight route of the UAV, just the currently relevant flight segments. This is already partially achievable by using the LTE Release 15 mechanism for flight path reporting.

In [11] the recommendation is to use the so-called Pose Data Structure which “provides a location in space (3D), orientation of an aircraft and time applicable” and includes information on the aquisitionDateTime, position, altitude and orientation. The aquisitionDateTime is defined also as future time stamp value, i.e., used to indicate planned/estimated flight paths. Firstly, this recommended reporting of the location/path information at UAV application level leads to the conclusion that the information required to provide the Pose Data Structure is already assumed to be available at the UAV, else the USS would not authorize the flight mission and be able to track the UAV during the flight. Secondly, having such information available at the UAV means that it can also be used to build the enhanced FlightPathInfoReport which the NG-RAN would then use.

Observation 1: As per GSMA recommendations, a reliable flight path plan availability at the NG-RAN is a key enabler of reliable communication throughout the UAV’s flight.

We further note that in the email discussion [POST121][314][UAV] [4] the proposal 7 on “RAN2 to discuss the support of critical information related to flight path reporting such as emergency landing” has been suggested. This indicates that the content of the FPP has to be meaningful and usable by the network in order to allow for such critical operations as emergency landing.

Observation 2: Critical operations such as emergency landing, suggested in RAN2, are not possible without a well -defined FPP with sufficiently detailed information included to allow the network (operator) to take critical decisions about the UAV mission.

The recent discussions within SA WG#2 on Release 19 UAV study item topics (UAS_Ph3_SID)[S2-2308391] have identified as part of the primary objectives to “[…] support service exposure and interactions between MNOs and UTM functions for e.g. pre-mission flight planning, in-mission flight monitoring […]”. It is evident that the in-mission flight monitoring cannot be performed by an MNO unless it has reliable near real-time information about the flight path plan of the aerial UEs it is serving.

## 2.2	Flight path content

In LTE flight path plan report (FlightPathInfoReport) is defined as follows [9]:

FlightPathInfoReport-r15 ::=		SEQUENCE  {

flightPath-r15	SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15	OPTIONAL,

dummy							SEQUENCE  {}							OPTIONAL

}

with the WayPointLocation IE containing these parameters [9]:

WayPointLocation-r15 ::=			SEQUENCE  {

wayPointLocation-r15						LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10		OPTIONAL

}

It was agreed to use LocationCoordinates, as defined in TS 37.355 [13], to report the waypoint location in Release 18 NR. As also discussed in [8], in this option, each waypoint, including time information, would be encoded using up to 776 bits (728 bits for LocationCoordinates using a maximum of 15 PolygonPoints, and 48 bits for AbsoluteTimeInfo-r16). Thus, for a configuration with the maximum number of 20 waypoints, the FPP report size would be 15520 bits, very close to the size limit of one RRC message. However, using a EllipsoidPointWith* instead of Polygon information element types, the 20 waypoints FPP report size would be up to 2960 bits; while with HighAccuracyEllipsoidPointWith* the 20 waypoints FPP report size would be up to 3580 bits. The use of Ellipsoid-Point would reduce the report size to 1920 bits. We also note that the use of 15 PolygonPoints for one waypoint is likely not justified as in general the location accuracy/uncertainty regions (3-D) are symmetrical around the true location, hence fewer PolygonPoints can also be used.

As agreed, the FPP reporting is network controlled, therefore the network can request the UE to use any of the above options (via ASN.1 CHOICE element) for providing location information with different accuracies, hence effectively control the signalling overhead, depending on the actual use of the FPP. For example, the AV UE could be required to report FPP with high accuracy less frequently during the cruise flight segments, while higher waypoint accuracy would be required during take-off and landing flight segments.

Release 10 LTE, however, defines a similar IE to LocationCoordinates in TS 36.331 [9] called LocationInfo-r10. These have been copied into Table 1 below. Note that while the LocationInfo-r10 IE in TS 36.331 [9] contains a field called locationCoordinates-r10, this is not the same as LocationCoordinates, which is currently specified in TS 37.355 [13]. Fields that are common between the two versions have been highlighted in yellow so that we can analyse the differences.

Notably missing from LocationInfo-r10 are the high accuracy position estimates, and from LocationCoordinates are the horizontal and vertical velocity, and the GNSS ToD. Since the timestamp is reported separately, the latter is not needed.

Table 1 Comparison of LocationCoordinates and LocationInfo-10

| 37.355 (v17.4) [10] | 36.331 (v17.4) [7] |
| --- | --- |
| LocationCoordinates ::= CHOICE { ellipsoidPoint Ellipsoid-Point, ellipsoidPointWithUncertaintyCircle Ellipsoid-PointWithUncertaintyCircle, ellipsoidPointWithUncertaintyEllipse EllipsoidPointWithUncertaintyEllipse, polygon Polygon, ellipsoidPointWithAltitude EllipsoidPointWithAltitude, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid EllipsoidPointWithAltitudeAndUncertaintyEllipsoid, ellipsoidArc EllipsoidArc, …, highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510 HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15, highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510 HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15, ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680 HA-EllipsoidPointWithScalableUncertaintyEllipse-r16, ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680 HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 } | LocationInfo-r10 ::= SEQUENCE { locationCoordinates-r10 CHOICE { ellipsoid-Point-r10 OCTET STRING, ellipsoidPointWithAltitude-r10 OCTET STRING, …, ellipsoidPointWithUncertaintyCircle-r11 OCTET STRING, ellipsoidPointWithUncertaintyEllipse-r11 OCTET STRING, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11 OCTET STRING, ellipsoidArc-r11 OCTET STRING, polygon-r11 OCTET STRING }, horizontalVelocity-r10 OCTET STRING OPTIONAL, gnss-TOD-msec-r10 OCTET STRING OPTIONAL, …, [[ verticalVelocityInfo-r15 CHOICE { verticalVelocity-r15 OCTET STRING, verticalVelocityAndUncertainty-r15 OCTET STRING } OPTIONAL ]] } |

Observation 3: In LTE, the WayPointLocation-r15 contains wayPointLocation-r15, which is defined as LocationInfo-r10, which reports horizontal and vertical velocity as well as GNSS ToD.

Observation 4: In LPP, the LocationCoordinates IE does not contain any velocity information, but includes high accuracy position estimates as choices.

TS 38.331 v17.4.0 [14] defines LocationInfo-r16, which is similar to LocationInfo-r10 [9]. Unlike LocationInfo-r10, LocationInfo-r16 reuses LocationCoordinates, defined in 37.355 [13] through the OCTET STRING locationCoordinate-r16, but similarly to LocationInfo-r10, also includes the GNSS ToD and velocity estimates as optional information.

LocationInfo-r16 ::=      SEQUENCE {

commonLocationInfo-r16    CommonLocationInfo-r16          OPTIONAL,

bt-LocationInfo-r16       LogMeasResultListBT-r16         OPTIONAL,

wlan-LocationInfo-r16     LogMeasResultListWLAN-r16       OPTIONAL,

sensor-LocationInfo-r16   Sensor-LocationInfo-r16         OPTIONAL,

…

}

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

Observation 5: Like LTE, NR also defines LocationInfo. The NR version of LocationInfo directly includes LocationCoordinates as defined in the LPP specification, TS 37.355, as an OCTET STRING.

If the intention is to exclude velocity information from the waypoint location information and to include higher accuracy location estimates, then no change is needed to the agreement made regarding the use of LocationCoordinates from TS 37.355 [13] as the location field in WayPointLocation.

When using CommonLocationInfo-r16 or LocationInfo-r16 from 38.331[14], the size of the FPP report is larger, partly due to the byte alignment requirement of the OCTET STRING. However, the use of CommonLocationInfo-r16 or LocationInfo-r16 from 38.331[14] could be motivated. For example, when using a combination of EllipsoidPointWithAltitude  (64 bits) with gnss-TOD-msec (24 bits) and horizontalVelocity (24 bits) information, this results in a total 20 waypoint FPP report size of 2240 bits. This size is comparable with the report sizes when using LocationCoordinates from TS 37.355 [13], with the added benefit of having velocity information included.

If we choose to use CommonLocationInfo-r16 or LocationInfo-r16 from 38.331[14] instead of LocationCoordinates from 37.355 [13], then we propose the following. The use of the LocationInfo-r16 from 38.331[14] is further motivated based on the existence of the gnss-TOD-msec-r10 information element, directly associated with locationCoordinates-r10; therefore, the inclusion of a separate timeStamp-r18 in the way points of the flight path report can be kept optional.

Proposal 1: Use OCTET STRING to convey the LocationCoordinates in the WayPoint definition.

Observation 6:  Timestamp (a.k.a. time information) is an essential part of the flight path plan, enabling the network to take proper decisions regarding radio resource reservation and QoS. The flight plan report must also include a timestamp of each included waypoint.

Proposal 2: Use the name WayPoint instead of WayPointLocation in NR.

Below the exemplary ASN.1:

maxWayPoint-r18 ::=		INTEGER :: 20 -- Maximum number of flight path information waypoints

FlightPathInfoReport-r18 ::=		SEQUENCE {

flightPath-r18	SEQUENCE (SIZE (1..maxWayPoint-r18)) OF WayPoint-r18	        OPTIONAL

}

WayPoint-r18 ::=			SEQUENCE {

locationCoordinate-r18					OCTET STRING,

timeStamp-r18							AbsoluteTimeInfo-r16		            OPTIONAL

}

AbsoluteTimeInfo-r16 ::=			BIT STRING(SIZE(48))

locationCoordinate field descriptions

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [9]. The first/leftmost bit of the first octet contains the most significant bit. AbsoluteTimeInfo Indicates an absolute time in a format YY-MM-DD HH:MM:SS and using BCD encoding. The first/ leftmost bit of the bit string contains the most significant bit of the most significant digit of the year and so on.

Proposal 3: Use the above ASN.1 WayPoint definition as starting point to define the content of the FlightPathInfoReport with the flightPath-r18 as mandatory field in this IE.

Some opinions expressed at RAN2#119bis stated that UAV UE anyway is not required to be in the reported location within the associated time information i.e., what if the UAV UE is not there at the previously reported time? It could be true that 3GPP RAN WGs will not work on the binding requirements in this area and the UE’s behaviour will be best effort in this case. It has been agreed in  [4] that the UAV UE can report deviation from the initially reported FPP. Moreover, as presented in section 2.1, the USS which controls the UAV flight and mission relies on the flight path/location information received from the UAV, thus this information can be assumed to be reliable also for 3GPP RAN purposes.

As we have already pointed out, in LTE Rel-15 there is nothing determined with respect to e.g., how the waypoints are spaced in time/location and in fact the UE could even report 20 times the same value [9]. This issue has not been addressed at RAN2#120 and RAN2#121, on the contrary – it has been agreed there will be no spatial requirements regarding how the waypoints are distributed. Thus, there will be no requirement that among those waypoints there needs to be the one representing the start or destination, while in our opinion, these two locations may give some overall knowledge on the expected UAV flight path, at least locally, and valid for a limited time window. For example, the start-/end- info could be linked to a shorter or longer time window, estimated at the UAV, and depending on the UAV application implementation and/or flight mission. Under this assumption, we consider it is reasonable and useful, the FlightPathInfoReport at minimum shall contain two waypoints with mandatory timestamps as estimated by the UAV application. Any other additional waypoints, between start and end, if included in the FlightPathInfoReport can optionally include the timestamp information when the includeTimeStamp is set to TRUE in the flightPathInfoReq.

Observation 7: Due to the decision not to define requirements for spatial distribution of waypoints, the network has no guarantee whatsoever that the content of FPP will be meaningful for any decisions regarding radio resource reservation and QoS.

Due to these circumstances, we would still insist on having at least two clearly defined waypoints in FPP (start and destination) with mandatory time stamp information. The remaining, up to 18 waypoints, can be optionally included in the flight path report and may not have spatial or temporal requirements.

Proposal 4: Flight path plan in NR Rel-18 distinguishes the start- and end-related waypoints from other reported waypoints. These waypoints shall contain timestamp. Other remaining timestamps may have no spatial or temporal requirements.

FlightPathInfoReport-r18 ::=		SEQUENCE {

startWayPoint-r18				WayPoint-r18,

endWayPoint-r18					WayPoint-r18,

flightPath-r18		SEQUENCE (SIZE (1..maxWayPoint-r18 - 2)) OF WayPoint-r18	    OPTIONAL

}

WayPoint-r18 ::=			SEQUENCE {

locationCoordinate-r18					OCTET STRING,

timeStamp-r18							AbsoluteTimeInfo-r16		            OPTIONAL

}

## 2.3	Flight path reporting

For triggering the UE to report whether it has a new FPP, we could consider time and distance, as follows also from the RAN2 #122 discussions [5]:

- The time-based event trigger will make the UE to inform the network of a new FPP if the timestamps for the included waypoints have changed more than a configurable threshold e.g., estimated timestamps are more than 10 seconds in the future).

- The distance-based event trigger will trigger the UE to inform the network of a new FPP if it deviates more than a configurable distance threshold from the currently available FPP (distance calculated based on all waypoints) e.g., estimated FPP deviation is more than 10 meters.

Observation 8: Even though there may be no RAN-level UAV UE requirement for being present in the reported location within the associated time information, the UE shall indicate to the NW if it has an updated FPP when the update meets the defined criteria for sending such indication.

The triggers listed above could be combined, depending on the UAV mission type/scenario, requests from USS/UTM, overall flight path info as received via the CN, etc. The flight path info and subsequent updates triggering conditions could be configured in the flightPathInfoReportConfig  [9], during the first flight path report configuration. The flightPathInfoReportConfig configuration could also include the actual format of the location information (hence its accuracy) to be included in the report (see Section 2.2), thus allow full flexibility for the network.

Observation 9: Triggering configuration for indicating a new flight path could be signalled in FlightPathInfoReportConfig.

Proposal 5: For the triggering of flightPathInfoAvailable in the UEAssistanceInformation based on time-based event trigger and distance-based event trigger consider the ‘X waypoints changed’ as the main condition to be checked; the change of the waypoints is measured either in location or time and is relative to the corresponding threshold values.

Delta signaling for FPP could be considered helpful to reduce the size of the message frequently sent from/to the network for some cases/scenarios. However, the options listed in Proposal 5 can handle the main cases identified where FPP updates are required in the form of delta reporting (e.g. Case 2 and 3 in [8]). We note that in practice, other cases with outdated waypoints along a FPP or new waypoints due to changes in the FPP, can be handled similarly to the main cases addressing timestamp or location changes.

Furthermore, for flight path plan reporting, if the triggers to update the NW with a new FPP are set appropriately, it is not expected such ‘update’ reporting will occur often. The network has all the means to control the rate of FPP reporting and the type of location information to be included (see Section 2.2). Thus, we do not think agreeing to support delta signaling for FPP is a necessary step.

Proposal 6: RAN2 does not pursue the specification for delta signaling for flight path reporting in NR Release 18.

Similar simplicity should be kept for resolving the issue on whether initial and updated FPP indication shall be differentiated. We see no point in distinguishing these two indications. After all, this is reported per UE and the gNB knows whether it has received a flight path plan from that particular UE. Thus, the NW can tell if the current indication is for the initial reporting or reporting of an updated version of FPP.

Proposal 7:  Confirm that a single indication is used for both initial and updated flightpath plan.

# 3	Conclusion

In this document we have made the following proposals and observations:

Observation 1: As per GSMA recommendations, a reliable flight path plan availability at the NG-RAN is a key enabler of reliable communication throughout the UAV’s flight.

Observation 2: Critical operations such as emergency landing, suggested in RAN2, are not possible without a well -defined FPP with sufficiently detailed information included to allow the network (operator) to take critical decisions about the UAV mission.

Observation 3: In LTE, the WayPointLocation-r15 contains wayPointLocation-r15, which is defined as LocationInfo-r10, which reports horizontal and vertical velocity as well as GNSS ToD.

Observation 4: In LPP, the LocationCoordinates IE does not contain any velocity information, but includes high accuracy position estimates as choices.

Observation 5: Like LTE, NR also defines LocationInfo. The NR version of LocationInfo directly includes LocationCoordinates as defined in the LPP specification, TS 37.355, as an OCTET STRING.Proposal 1: Use OCTET STRING to convey the LocationCoordinates in the WayPoint definition.

Observation 6:  Timestamp (a.k.a. time information) is an essential part of the flight path plan, enabling the network to take proper decisions regarding radio resource reservation and QoS. The flight plan report must also include a timestamp of each included waypoint.

Proposal 2: Use the name WayPoint instead of WayPointLocation in NR.

Proposal 3: Use the above ASN.1 WayPoint definition as starting point to define the content of the FlightPathInfoReport with the flightPath-r18 as mandatory field in this IE.Observation 7: Due to the decision not to define requirements for spatial distribution of waypoints, the network has no guarantee whatsoever that the content of FPP will be meaningful for any decisions regarding radio resource reservation and QoS.

Proposal 4: Flight path plan in NR Rel-18 distinguishes the start- and end-related waypoints from other reported waypoints. These waypoints shall contain timestamp. Other remaining timestamps may have no spatial or temporal requirements.

Observation 8: Even though there may be no RAN-level UAV UE requirement for being present in the reported location within the associated time information, the UE shall indicate to the NW if it has an updated FPP when the update meets the defined criteria for sending such indication.

Observation 9: Triggering configuration for indicating a new flight path could be signalled in FlightPathInfoReportConfig.

Proposal 5: For the triggering of flightPathInfoAvailable in the UEAssistanceInformation based on time-based event trigger and distance-based event trigger consider the ‘X waypoints changed’ as the main condition to be checked; the change of the waypoints is measured either in location or time and is relative to the corresponding threshold values.

Proposal 6: RAN2 does not pursue the specification for delta signaling for flight path reporting in NR Release 18.

Proposal 7:  Confirm that a single indication is used for both initial and updated flightpath plan.

# References

- R2-2210803	Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV	Session chair (InterDigital) 3GPP TSG RAN2 Meeting #119bis Electronic Meeting, Oct 10 - 19, 2022.

- R2-2213352	Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning 3GPP TSG RAN2 Meeting #120, Nov 14 - 18, 2022.

- R2-2301903	Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning	Session chair (InterDigital) 3GPP TSG RAN2 Meeting #121, Feb 27 – Mar 3, 2023.

- R2-2302867 Report from [POST121][314][UAV] Flight path reporting (Intel), 3GPP TSG-RAN WG2 Meeting #121-bis-e, 17th April – 26th April 2023

- R2-2306543 Report from UP, Small data, URLLC/IIoT, RACH indication, NES and UAV, 3GPP TSG-RAN WG2 Meeting #122, Incheon, South Korea, 22nd  – 26th  of May 2023.

- R2-2302901, On Flight Path Plan (FPP), Nokia, 3GPP TSG-RAN WG2 Meeting #121-bis-e, 17th April – 26th April 2023.

- R2-2305887, Further Details on Flight Path Plan (FPP), Nokia, 3GPP TSG-RAN WG2 Meeting #122, Incheon, South Korea, 22nd  – 26th  of May 2023.

- R2-2305109, Delta reporting of flight path plan, Qualcomm Incorporated, 3GPP TSG-RAN WG2 Meeting #122, Incheon, South Korea, 22nd  – 26th  of May 2023.

- 3GPP Technical Specification 36.331, version 17.4.0, March 2023.

- J. Stanczak, D. Kozioł, I. Z. Kovács, J. Wigard, M. Wimmer and R. Amorim, "Enhanced Unmanned Aerial Vehicle Communication Support in LTE-Advanced," 2018 IEEE Conference on Standards for Communications and Networking (CSCN), 2018, pp. 1-6, doi: 10.1109/CSCN.2018.8581827.

- GSMA/ACJA/GUTMA, “Interface for Data Exchange between MNOs and the UTM Ecosystem Network Coverage Service Definition” v1.00, February 2021.

- GSMA, “Aerial Connectivity Joint Activity – Work Task #2: Reference Method for assessing Cellular C2 Link Performance and RF Environment Characterization for UAS”, October 2022.

- 3GPP Technical Specification 37.355, version 17.4.0, March 2023.

- 3GPP Technical Specification 38.331, version 17.4.0, March 2023.

- R2-2210355	On measurements and measurement reporting enhancements for Rel-18 UAVs 3GPP TSG RAN2 Meeting #119bis Electronic Meeting, Oct 10 - 19, 2022.