3GPP TSG-RAN WG2 Meeting #123															R2-2307596

Toulouse, France, August 21-25, 2023

Agenda item:		7.11.3

Source:				Samsung

Title:					Rel-18 MII Enhancements

Document for:		Discussion & Decision

# 1 Introduction

The following agreements were made in previous RAN2 meeting:

- R2-122: - The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs. - No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed). - When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. - FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII) - No additional information is added to MII on top of what has been already agreed.

This contribution discusses the open issues and proposals for Rel-18 MBS Interest Indication (MII) enhancements.

# 2 Discussion

## 2.1 Rel-18 MII Enhancement - Frequency of interest determination

Procedure for frequency of interest determination considers a frequency to be part of MBS frequencies of interest if certain conditions are fulfilled. For Rel-18 MII reporting enhancement, the procedure needs to be amended to consider a condition that “at least one of the MBS sessions is from non-serving cell for the concerned frequency”. This is an alternative condition for the frequency of interest determination on the basis of PCell’s SIB21 or USD in the legacy spec. Further, MII reporting is performed when SIB1 from PCell includes nonServingCellMII.

Proposal 1. For Rel-18 MII reporting enhancement, MII procedure for frequency of interest determination is amended to consider a condition that at least one of the MBS sessions is from non-serving cell for the concerned frequency (Adopt TP#1 in annex).

## 2.2 Rel-18 MII Enhancement - Trigger for MBS Interest Indication

“Upon change to a PCell providing nonServingCellMII in SIB1”, UE should initiate the MII procedure. We understand this is quite reasonable and important in order to update network about the UE’s capability at this event. However, this trigger is missed in the triggers list (refer to Initiation section in the specification) and should be captured.

Proposal 2. Add “upon change to a PCell providing nonServingCellMII in SIB1” as a trigger to initiate the MII procedure (Adopt TP#2 in annex).

## 2.3 Rel-18 MII Enhancement - Separately handling set of frequencies of interest for non-serving cell

The reporting of MBS Interest Indication information for legacy (serving cells) and that for non-serving cell(s) are quite distinct and independent. Consequently, it is needed that there should be a clear separation when dealing with the set of frequencies for MBS broadcast reception for serving cell(s) and the set of frequencies for MBS broadcast reception for non-serving cell(s). Accordingly, the procedural description in the specification needs to be amended to reflect this distinction.

Proposal 3. MII procedure handles set of frequencies of interest for non-serving cell separately from the legacy set of frequencies of interest for serving cell(s) to be reported in MII (Adopt TP#3 in annex).

## 2.4 Rel-18 MII Enhancement – Configuration update on non-serving cell

UE’s full capability reported by UE capability signalling may be shared by unicast/multicast and broadcast. UE’s actual processing capability available for unicast is the UE capability which can be used for unicast and multicast. Depending on the configuration update on non-serving cell for UE’s broadcast reception, the actual processing capability available for unicast may change for a UE, based on simultaneous support for broadcast and unicast. The configuration update may include the change in scheduling configuration for broadcast reception on non-serving cell. MII signalling for the actual processing capability used for broadcast reception based on configuration update on non-serving cell is useful for UE’s proper operation. gNB may use this information to adapt UE’s RRC configuration on unicast and multicast.

Observation 1: UE’s actual processing capability available for unicast (i.e., full processing capability minus processing capability used for broadcast reception on non-serving cell) may change based on configuration update on non-serving cell.

Figure 1: Example of gap between estimated processing capability used by broadcast and actual processing capability used by broadcast (averaged over a scheduling window)

It can be noticed that agreed information to be signaled in MBSInterestIndication message (i.e. broadcast frequency, sub-carrier spacing, and bandwidth) are all pertaining to frequency domain. However, as the broadcast service reception is not a continuous activity and is typically have a scheduling pattern e.g. DRX configuration based MTCH reception. Further, the broadcast services and/or their scheduling configuration may be subject to change. Without consideration to time-domain scheduling configuration information, there will be over-estimation on the processing capability used for broadcast reception. Resultantly, UE’s actual processing capability available for unicast is under estimated and UE’s configuration/performance for unicast is lower than achievable. This is illustrated in Figure 1.

Observation 2: Without consideration to broadcast scheduling configuration information in MII signaling, there will be over-estimation on the processing capability used for broadcast reception. Resultantly, UE’s actual processing capability available for unicast is under estimated and UE’s configuration/performance for unicast is lower than achievable.

To overcome this in an efficient and simple manner, a ‘mbs-NonServingFactor’ can be included to consider broadcast scheduling configuration information for non-serving cell in MII e.g. if the broadcast reception is 40% of the time on the non-serving cell, the mbs-NonServingFactor can be indicated as 0.4 (or 4/10). That is, actual UE processing capability used for broadcast is a fraction of the reported UE processing capability for broadcast. With this, actual UE processing capability used for broadcast reception can be determined effectively and accurately. At the same time, this is quite signaling-efficient and readily actionable-information than signaling entire DRX configuration parameters.

Proposal 4. RAN2 to consider configuration updates for broadcast reception on non-serving cell for the Rel-18 MII enhancement. UE includes a ‘mbs-NonServingFactor’ in MII for deriving actual processing capability used for MBS broadcast reception on non-serving cell e.g. when broadcast scheduling configuration is updated (Adopt TP#4 in annex).

# 3 Conclusion

Request RAN2 to discuss and agree to the following observations and proposals:

Proposal 1. For Rel-18 MII reporting enhancement, MII procedure for frequency of interest determination is amended to consider a condition that at least one of the MBS sessions is from non-serving cell for the concerned frequency (Adopt TP#1 in annex).

Proposal 2. Add “upon change to a PCell providing nonServingCellMII in SIB1” as a trigger to initiate the MII procedure (Adopt TP#2 in annex).

Proposal 3. MII procedure handles set of frequencies of interest for non-serving cell separately from the legacy set of frequencies of interest for serving cell(s) to be reported in MII (Adopt TP#3 in annex).

Observation 1: UE’s actual processing capability available for unicast (i.e., full processing capability minus processing capability used for broadcast reception on non-serving cell) may change based on configuration update on non-serving cell.

Observation 2: Without consideration to broadcast scheduling configuration information in MII signaling, there will be over-estimation on the processing capability used for broadcast reception. Resultantly, UE’s actual processing capability available for unicast is under estimated and UE’s configuration/performance for unicast is lower than achievable.

Proposal 4. RAN2 to consider configuration updates for broadcast reception on non-serving cell for the Rel-18 MII enhancement. UE includes a ‘mbs-NonServingFactor’ in MII for deriving actual processing capability used for MBS broadcast reception on non-serving cell e.g. when broadcast scheduling configuration is updated (Adopt TP#4 in annex).

# 4 Annex (TP to TS38.331)

## [Base document: R2-2306855 RRC Running CR for eMBS]

## 4.1 Text Proposal # 1

#### 5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session or at least one of these MBS sessions is from non-serving cell for the concerned frequency; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies, the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.

## 4.2 Text Proposal # 2

#### 5.9.4.2	Initiation

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon start or stop receiving at least one MBS broadcast service on a non-serving cell, upon change of bandwidth or subcarrier spacing of MBS broadcast reception on a non-serving cell.

…..

## 4.3 Text Proposal # 3

#### 5.9.4.2	Initiation

<Omitted Text>

Upon initiating the procedure, the UE shall:

1>	if SIB21 is provided by the PCell; or

1>	if nonServingCellMII is provided in SIB1 by the PCell;

2>	ensure having a valid version of SIB21 for the PCell, if present;

2>	if the UE did not transmit MBS Interest Indication since last entering RRC_CONNECTED state; or

2>	if since the last time the UE transmitted an MBS Interest Indication, the UE connected to a PCell neither providing SIB21 nor including nonServingCellMII in SIB1:

3>	if the set of MBS broadcast frequencies of interest for MBS broadcast reception on serving cell(s), determined in accordance with 5.9.4.3, is not empty; or

3>	if the set of MBS broadcast frequencies of interest for MBS broadcast reception on non-serving cell(s), determined in accordance with 5.9.4.3, is not empty:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

2>	else:

3>	if the set of MBS broadcast frequencies of interest for MBS broadcast reception on serving cell(s), determined in accordance with 5.9.4.3, is different from mbs-FreqList included in the last transmission of the MBS Interest Indication; or

3> if the set of MBS broadcast frequencies of interest for MBS broadcast reception on non-serving cell(s), determined in accordance with 5.9.4.3, is different from the list of MBS broadcast frequencies of interest for MBS broadcast reception on non-serving cell included in the last transmission of the MBS Interest Indication; or

3>	if at least one of the subcarrier spacing and the bandwidth for MBS broadcast reception on non-serving cell, has changed since the last transmission of the MBS Interest Indication; or

3> if the prioritisation of reception of all indicated MBS broadcast frequencies compared to reception of any of the established unicast bearers and multicast MRBs has changed since the last transmission of the MBS Interest Indication:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

NOTE:	The UE may send MBS Interest Indication even when it is able to receive the MBS services it is interested in, i.e. to avoid that the network allocates a configuration inhibiting MBS broadcast reception.

3>	else if SIB20 is provided for the PCell or for the SCell:

4>	if since the last time the UE transmitted the MBS Interest Indication, the UE connected to a PCell not providing SIB20 and the UE was not provided with SIB20 for an SCell; or

4>	if the set of MBS broadcast services of interest determined in accordance with 5.9.4.4 is different from mbs-ServiceList included in the last transmission of the MBS Interest Indication:

5>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate the transmission of MBSInterestIndication message.

## 4.4 Text Proposal # 4

MBSInterestIndication message

-- ASN1START

-- TAG-MBSINTERESTINDICATION-START

MBSInterestIndication-r17 ::= SEQUENCE {

criticalExtensions            CHOICE {

mbsInterestIndication-r17     MBSInterestIndication-r17-IEs,

criticalExtensionsFuture      SEQUENCE {}

}

}

MBSInterestIndication-r17-IEs ::= SEQUENCE {

mbs-FreqList-r17                  CarrierFreqListMBS-r17              OPTIONAL,

mbs-Priority-r17                  ENUMERATED {true}                   OPTIONAL,

mbs-ServiceList-r17               MBS-ServiceList-r17                 OPTIONAL,

lateNonCriticalExtension          OCTET STRING                        OPTIONAL,

nonCriticalExtension              MBSInterestIndication-v18xy         OPTIONAL

}

MBSInterestIndication-v18xy ::= SEQUENCE {

mbs-NonServingInfo-r18            CarrierFreqListMBS-r18              OPTIONAL,

mbs-NonServingFactor-r18          INTEGER (1…10)                     OPTIONAL,

lateNonCriticalExtension          OCTET STRING                        OPTIONAL,

nonCriticalExtension              SEQUENCE {}                         OPTIONAL

}

-- TAG-MBSINTERESTINDICATION-STOP

-- ASN1STOP

MBSInterestIndication field descriptions

mbs-FreqList List of MBS frequencies on which the UE is receiving or interested to receive MBS broadcast service via a broadcast MRB.

mbs-Priority Indicates whether the UE prioritises MBS broadcast reception above unicast and MBS multicast reception. The field is present (i.e. value true), if the UE prioritises reception of broadcast services, on frequencies indicated in mbs-FreqList, above a reception of any of the unicast bearers and multicast MRBs. Otherwise the field is absent.

mbs-ServiceList List of MBS broadcast services which the UE is receiving or interested to receive.

mbs-NonServingInfo Indicates the information for MBS broadcast reception on the non-serving cell.

mbs-NonServingFactor Indicates the UE’s actual processing capability used for MBS broadcast reception on the non-serving cell as a fraction of the reported processing capability used for MBS broadcast reception on the non-serving cell. Value 1 indicates actual processing capability used is (1/10 * reported processing capability), value 2 indicates (2/10 * reported processing capability) and so on.