3GPP TSG-RAN WG2 Meeting #123	R2-2307614

Toulouse, France, 21 - 25 August 2023

Agenda Item:	7.20.2

Source:	Huawei, HiSilicon

Title:	Two TAs for multi-DCI multi-TRP

Document for:	Discussion and decision

# 1. Introduction

In [1], a WID on MIMO evolution was approved. One of the objectives with RAN2 impact is the following:

Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Both RAN1 and RAN2 have already made a number of agreements on 2 TAs for UL multi-DCI for multi-TRP operation. An email discussion was assigned for discussing issue on 2TA.

In this contribution, we further discuss the RAN2 impacts of 2 TAs for UL mTRP multi-DCI operation based on the email discussion for 2TA and agreement as far.

# 2. Discussion

In Rel-16, in multi-DCI multi-TRP operation, the UE uses the same TA for both TRPs. In Rel-17, Inter-Cell Beam Management (ICBM) was introduced in which the UE is configured with additional PCIs and transmission can switch between the serving cell TRP and a TRP associated with an additional PCI, while using the same TA for UL. Rel-18 will allow multi-DCI multi-TRP operation using TRP associated with an additional PCI as well as using 2 different TAs for different TRPs.

## 2.1 TAG modelling for mTRP

At RAN2#122, RAN2 confirmed that two TAGs for 2 TAs are configured for the same serving cell as shown below:

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

Based on company views from email discussion, several companies think that the 2 TAGs modelling issue should be first discussed before discussion on the UE behaviour upon either TAT expiry. Companies also provided detailed solutions (as shown below) on TAG modelling for 2 TAGs of the SpCell with mTRP operation. We provide further analysis of the solutions.

- Option 1: one is PTAG and the other is STAG.

For example, the TAG associated with type 1 CSS in the SpCell is considered as the PTAG. This option reuses the existing procedures: all transmissions of the other TAG are impacted when the TAT of the PTAG expires but when the TAT of an STAG expires, there is no impact to the transmissions from the other TAGs. On the other hand, the network shall make sure that the TAT of PTAG cannot expire if it would keep scheduling on the TRP associated with the other TAG.

- Option 2: two independent PTAGs

In this option, similarly to the independent handling at TAT expiry for STAG, when the TAT of a PTAG expires, the transmission of the other PTAG would not be impacted, which is aligned with the behaviour confirmed by the RAN1 LS but RAN1 left the decision on TAG modelling up to RAN2.

In general, both options work and have pros/cons as shown in the table below:

| options | Pros | Cons |
| --- | --- | --- |
| Option 1 | Existing procedures can be reused | Resuming MTRP operation after expiry of the TAT associated with the PTAG requires two RACH procedures, one per TRP |
| Option 2 | Resuming MTRP operation after expiry of the TAT associated with the PTAG requires only one RACH procedure | Changes to existing procedures is needed. |

Option 2 gives more flexibility for the gNB to determine for which TAG the network keeps the TAT running when deciding to leave one TAT expire, and provides a faster recovery of MTRP operation. From that perspective, we slightly prefer the option 2.

Two independent PTAGs are used for multi-TA mTRP operation. When the TAT of a PTAG expires, transmissions associated with the other PTAG are not impacted.

## 2.2 Parallel RACH

RAN1 agreed that 2 TA acquisition is done by CFRA triggered by PDCCH order for the serving PCI and for the additional PCI. Currently, parallel RACH in a MAC entity is not supported in order to reduce the UE complexity, i.e. if a new RA is triggered, the UE either stops the ongoing RA procedure and starts a new RA procedure or continues the ongoing RA procedure as captured below [2];

NOTE 1: If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

- This principle could be also applicable to RA in mTRP, i.e. when RA procedures are triggered on both the serving PCI and on the additional PCI for the same serving cell.

In multi-DCI multi-TRP operation, parallel RACH on different TRPs is not supported.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

- Two independent PTAGs are used for multi-TA multi-TRP operation. When the TAT of a PTAG expires, transmissions associated with the other PTAG are not impacted.

In multi-DCI multi-TRP operation, parallel RACH on different TRPs is not supported.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

[2] 3GPP TS 38.321, V17.3.0