3GPP TSG-RAN2 Meeting #123	R2-2307633

# Toulouse, France, Aug 21-25, 2023

Agenda Item:	6.1.1

Work Item:	NR_MBS-Core

Source: 	Qualcomm Incorporated

Title: 	Discussion on and draft reply to SA2 LS on addressing packet loss during multicast MBS delivery

Document for:	Discussion/Decision

# Introduction

SA6 sent LS to SA2 (S2-2306284/S6-231018) including the following:

In SA6 there were concerns that packet loss could happen in multicast MBS delivery due to AF being not aware of the multicast MBS session state in some cases, e.g.: - A talker starts to speak to the group after a silent period with no DL media. The media packet from the talker needs to be delivered via the multicast MBS session. - The multicast MBS session is configured and the group members have joined in advance. After a while, the application layer signalling, e.g., group call request, MapGroupToSessionStream, floor taken needs to be delivered via the multicast MBS session. During the silent period without DL media, the multicast MBS session used for this group communication might become inactive and the joined UEs become IDLE. The new DL packets trigger the multicast MBS session being activated and the IDLE UEs being paged. However, not all the UEs become CONNECTED at the same time. The late CONNECTED UEs will lose the packet as the NG-RAN will start to transmit the packet when the first UE becomes CONNECTED which triggers the shared delivery restoring again.

SA2 discussed the above and responded with following questions to RAN2 (S2-2307982)

1. 	what are the conditions upon which gNB may move the UE involved in an active MBS Session to RRC_INACTIVE (as quoted in clause 16.10.5.2 of TS 38.300) and whether the QoS information (e.g. 5QI, ARP) of the multicast session is taken into account when deciding whether to move the UE to RRC_INACTIVE state or not.

2. 	what is the typical latency of first downlink packet(s) transmission if the UE is in RRC_INACTIVE state?

This document discusses on potential RAN2 response to the above questions.

# Discussion on Question 1: conditions to move MBS active UE to RRC_INACTIVE

Following is from TS 38.300, where moving to RRC_INACTIVE is explicitly captured but moving to RRC_IDLE is not.

#### 16.10.5.2 Configuration A UE can receive data of MBS multicast session only in RRC_CONNECTED state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB may send RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE. When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state. When an MBS multicast session is deactivated, the gNB may move the UE to RRC_IDLE or RRC_INACTIVE state. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC_IDLE or RRC_INACTIVE state when a multicast session has been activated by the CN. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC_INACTIVE state when the session is already activated and the gNB has multicast session data to deliver. Upon reception of the group notification, the UEs reconnect to the network or resume the connection and transition to RRC_CONNECTED state. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in clause 9.2.5. Paging message for group notification contains MBS session ID which is utilized to page all UEs in RRC_IDLE and RRC_INACTIVE states that joined the associated MBS multicast session, i.e., UEs are not paged individually. The UE stops monitoring for group notifications related to a specific multicast session, i.e., stops checking for the MBS session ID in the Paging message, when the UE enters RRC_CONNECTED state. The UE does not monitor for group notifications for these cases, i.e., once this UE leaves this multicast session or the network requests the UE to leave, or the network releases the multicast session. If the UE in RRC_IDLE state that joined an MBS multicast session is camping on the gNB not supporting MBS, the UE may be notified by CN-initiated paging where CN pages each UE individually due to session activation or data availability, as described in clause 9.2.5. If the UE in RRC_INACTIVE state that joined MBS multicast session is camping on the gNB not supporting MBS, the UE may be notified individually by RAN-initiated paging due to session activation or data availability, as described in clause 9.2.5.

Following can be observed from the above text:

gNB ‘may’ move the UE to RRC_INACTIVE state if there is temporarily no data to be sent to the UEs for the multicast session that is active.

Other than the ‘temporary no data’ condition, no multicast specific conditions are specified in RAN2 specs for the state transition. Following text already existed in TS 38.300 about CN assistance which is common for both unicast and multicast.

### 9.2.2 Mobility in RRC_INACTIVE #### 9.2.2.1 Overview … The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes the registration area configured for the UE, the Periodic Registration Update timer, and the UE Identity Index value, and may include the UE specific DRX, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, the Expected UE Behaviour, the UE Radio Capability for Paging, the PEI with Paging Subgrouping assistance information, the NR Paging eDRX Information, the Paging Cause Indication for Voice Service and the Hashed UE Identity Index Value. The UE registration area is taken into account by the NG-RAN node when configuring the RNA. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RNA Update timer. The NG-RAN node takes into account the Expected UE Behaviour to assist the UE RRC state transition decision. The NG-RAN node may use the UE Radio Capability for Paging during RAN Paging. The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE. When sending the XnAP RAN Paging to neighbour NG-RAN node(s), the PEI with Paging Subgrouping assistance information may be included. The NG-RAN node takes into account the NR Paging eDRX Information to configure the RAN Paging when the NR UE is in RRC_INACTIVE. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the NR Paging eDRX Information for RRC_IDLE and for RRC_INACTIVE may be included. The NG-RAN node takes into consideration the Paging Cause Indication for Voice Service to include the Paging Cause in RAN Paging for a UE in RRC_INACTIVE state. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Paging Cause may be included. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Hashed UE Identity Index Value may be included to determine the start point of PTW.

RAN2 specifications do not explicitly capture any other parameters and/or conditions for the network nodes (e.g. whether the QoS information of the multicast session is taken into account) to make the decision of moving the UE with active multicast session to RRC_INACTIVE other than ‘temporary no data’ for multicast and ‘Expected UE Behaviour’ for unicast/multicast.

gNB is not anticipated to proactively move the UE with active MBS session to RRC_IDLE. However, the UE may need to transition to RRC_IDLE e.g. due to RLF. Only in the case of deactivated MBS session, the UE may be moved by gNB to RRC_IDLE (or RRC_INACTIVE). This is in accordance with SA2 understanding indicated in the LS and the procedural text in TS 23.247.

Note that in case the UE moves to RRC_IDLE, e.g. due to RLF, the multicast MRB is released (and a new multicast MRB configuration would be needed when the UE goes back to RRC_CONNECTED), whereas if the UE is moved to RRC_INACTIVE, the multicast MRB(s) are suspended, which can be resumed upon transitioning back to RRC_CONNECTED without needing explicit MRB (re)configuration.

Respond to SA2’s Question 1 with observations 2 and 3 above.

# Discussion on Question 2: typical latency of first DL packet for transition from RRC_INACTIVE

Let’s consider network initiated/triggered transition from RRC_INACTIVE to RRC_CONNECTED, and the example of UE resuming in different gNB. Following call flows are from TS 38.300:

Figure 9.2.2.4.2-1: Network triggered transition from RRC_INACTIVE to RRC_CONNECTED

In the above call flow, step 4 best case is the following (which is the same as UE-triggered transition with UE context retrieval success):

Figure 9.2.2.4.1-1: UE triggered transition from RRC_INACTIVE to RRC_CONNECTED
(UE context retrieval success)

Therefore, the delay to first DL packet can include the following components:

- Paging delay due to DRX

- Delay from paging message reception to initial access

  - RACH procedure

  - Resume request

  - Response from network, resume complete

  - Packet transfer start

One of the main delays in the above list is for paging the UE. If we consider the case of UE being in eDRX, extended RAN paging cycle can be from rf256 (i.e. 2.56s) up to 1024 rf (i.e. over 10 seconds). Assuming the best-case scenario of single paging attempt is received by all UEs, statistically the average delay is half of the configured periodicity (some get 0 delay, others get max delay). However, the worst-case delay can be up to the configured eDRX cycle or even longer.

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

Without eDRX, paging cycle can be configured to be one of the values shown above (from 320ms up to 2.56s). Assuming the lowest value rf32 for paging cycle, max delay to get the paging would be 320ms assuming first paging attempt is successful.

Then initial access (RACH/msg2/msg3 etc.) can typically take ~10-50ms.

Before msg4 is sent by the network, if the UE is resuming in different gNB, context retrieval is needed. This can be typically ~50ms. (0ms if resuming in same gNB.)

After sending resume complete, path switch procedure may need another ~50ms or more.

As discussed above, a multicast UE with active multicast session is not expected to be moved to RRC_IDLE except in error cases. And for the UE in RRC_INACTIVE with an active multicast session, MRB would be typically already configured.

When eDRX is configured for the UE, typical delay for transition from RRC_INACTIVE to RRC_CONNECTED can be several seconds.

Now let’s consider the KPIs for MCPTT requirements from TS 22.179:

The requirements for the KPIs are as follows:

With the lowest paging cycle of 320ms and no eDRX, in worst case of needing two full cycles for paging (i.e. all UEs having just missed the DRX cycle and every UE missing the first page), the total delay of paging and the rest of the signalling delay can be expected to be below the KPI2 limit of 1s (i.e. <1000ms).

Assuming lowest paging cycle of 320ms and no eDRX, even with assumption of two full cycles for successful paging (i.e. all UEs having just missed the DRX cycle and every UE missing the first page), the total delay from paging in RRC_INACTIVE to the transition to RRC_CONNECTED can be expected to be below 1s (i.e. <1000ms).

Based on the above, following is proposed:

Respond to SA2’s Question 2 with observations 4 and 5 above.

Based on the above discussion and proposals, draft LS reply text is shown in the annex.

Send reply LS to SA2 (cc SA6, RAN3) taking the draft in the Annex as baseline.

# Summary and Proposals

As discussed above, we have following observations:

Observation 1.	gNB ‘may’ move the UE to RRC_INACTIVE state if there is temporarily no data to be sent to the UEs for the multicast session that is active.

Observation 2.	RAN2 specifications do not explicitly capture any other parameters and/or conditions for the network nodes (e.g. whether the QoS information of the multicast session is taken into account) to make the decision of moving the UE with active multicast session to RRC_INACTIVE other than ‘temporary no data’ for multicast and ‘Expected UE Behaviour’ for unicast/multicast.

Observation 3.	gNB is not anticipated to proactively move the UE with active MBS session to RRC_IDLE. However, the UE may need to transition to RRC_IDLE e.g. due to RLF. Only in the case of deactivated MBS session, the UE may be moved by gNB to RRC_IDLE (or RRC_INACTIVE). This is in accordance with SA2 understanding indicated in the LS and the procedural text in TS 23.247.

Observation 4.	When eDRX is configured for the UE, typical delay for transition from RRC_INACTIVE to RRC_CONNECTED can be several seconds.

Observation 5.	Assuming lowest paging cycle of 320ms and no eDRX, even with assumption of two full cycles for successful paging (i.e. all UEs having just missed the DRX cycle and every UE missing the first page), the total delay from paging in RRC_INACTIVE to the transition to RRC_CONNECTED can be expected to be below 1s (i.e. <1000ms).

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	Respond to SA2’s Question 1 with observations 2 and 3 above.

Proposal 2.	Respond to SA2’s Question 2 with observations 4 and 5 above.

Proposal 3.	Send reply LS to SA2 (cc SA6, RAN3) taking the draft in the Annex as baseline.

# Annex: Draft Reply LS

3GPP TSG-RAN2 Meeting #123	R2-23xx

# Toulouse, France, Aug 21-25, 2023

Title:	[DRAFT] Reply to LS on addressing packet loss during multicast MBS delivery

Response to:	(R2-2307062/S2-2307982) LS on addressing packet loss during multicast MBS delivery

Release:	Rel-17

Work Item:	5MBS, MCOver5MBS

Source:	RAN2

To:	SA2

Cc:	SA6, RAN3

Contact Person:

Name:			Umesh Phuyal

E-mail Address:	uphuyal@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	None

1. Overall Description:

RAN2 thanks SA2 for the LS on addressing packet loss during multicast MBS delivery.

RAN2 discussed the questions from SA2 and would like to provide the following answers:

Question 1. 	what are the conditions upon which gNB may move the UE involved in an active MBS Session to RRC_INACTIVE (as quoted in clause 16.10.5.2 of TS 38.300) and whether the QoS information (e.g. 5QI, ARP) of the multicast session is taken into account when deciding whether to move the UE to RRC_INACTIVE state or not.

Answer 1: RAN2 specifications do not explicitly capture any other parameters and/or conditions for the network nodes (e.g. whether the QoS information of the multicast session is taken into account) to make the decision of moving the UE with active multicast session to RRC_INACTIVE other than ‘temporary no data’ for multicast and ‘Expected UE Behaviour’ for unicast/multicast.

gNB is not anticipated to proactively move the UE with active MBS session to RRC_IDLE. However, the UE may need to transition to RRC_IDLE e.g. due to RLF. Only in the case of deactivated MBS session, the UE may be moved by gNB to RRC_IDLE (or RRC_INACTIVE). This is in accordance with SA2 understanding indicated in the LS and the procedural text in TS 23.247.

Question 2. 	what is the typical latency of first downlink packet(s) transmission if the UE is in RRC_INACTIVE state?

Answer 2: When eDRX is configured for the UE, typical delay for transition from RRC_INACTIVE to RRC_CONNECTED can be several seconds.

Assuming lowest paging cycle of 320ms and no eDRX, even with assumption of two full cycles for successful paging (i.e. all UEs having just missed the DRX cycle and every UE missing the first page), the total delay from paging in RRC_INACTIVE to the transition to RRC_CONNECTED can be expected to be below 1s (i.e. <1000ms).

2. Actions:

To SA2:

ACTION: 	RAN2 respectfully asks SA2 to take the above answers in account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-RAN2 Meeting #123bis   Oct 9-13, 2023 	Xiamen, China

TSG-RAN2 Meeting #124        Nov 13-17, 2023 	Chicago, USA