3GPP TSG-RAN2 Meeting #123	R2-2307635

# Toulouse, France, Aug 21-25, 2023

Agenda item:	7.8.2

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Measurement and reporting enhancements

Document for:     Discussion/Decision

# Introduction

3GPP studied support of Aerial Vehicles during Rel-15 LTE and documented the extensive performance evaluation results in TR 36.777 [2]. Normative work was also finished in Rel-15 for LTE, where several enhancements were added for measurement and reporting taking UAV characteristics into account.

One of the objectives in the Rel-18 WI on UAV is intended to adapt the LTE solutions to NR while developing NR-specific enhancements as needed. In this contribution, we discuss some remaining issues on the measurement and reporting aspects.

# Height-dependent configuration

## Number of height ranges for height-based SSB-ToMeasure

RAN2#122 agreed the following:

      - Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

In the future when aerial vehicles become more popular and ubiquitous, one can imagine that there could be multiple different layers of aerial highways, and networks may correspondingly optimize and configure the corresponding height dependent parameters. Therefore, for future proofing, we propose the number of height ranges to be 32. We do not see strong need to be conservative on that max number because:

- It is up to the network how many such ranged are configured regardless of the possibility in signalling.

- Being conservative with this only saves a few bits in signalling. (E.g., if we choose 8 instead of 32 for max number, if the network is actually configuring say only 3 such ranges, the difference in total encoded size is expected to be just 2 bits.)

- If in the future we wanted to extend the max size of the list, then it will add much more overhead due to extension markers compared to the 2 bits savings now.

For height-based list of SSB-ToMeasure, number of height ranges is 32.

## Height-based SSB-ToMeasure in RRC_IDLE/RRC_INACTIVE

The previous RAN2 agreement to add height-based SSB-ToMeasure configured in MeasObjectNR for an UAV applies in RRC_CONNECTED mode. However, the UAVs while in flight mode can be in RRC_IDLE/ RRC_INACTIVE as well. (They do not need to be always in RRC_CONNECTED mode since there may not be always active data communication.)

Based on earlier discussions, it is reasonable, beneficial, and straightforward to enable the height-based SSB-ToMeasure for the UAVs for RRC_IDLE/RRC_INACTIVE mode measurements also. This can be done, e.g., by adding the height-based SSB-ToMeasure list to MeasIdleCarrierNR-r16 in MeasIdleConfig, to intraFreqCellReselectionInfo in SIB2 and to InterFreqCarrierFreqInfo in SIB4.

Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis for RRC_IDLE/RRC_INACTIVE measurements.

# Reporting of height, location, and velocity

RAN2#121 agreed the following:

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

Regarding the above FFS, RAN2#122 agreed the following:

5. Whether UE height is included when UAV specific MR is triggered is configurable by the network.

## Range of values for UE height reporting

RAN2#122 agreed: “We will use LTE UEheight”. Note that UE height parameter in LTE is defined as follows (see TS 36.331):

heightUE-r15                      INTEGER (-400..8880)       OPTIONAL

heightUE

Indicates height of the UE in meters relative to the sea level. Value 0 corresponds to sea level (i.e., negative value indicates depth of the UE below sea level). Value -400 corresponds to -400 m, value -399 corresponds to -399 m and so on.

Similarly, the absolute height thresholds for events H1/H2 are configured in two parts as follows:

Absolute value of threshold = Common reference value indicated by heightThreshRef in MeasConfig + Event specific Offset value indicated by hN-ThresholdOffset in ReportConfigEUTRA

heightThreshRef-r15 CHOICE {

release               NULL,

setup                 INTEGER (0..31)

}

heightThreshRef

Reference height threshold for eventH1 and eventH2 in reportConfig. Value 0 refers to -420m, value 1 refers to –120m, and so on until value 30 refers to 8880m. The actual value is height in meters relative to sea level. Value 31 is reserved.

h1-ThresholdOffset-r15             INTEGER (0..300),

h2-ThresholdOffset-r15             INTEGER (0..300),

h1-ThresholdOffset, h2-ThresholdOffset

An offset value to heightThreshRef to obtain the threshold to be used in EUTRA height report triggering condition for event H1 and event H2. The value for h1-ThresholdOffset and h2-ThresholdOffset is expressed in meters such that granularity is 2meters. Value 0 corresponds to offset value 0m, value 1 corresponds to offset value 2m, value 2 correspond to offset value 4m, and so on.

These values were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest).

During the LTE discussions on this topic, it was not anticipated that the UE receiving communication services using 3GPP technologies would go beyond that height. However, in present context, we should be open to expand this UE height range well beyond the top of the Mt. Everest.

For example, the high-altitude platforms (HAPs) are typically deployed between 8km and 50km above ground level (see TR 38.811). While HAPs are neither expected to be UE nor UAV, it is reasonable to assume that the UAVs of the future may be able to go beyond 8880m and still be provided services by 3GPP cellular technologies.

In LTE, values for UE height range for configuration and reporting were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest). The UAVs of the future may be able to go beyond 8880m and still be provided services by 3GPP cellular technologies.

For RRC signalling, the maximum UE height both for configuration and reporting is 20km.

Furthermore, it can be observed that in LTE, once heightThreshRef is set to a value (which is common for all MOs/Meas IDs), the Offset for H1/H2 thresholds can be 0, 2m, 4m… up to 600m (i.e., 2m granularity and 300 codepoints). This is because it was assumed that a network deployed at a certain location would not provide beyond 600m of vertical coverage. Let’s take an example. Assume the network wants to configure H1 threshold at 50m. This is configured by setting the following values:

heightThreshRef-r15 = Integer value 1 (corresponds to value -120m) and h1-ThresholdOffset-r15 = Integer value 85 (corresponds to value 170m) => actual threshold = 50m.

(Alternatively, heightThreshRef-r15 = Integer value 0 (corresponds to value -420m) and h1-ThresholdOffset-r15 = Integer value 285 (corresponds to value 570m) can also signal actual threshold = 50m, which became possible due to the Offset being changed to granularity of 2m but not reducing the signalling range of Offset.)

In such case, the maximum actual offset that can be signalled simultaneously corresponds to h1-ThresholdOffset-r15 = Integer value 300 (corresponds to value 600m) => actual threshold = 480m (since heightThreshRef-r15 = Integer value 1 is common for all configurations).

From the above example, an LTE network cannot configure H1 threshold = 50m and H2 threshold = 500m simultaneously!

The best case of range of thresholds that can be configured spans just 600m when one of the thresholds is exactly at the reference (offset =0), e.g. one threshold 180m (reference = value 2, offset = value 0) and other max threshold 780m (offset = value 300 with the reference = value 2).

The worst case of range of thresholds spans 302m when one of the thresholds is exactly below the next reference codepoint (needing offset = 149), e.g. one threshold 178m (reference = value 1, offset = value 149) then other max threshold would be 480m (offset = value 300 with the reference = value 1).

In LTE, the range of height thresholds for H1/H2 that can be configured simultaneously is 600m in the best case, and 302m in worst case, which is too limited for future scenarios and uses cases.

This is too limiting in present context. So, we propose that the signalling should allow for the network to simultaneously configured height-based event thresholds for the whole range of possible UE heights.

Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights.

## reportOnLeave for H1/H2

Note that in LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). However, in NR, all the event triggers defined thus far have configuration for reportOnLeave by default.

In LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). However, in NR, all the event triggers defined thus far have configuration for reportOnLeave by default.

Similar to all other events in NR, allow reportOnLeave configuration for H1/H2 and all new events.

## Standalone reporting of height, location, velocity

NOTE: Proposal in this subsection was briefly discussed in RAN2#122 but no explicit conclusion was reached. It can be interpreted as already allowed by an agreement from RAN2#121 but is not explicitly clear. So, this proposal is presented again for explicit agreement.

Height measurement alone cannot be sent in LTE without including it with other radio measurements. Similarly, UAV’s location and velocity measurements cannot be sent alone in LTE without including it with other radio measurements.

In some cases, the network may benefit from the ability to configure height, location and/or velocity report alone (e.g. if a large UL grant could not be provided to fit the full RRM report).

Therefore, RAN2#121 previously agreed the following:

When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results)

Since the above “and/or” can be interpreted differently, following is proposed for explicit agreement.

[Clarification to RAN2#121 agreement] UE height, location and/or velocity reporting without being accompanied by RSRP/RSRQ measurement results is supported.

# Summary

In this contribution, we made the following observations:

Observation 1.	In LTE, values for UE height range for configuration and reporting were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest). The UAVs of the future may be able to go beyond 8880m and still be provided services by 3GPP cellular technologies.

Observation 2.	In LTE, the range of height thresholds for H1/H2 that can be configured simultaneously is 600m in the best case, and 302m in worst case, which is too limited for future scenarios and uses cases.

Observation 3.	In LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). However, in NR, all the event triggers defined thus far have configuration for reportOnLeave by default.

Based on the discussion and observations, we propose:

Proposal 1.	For height-based list of SSB-ToMeasure, number of height ranges is 32.

Proposal 2.	Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis for RRC_IDLE/RRC_INACTIVE measurements.

Proposal 3.	For RRC signalling, the maximum UE height both for configuration and reporting is 20km.

Proposal 4.	Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights.

Proposal 5.	Similar to all other events in NR, allow reportOnLeave configuration for H1/H2 and all new events.

Proposal 6.	[Clarification to RAN2#121 agreement] UE height, location and/or velocity reporting without being accompanied by RSRP/RSRQ measurement results is supported.

# References

[1] RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[2] TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles (Release 15), Technical Report