3GPP TSG-RAN WG2 Meeting #123	R2-2307653

Toulouse, France, August 21-25, 2023

Agenda item:	    7.21.3

Source: 		Qualcomm Incorporated

Title: 	UL Coverage Enhancements – User Plane

Document for:	Discussion

# Introduction

In this contribution we discuss some views on Rel-18 Further UL Coverage Enhancements. The WID [1] asks RAN2 to handle the first objective below:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. The detailed objectives of the work item are as follows: - Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

The following was agreed in the last RAN2 meeting [2]:

- MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg3-based SI request - FFS for MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg1-based SI request. - RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases - RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers - The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP - RAN2 to further discusss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure.

An ongoing email discussion is currently underway to discuss the UP open issues, especially, as it relates to fallback [3].

# Supported Fallbacks

Early on in the WI, RAN2 has tried to assess whether “fallbacks” should be supported with the following four cases presented to companies to indicate support [3]:

- Case 1: Fallback from legacy 4-step RA to 4-step RA with Msg1 repetition; ----Not supported - Case 2: Fallback from Msg1 repetition with lower number to Msg1 repetition with higher number; ----FFS, Supported by multiple companies - Case 3: Fallback from 2-step RA to 4-step RA with Msg1 repetition; ----FFS, low priority (not much support) - Case 4: Fallback from CFRA to CBRA with Msg1 repetition. ----FFS, low priority (not much support)

As indicated during the meeting, fallbacks as a tool if agreed can be used to support more than once case seamlessly. In this case as it looks clearer that Case 2 at least would be supported, we would like to propose rediscussing how Case 1 can be supported. The reasoning to support case 1 is clear as both the NW and the UE may prefer that the UE attempts a legacy RA (w/o repetition) at the first RACH attempt since this may have a high likelihood of success, then reserve the {2,4,8} repetitions to fallbacks only for UEs which undergo a RACH failure. This may be the most attractive for the feature from a coverage-utilization/energy standpoint.

Observation 1: There is a great utility for both NW and UE in supporting fallbacks from legacy RA to Msg 1 repetition.

To this end, it is worth discussing if the “to-be-agreed” fallback mechanism can be applied to Case 1.

Proposal 1: RAN2 to discuss if the fallback mechanism to be supported, if any, can be applied to support fallback from legacy RA to Msg 1 repetition (Case 1).

# Modelling of Fallback at MAC

The agreement from last meeting [2]:

RAN2 to further discusss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure.

Fallback from 2-step to 4-step RACH is already supported in the following clause of 38.321:

3> if the Random Access procedure is not completed: 4> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 5> set the RA_TYPE to 4-stepRA; 5> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 5> if the Msg3 buffer is empty: 6> obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer; 5> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 5> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 5> perform the Random Access Resource selection procedure as specified in clause 5.1.2. 4> else: 5> perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a). NOTE: The MSGA transmission includes the transmission of the PRACH Preamble as well as the contents of the MSGA buffer in the PUSCH resource corresponding to the selected PRACH occasion and PREAMBLE_INDEX (see TS 38.213 [6])

In our view, the MAC modelling for fallbacks can use the 2-step to 4-step RACH fallbacks as a starting point. It is useful for RAN2 to then identify which MAC parameters, if any, can be treated differently in the Msg-1 repetition number related fallbacks.

Proposal 2:  Fallback from 2-step RACH to 4-step RACH is taken as a baseline for MAC modelling of Msg-1 repetition- related fallbacks.

# Conclusion

Observation 1: There is a great utility for both NW and UE in supporting fallbacks from legacy RA to Msg 1 repetition.

Proposal 1: RAN2 to discuss if the fallback mechanism to be supported, if any, can be applied to support fallback from legacy RA to Msg 1 repetition (Case 1).

Proposal 2:  Fallback from 2-step RACH to 4-step RACH is taken as a baseline for MAC modelling of Msg-1 repetition- related fallbacks.

# References

[1] RP-221858 Revised WID on Further NR coverage enhancements

[2] RAN2 122 Chair Notes

[3] Summary of [Post122][802][R18CEenh-UP] UP open issues (ZTE)