| 3GPP TSG RAN WG2 Meeting #123 |  | R2-2307656 |
| --- | --- | --- |
| Toulouse, France, August 21st – August 25th, 2023 |  |  |
| Agenda item: | 7.9.4 |  |
| Source: | Fraunhofer IIS, Fraunhofer HHI |  |
| Title: | Throughput Enhancement in U2N Relaying |  |
| Document for: | Discussion and Decision |  |

# Introduction

A new WID for Rel-18 on NR sidelink relay enhancements has been approved at RAN#95 and revised in RAN#98e [1]. Objective 4 of the WI is as follow:

- Specify mechanisms to support the following multi-path scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

Note 3A: The mechanisms to support scenario 1 and scenario 2 are specified based on the assumptions and restrictions agreed in study phase.

Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline.

Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

This document discusses open issues for throughput enhancements in U2N relaying.

# Throughput Enhancements

The RAN WG2 #122 contributions [2] and [3] discussed the topic of throughput/reliability enhancements in multi-path sidelink relaying. When reliability and throughput enhancements are mentioned for multi-path at the same TDoc section, different techniques and concepts are referenced with the majority of contributions targeting reliability enhancements.

For reliability enhancements PDCP duplication is used to send two (or more) copies of the same PDCP packet via multiple paths to the network. These paths can be direct/direct (e.g. MR-DC) or direct/indirect (e.g. Uu/SL). For more than two copies, two or more direct and one indirect or at least one direct and two or more indirect paths can be used. Duplication uses a “split bearer” for traffic configured for duplication, see Figure 1.

PDCP duplication via split bearer can use multiple paths, e.g. at least two indirect or a combination of direct/indirect paths.

Figure 1 – Protocol architecture for MR-DC with 5GC [4]

Throughput enhancements can be achieved by carrier aggregation (CA) of multiple component carriers. The concept of CA is not yet specified for sidelink communication, but is an objectivce of the Rel-18 NR Sidelik WI [5].

Carrier aggregation for sidelink will be specified in Rel-18.

On the Uu interface, carrier aggregation is scheduling data from a radio bearer to multiple component carrier in the MAC. This is not directly possible by using multi-path relaying due to the additional SRAP protocol layer on the indirect path and separated MAC entities for Uu and SL as well as the missing CA feature for SL.

Throughput enhancement by using a ‘split bearer’ concept in multi-path relaying is not supported by the current protocol design.

As mentioned in [3], CA can be used in the Uu interface of the remote UE and the Uu interface of the relay UE. But the case of CA iused for PDCP duplication is not yet supported,.

For the topic of throughput enhancement, there is no further mentioning in [3] to allow the remote UE to use CA with a direct and indirect link, which would need adjustments of the protocol stack:

Currently, CA duplication is performed at PDCP level. However, there is no PDCP in the relay UE. Therefore, in order to support CA duplication over relay UE’s Uu link, RAN2 needs to discuss how to perform the duplication without PDCP. For instance, the SRAP can perform the packet duplication. [3]

Because of the already necessary protocol adjustments in the SRAP for CA/PDCP duplication, RAN 2 should also consider discussing to enable throughput enhancements of a bearer via carrier aggregation, including at least one indirect path.

RAN2 to discuss how to enable throughput enhancements for a single bearer with at least one indirect path for multi-path relaying.

In CA, the activation/deactivation of component carriers can be done using MAC CEs. This could be extended to allow the activation/deactivation of the indirect path for a certain bearer in the same way. Therefore it is necessary to define how the routing of traffic from a single bearer to direct/indirect component carriers is configured.

RAN2 to discuss how the routing of data from a single bearer to associated component carriers is configured in order to allow throughput enhancements for multi-path relaying.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1:	PDCP duplication via split bearer can use multiple paths, e.g. at least two indirect or a combination of direct/indirect paths.

Observation 2:	Carrier aggregation for sidelink will be specified in Rel-18.

Observation 3:	Throughput enhancement by using a ‘split bearer’ concept in multi-path relaying is not supported by the current protocol design.

Proposal 1:	RAN2 to discuss how to enable throughput enhancements for a single bearer with at least one indirect path for multi-path relaying.

Proposal 2:	RAN2 to discuss how the routing of data from a single bearer to associated component carriers is configured in order to allow throughput enhancements for multi-path relaying.

# References

[1] RP-223501 “3GPP WID: NR sidelink relay enhancements”, RAN#98e.

[2] R2-2305945 “Discussion on Multi-path relaying”, Lenovo, RAN2#122

[3] R2-2306313 “Remaining issues on multipath SL relay”, Nokia, RAN2#122

[4] TS 37.340 V17.4.0 – Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; 2023-03

[5] RP-230077, “WID revision: NR sidelink evolution”, OPPO, 3GPP RAN#99, March 2023.