3GPP TSG-RAN WG2 Meeting #123	R2-2307661

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:			7.2.2

Source:			Intel Corporation

Title:		Further Considerations on Sidelink Positioning

Document for:	 	Discussion and decision

# Introduction

At RAN2#122, there was extended discussion on the open issues related to sidelink positioning and the following agreements were made [1]:

Agreements: SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast. Inform SA2 about our agreements on sidelink positioning, with “take into account” action. SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction. Agreements: Anchor UE selection is supported by information about the candidate anchor UEs. At least the following list can be discussed for use in anchor UE selection: 1. UE roles 2. Supported positioning method 3. In coverage or not 4. RSRP 5. LOS/NLOS 6. Location 7. PLMN A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified. RAN2 impact of this information to be determined. FFS which information would be determined statically/dynamically. Agreements: SLPP can support multiple target UEs in the same session when LCS requests. RAN2 will not specify group management for multiple target UEs. RAN2 assumption is that a group ID will be provided from upper layers. FFS how session IDs are managed between multiple UEs.

As can be seen from above, there are still many FFS open issues that need to be resolved, particularly related to the anchor UE selection and reliable transport of SLPP. In this contribution, we continue the discussion on these open issues and present our view.

# Discussion

### 2.1 SLPP Reliable transport

Based on the discussion in the last meeting, it is clear that SLPP needs to support reliable transport over PC5 interface, given that there is no underlying TCP/IP to ensure such reliable transmission. To this end, we think is quite reasonable to simply apply the same principles for the transport layer requirements for reliability as LPP. In particular, a UE implementing LPP over CP supports LPP reliable transport by way of duplicate detection, acknowledgement and retransmissions. At minimum, the same requirement can be applicable for a UE implementing SLPP over PC5 interface as well.

The duplicate detection may rely on use of specific sequence numbers allocated to all SLPP messages and the RX UE may simply discard the detected duplicate message based on the sequence numbers. Each SLPP message may also carry an ACK request and/or an ACK indicator for a specific sequence number. The TX UE may only send the next SLPP message once the previous sequence number has been successfully acknowledged. In case the acknowledgement has not been received, the TX UE may optionally resend the original SLPP message up to 3 times, followed by a timeout period (for each retransmission). If the acknowledgement is still not received, the SLPP session may be aborted.

Proposal 1: The same principles of LPP reliable transport (including duplicate detection, acknowledgement and message retransmissions) shall be used for SLPP.

Proposal 2a: The duplicate detection and discard at the RX UE shall rely on use of specific sequence numbers assigned to all SLPP messages.

Proposal 2b: Each SLPP message may carry an ACK request and/or ACK indicator for a specific sequence number and the TX UE may only send the next SLPP message once the previous sequence number has been successfully ACKed.

Proposal 2c: Up to 3 SLPP retransmissions may be sent by the TX UE and in case no ACK is received, the SLPP session may be aborted.

In addition to reusing the above procedures for SLPP, RAN2 also need to consider SL specific and cast specific aspects. For instance, the above procedures may work as is for unicast, but in case of groupcast, it is not clear if the TX UE needs to keep track of acknowledgements for each member RX UE within the group and perform selective retransmissions to those UEs in a unicast way. Similarly, in case no acknowledgement is received from certain UEs within the group even after successive retransmissions, does the TX UE terminate SLPP session with all group members? Given that we have already agreed that the group member information is transparent to the AS layer, selecting a specific number of acknowledgments received as a threshold may not be desirable, since the number of group members may be quite dynamic. We think that unless such aspects are resolved, the support of reliable transport for SLPP in case of groupcast/broadcast is a distant goal and may not be feasible. It is also worth noting that RAN2 already agreed that group management for SLPP is out of RAN2 scope.

Another aspect related to groupcast operation is related to the minimum communication range (MCR). Given how the PC5 QoS also includes the communication range as a key metric (e.g. for determining sending of ACK/NACK in case of groupcast communication), the impact of range on transmission of ACK from group members also needs to be considered.

Proposal 3: Reliable transport for groupcast SLPP shall be deprioritized for this release.

### 2.2 Aspects related to SLPP Session

With respect to the session-based vs session-less approach discussed in RAN2 in the previous meetings, the following was agreed in RAN2#120 meeting [2]:

Agreement: Sidelink positioning supports a session-based concept in SLPP, in which signalling messages within a session can be associated with one another by the involved UEs. The relationship to upper-layer designs from SA2 can be discussed during normative work. FFS if there is also sessionless operation and what aspects of session-based operation would not be included. Agreement: At least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP sessionless operation can be supported. FFS if sessionless operation can be operated with security.

Agreements:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

TP in R2-2304005 is postponed.

RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)”

RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

Based on the discussion in the previous meetings where most of the focus has been on the session based operation and not much has been discussed related to the session-less case, it is also evident that design of SLPP and session based operation leaves little room for the sessionless operation, since it is not clear how the ranging/SL Positioning layer operates without having the concept of a well-defined session as previously discussed in RAN2. For sessionless concept, the main use case may be related to broadcast/groupcast discussion, i.e. similar to UE based positioning, the UE may not need the interaction with location server, and therefore no session is needed. However, it has been agreed that group management in such scenario is not in RAN2 scope. It is also worth noting that if group positioning may end being supported by way of session based operation, the need for session-less operation is even less motivated. Therefore, we think RAN2 can deprioritize the support of broadcast/groupcast until after the session-based operation is matured and the associated security aspects are clear.

Proposal 4: Postpone the discussion on support of the sessionless operation until the session-based operation is matured and the associated security aspects are clear.

### 2.2 Aspects related to Discovery

With respect to the discovery procedure, there was some discussion on this aspect in the last meeting and the following was agreed:

Agreements:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and which messages.

It was agreed that some information for discovery and candidate selection of SL positioning (anchor) UEs may be included as part of the discovery messages, but what information is included was left FFS. At least the UE role may be included as per SA2 decision, but we need to discuss what information may be included to aid the discovery procedure from RAN2 perspective.

As detailed in TS 23.304, the legacy discovery mechanism relies on exchange of discovery messages (using both Model A and Model B) and may not contain AS layer information specifically related to sidelink positioning. We certainly see benefit of including some SL positioning related information within the discovery message, e.g. related to supported SL positioning capability or supported positioning methods, etc. The PC5 discovery procedure may be enhanced to include specific information within the discovery message that may aid the support of sidelink positioning. At least the following may be considered:

a)	SL positioning capabilities, including supported positioning methods by the anchor UE

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

For the case of model A, the target UE may broadcast the discovery announcement message which may include the SL positioning/ranging service code as well as the above information, which implies its interest in finding a suitable anchor UE. An anchor UE that supports the positioning method(s) and other information indicated by the target UE may respond by setting up a direct PC5 connection with the target UE to aid in the positioning session.

In case of model B, the anchor UEs that have been authorized and provisioned by the network to act as the anchor UE for sidelink positioning may periodically broadcast a solicitation message, which can include the service code for SL positioning/ranging as well as the above information, i.e. supported positioning methods, ability to perform location estimate, etc. The target UE which is interested in SL positioning can then respond to the discovery message and set up a PC5 connection with the anchor UE.

Observation 1: With respect to the discovery procedure for the session based approach, AS layer parameters related to sidelink positioning are useful to include as part of the discovery message (e.g. supported positioning methods).

Proposal 5: In addition to the UE role, RAN2 to discuss which of the following information shall be included as part of the discovery messages (for both model A and B):

a)	SL positioning capabilities, including supported positioning methods by the anchor UE(s)

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

### 2.4 Aspects related to Anchor UE (Re-)selection

The process of selection of anchor UE for sidelink positioning was a point of discussion in the previous meetings and it was RAN2 assumption that at least part of the selection procedures shall be in RAN2 scope. Moreover, RAN2 also captured several criteria which may be applicable for anchor UE (re)selection. In this context, we think there are two different scenarios to consider.

Firstly, when the SL positioning procedure is initiated, the target UE may need to discovery suitable candidate anchor UEs in its vicinity. Once the discovery is performed and the target UE is able to discover such UEs, the next logical step is selection of one or multiple anchor UEs for the sidelink positioning to proceed. Note that unlike the discovery procedure, this step needs to include the LMF and/or the positioning server UE. To enable this, at least the following options may be considered:

- Option 1: The target UE may provide the list of discovered candidate anchor UEs to the LMF and/or the positioning server UE together with other information, e.g. anchor UE ID, the supported positioning methods, the location calculation capability and the quality of the signal from the anchor UE, etc. This information may be obtained as part of the discovery procedure (as discussed in section 2.3 above and can be provided by the upper layer. Based on this information, the LMF and/or server UE may perform selection of the suitable anchor UE or UEs for the target UE for further selection.

- Option 2: Based on target UE location and/or service request (e.g. QoS, requested positioning method, etc.), the LMF/server UE may perform selection of the suitable anchor UE or UEs for the target UE for further selection based on preconfigured anchor UE lists (i.e. without target UE explicitly providing a list of candidate anchor UEs beforehand, in contrast with Option 1). To enable this, the anchor UEs may directly provide their capability and other pertinent information to the LMF for pre-configuration.

- Option 3: The LMF/server UE may provide a list of candidate anchor UEs to the target UE and the target UE shall select the anchor UE within the list. The target UE may be configured (or preconfigured in case of OOC) by the LMF with selection criteria to use for selection of anchor UE from the discovered candidate anchor UEs (e.g. the quality of the signal from the anchor UE, supported positioning methods, calculation capability, etc.). Based on these criteria, the AS layer shall select and inform the upper layer of the selected anchor UE.

The three options discussed above may broadly fall under either the LMF/Server UE based or LMF/Server UE assisted anchor UE selection methods. It can be seen in (section 6.20 of) [3] that SA2 has already captured the signaling steps based on Option 1 above, i.e. the target UE provides the list of candidate anchor UEs to the LMF (Step 8) and LMF then proceeds with SL positioning procedures with the selected UEs. Therefore, it would be good to discuss and confirm this in RAN2.

Proposal 6: For initial anchor UE selection after discovery, RAN2 is proposed to confirm that LMF/server UE based approach is used, where LMF/server UE obtains information about candidate anchor UEs selected from the target UE.

Moreover, SA2 also needs input from RAN2 on whether there is a need for the LMF to request the capabilities of the candidate UEs (steps 10 and 11) from the target UE. In our view, this information is required by the LMF/server UE in order to support the location request. So, the only real question is whether the target UE provides this information to the LMF directly when providing this list of candidate UEs (step 8), or whether the LMF explicitly requests this information subsequently (steps 10 and 11). We think both options can work, but including this information in the SL-MO-LR request itself (in step 8) reduces the signaling overhead somewhat and is preferrable.

Proposal 7:  From RAN2 perspective, target UE may include information about capabilities of the candidate UEs in the location request to the LMF/server UE.

The other case is when the positioning procedure is ongoing and anchor UE may need to be reselected by the target UE. There can be several reasons for this occurrence, the most common being deteriorating channel conditions between the target and anchor UE. Given the nature of sidelink channel and target and anchor UE being mobile, the support of such reselection seems warranted.

Observation 2: Due to the nature of sidelink channel and the mobile nature of the anchor and the target UE, the anchor UE may need to be reselected during an ongoing SL positioning procedure.

For this scenario, the fundamental question to consider is whether the positioning procedure may still continue after the target UE reselects to a different anchor UE, or whether a new positioning session needs to be initiated afterwards. From signalling overhead and latency perspective, it seems useful to continue the ongoing positioning procedure. However, considering the anchor node is changed, the original measurement results are no longer going to be useful. Therefore, the UE may need to report the need for anchor UE reselection to the LMF. It can be based on the error reporting methodology as in LPP or it can be defined within a new message.

Furthermore, RAN2 needs to discuss the triggers for when the anchor UE may need to be reselected. For this case, we can take inspiration from the relay reselection procedure and the following triggers may be considered:

- If there exists a unicast link between the target UE and the anchor UE, the SL link quality (e.g., based on SL-RSRP) can be used to determine whether the anchor UE is considered valid.

- The anchor UE can provide indication to the target UE based on its own criteria, e.g., in case it has to perform power saving and cannot continue with SL-PRS transmission or location estimation.

- Anchor UE can provide a bit indication on whether it can still support the required QoS for SL positioning method/session

Out of the above options, we think the most relevant one to consider is based on link quality. Since RAN1 is discussing use of both shared and dedicated resource pools for SL-PRS, it is likely that similar channel characteristics apply for data and SL-PRS transmissions. Therefore, at least SL link quality based trigger for anchor UE reselection is used. Note that this trigger has to be checked dynamically since the link quality can change quite dynamically due to SL channel. The other factors as captured in the meeting minutes do not seem essential to be captured in our view, especially if the determination of the anchor UE is done by the LMF. Note that the UE role and supported positioning methods would anyway be known by the UE given that they may be included as part of the discovery message (as per discussion in section 2.3 above).

Moreover, we do not think there is a need to specify the exact procedure to be followed for anchor UE reselection; instead, the link quality (RSRP) and any other criteria which are agreed can be captured in the stage-2 specification and how the LMF/UE determine the new anchor UE need not be specified.

Proposal 8:  RAN2 supports the procedure to allows the UE to report the need for anchor UE reselection to the LMF.

Proposal 9:  For anchor UE reselection during an ongoing SL positioning procedure, at least SL link quality based (SL-PRSRP) shall be considered.

Proposal 10:  The link quality (SL-RSRP) and any other criteria for anchor UE reselection can be captured in the stage 2 specification and there is no need to capture the details of how the LMF/UE determine the anchor UE (i.e. in stage 3).

# Conclusion

This contribution discusses the open issues related to sidelink positioning and makes the following observations and proposals:

Observation 1: With respect to the discovery procedure for the session based approach, AS layer parameters related to sidelink positioning are useful to include as part of the discovery message (e.g. supported positioning methods).

Observation 2: Due to the nature of sidelink channel and the mobile nature of the anchor and the target UE, the anchor UE may need to be reselected during an ongoing SL positioning procedure.

Proposal 1: The same principles of LPP reliable transport (including duplicate detection, acknowledgement and message retransmissions) shall be used for SLPP.

Proposal 2a: The duplicate detection and discard at the RX UE shall rely on use of specific sequence numbers assigned to all SLPP messages.

Proposal 2b: Each SLPP message may carry an ACK request and/or ACK indicator for a specific sequence number and the TX UE may only send the next SLPP message once the previous sequence number has been successfully ACKed.

Proposal 2c: Up to 3 SLPP retransmissions may be sent by the TX UE and in case no ACK is received, the SLPP session may be aborted.

Proposal 3: Reliable transport for groupcast SLPP shall be deprioritized for this release.

Proposal 4: Postpone the discussion on support of the sessionless operation until the session-based operation is matured and the associated security aspects are clear.

Proposal 5: In addition to the UE role, RAN2 to discuss which of the following information shall be included as part of the discovery messages (for both model A and B):

a)	SL positioning capabilities, including supported positioning methods by the anchor UE(s)

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

Proposal 6: For initial anchor UE selection after discovery, RAN2 is proposed to confirm that LMF/server UE based approach is used, where LMF/server UE obtains information about candidate anchor UEs selected from the target UE.

Proposal 7:  From RAN2 perspective, target UE may include information about capabilities of the candidate UEs in the location request to the LMF/server UE.

Proposal 8:  RAN2 supports the procedure to allows the UE to report the need for anchor UE reselection to the LMF.

Proposal 9:  For anchor UE reselection during an ongoing SL positioning procedure, at least SL link quality based (SL-PRSRP) shall be considered.

Proposal 10:  The link quality (SL-RSRP) and any other criteria for anchor UE reselection can be captured in the stage 2 specification and there is no need to capture the details of how the LMF/UE determine the anchor UE (i.e. in stage 3).

# References

| [1] | RAN2#122 meeting, Chairman Notes. |
| --- | --- |
| [2] | RAN2#120 meeting, Chairman Notes. |
| [3] | TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2 (Rel-18). |