3GPP TSG-RAN WG2 Meeting #123	R2-2yyyyyy

Toulouse, France, 21st - 25th August, 2023

Agenda Item		:	7.4.2.2

Source		:	LG Electronics Inc.

Title		:	Discussion on LTM RRC configuration

Document for		:	Discussion and Decision

1.	Introduction

In this paper, we discuss the following open issues on LTM RRC configuration:

- Whether separate configurations are necessary or not

- How the UE handles the radio bearer (RB) configuration during LTM cell switch

2.	Discussion

2.1.	Whether separate configurations are necessary or not

In RAN2#122 meeting, the following is agreed [1]:

The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

Based on the above agreement, the set of L1 RS configuration for measuring LTM candidate cells can be located in either: i) outside of both the candidate cell configuration and the reference configuration (i.e. as described in the current RRC running CR); or ii) inside of the reference configuration.

If the set of L1 RS configuration for measuring LTM candidate cells is located in outside of both the candidate cell configuration and the reference configuration (i.e. the case i), RAN2 needs to address how the UE applies the L1 RS configuration into the LTM complete configuration (i.e. target cell configuration) during LTM execution. To address the issue, for example, the UE generates the LTM complete configuration of the target cell and then perform an additional action of applying the L1 RS configuration to the LTM complete configuration during LTM execution, thus increasing mobility latency. Further, specification effort for additional operations is required. We don’t see any benefit to providing the set of L1 RS configuration for measuring LTM candidate cells separately from both the candidate cell configuration and the reference configuration.

Observation 1. There may be no benefit to providing the set of L1 RS configuration for measuring LTM candidate cells separately from both the candidate cell configuration and the reference configuration.

Otherwise, i.e., if the set of L1 RS configuration for measuring LTM candidate cells is located in the reference configuration (i.e., the case ii), additional latency due to separate configuration handling does not occur. We believe that the reason behind the above agreement is that the set of L1 RS configuration for measuring LTM candidate cells is common across all LTM candidate cells and the purpose of the reference configuration is to reduce signaling redundancy by including information common across all candidate cell configurations. In our view, the set of L1 RS configuration for measuring LTM candidate cells is the common information across all LTM candidate cells that should be included in the reference configuration.

Proposal 1. The set of L1 RS configuration for measuring candidate cells is included in the reference configuration.

In this paper, separate configuration is defined as the configuration that requires additional operation to create the target configuration during LTM cell switch, e.g., the L1 RS configuration in LTM-Config as captured in the current RRC running CR, which is provided separately from the reference and candidate cell configurations.

Observation 2. A separate configuration means the configuration that requires additional operation to create the target configuration during LTM cell switch, e.g., the L1 RS configuration in LTM-Config as captured in the current RRC running CR, which is provided separately from the reference and candidate cell configurations.

In RAN2#121 meeting, the following is agreed [2]:

To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

To implement the above agreement, in the current RRC running CR, the field ltm-ServingCellNoResetID is provided separately from the candidate cell configuration and the reference configuration and the field ltm-NoResetID is provided in the candidate cell configuration. In principle, the UE performs L2 reset during LTM execution if ltm-ServingCellNoResetID is not equal to ltm-NoResetID, the UE does not perform L2 reset during LTM execution otherwise. That is, ltm-ServingCellNoResetID and ltm-NoResetID are identifiers indicating to which set (i.e., set of cells in the above agreement) the current serving cell and the candidate cell belong, respectively.

For the first LTM execution, in order for the UE to know which cell set the current serving cell belongs, the field ltm-ServingCellNoResetID should be provided separately from both the reference configuration and the candidate cell configuration because it is up to UE implementation to determine when to interpret the reference configuration and the candidate cell configuration.

Observation 3. For the first LTM execution, it makes sense that the field ltm-ServingCellNoResetID described in the current RRC running CR is provided upon the initial LTM configuration in order for the UE to know which cell set the current serving cell belongs.

In our view, the field ltm-ServingCellNoResetID in the initial LTM configuration is just a state variable to be stored at the UE upon receiving the initial LTM configuration, not a configuration to be applied during LTM cell switch. Therefore, it is straightforward that the UE stores ltm-ServingCellNoResetID as a UE variable upon receiving the initial LTM configuration. Then, for every LTM execution, the UE replaces the UE variable storing ltm-ServingCellNoResetID with ltm-NoResetID included in the LTM target configuration.

Proposal 2. Upon receiving the initial LTM configuration, the UE stores ltm-ServingCellNoResetID as a UE variable.

Proposal 3. For every LTM execution, the UE replaces the UE variable storing ltm-ServingCellNoResetID with ltm-NoResetID included in the LTM target configuration.

The text proposal of Proposal 3 and Proposal 4 is given below.

Proposal 4. RAN2 adopt a TP given below.

#### 5.3.5.x LTM configuration and execution ##### 5.3.5.x.1 General …skipped… 1> if the LTM-Config includes ltm-ServingCellNoResetID: 2> consider the received ltm-ServingCellNoResetID value as the ltm-ServingCellNoResetID value associated with current serving cell for this cell group; 2> store ltm-ServingCellNoResetID in VarLTM-UE-Config …skipped…

##### 5.3.5.x.6 LTM cell switch execution …skipped… 1> if the value of field ltm-NoResetID contained within the LTM-Candidate IE related to the LTM candidate cell configuration identity as received by lower layers is equal to the value of ltm-ServingCellNoResetID in VarLTM-UE-Config: 2> continue using the current RLC entity in the LTM candidate cell configuration indicated by lower layers; 2> replace the value of ltm-ServingCellNoResetID in VarLTM-UE-Config with the value received within ltm-NoResetID; …skipped…

#### – VarLTM-UE-Config The IE VarLTM-UE-Config is used to store the generated UE configuration related to the received LTM candidate cell configurations. VarLTM-UE-Config UE variable -- ASN1START -- TAG-VARLTM-CONFIG-START VarLTM-UE-Config-r18-IEs ::= SEQUENCE { ue-ltm-ConfigCandidateList-r18 UE-LTM-ConfigCandidateList-r18 ltm-ServingCellNoResetID-r18 LTM-ServingCellNoResetID-r18 } UE-LTM-ConfigCandidateList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF UE-LTM-Candidate-r18 UE-LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ue-LTM-Config-r18 OCTET STRING (CONTAINING RRCReconfiguration) } -- TAG-VARLTM-CONFIG-STOP -- ASN1STOP

Based on above, we observe that the field ltm-ServingCellNoResetID in the initial LTM configuration is just a state variable to be stored at the UE upon receiving the initial LTM configuration, not a configuration to be applied during LTM cell switch. This implies that the field ltm-ServingCellNoResetID is not a separate configuration that is provided separately from the reference configuration and candidate cell configuration for application during LTM cell switch.

Observation 4. With Proposals 2, 3, and 4, the field ltm-ServingCellNoResetID is not a separate configuration.

As mentioned above, a separate configuration can incur additional latency by introducing e.g. additional action of applying the separate configuration to the LTM complete configuration. Therefore, the LTM complete configuration should be designed to include all necessary fields except for the first LTM execution.

Proposal 5. The LTM complete configuration includes all necessary fields for the LTM execution. No separate configuration is needed.

2.2.	Radio bearer (RB) configuration handling during LTM cell switch

Since the LTM configuration (i.e., the reference configuration or the candidate cell configuration) is provided in the form of RRCReconfiguration message, it seems that the configuration other than the cell group configuration (e.g., radioBearerConfig, radioBearerConfig2, etc.) may be provided by the LTM configuration. If the RB configuration is included in the LTM configuration (e.g., the reference configuration), misalignment between the UE and the network in terms of RB configuration can occur.

Figure 1. Example of RB configuration handling during LTM switch. Alt 1: The UE applies the target RB configuration without keeping the source RB configuration. Alt 2: The UE applies the target RB configuration with keeping the source RB configuration.

Consider the example described in Figure 1, where the UE configured with DRB #1/#2/#3 receives the LTM cell switch command including DRB configuration #1/#2. In this case, the UE can choose one of the following two alternatives:

- Alt 1: The UE applies the target RB configuration without keeping the source RB configuration;

- Alt 2: The UE applies the target RB configuration with keeping the source RB configuration.

In our view, the configuration for DRB #3 may be omitted from the LTM target configuration because the source cell triggers the LTM cell switch before the LTM configuration is updated to reflect DRB #3. Further, in legacy principle, the RB configuration of UE can only be released by the network, which implies that the UE should not release the RB configuration on its own. Therefore, Alt 1 is not appropriate for RB configuration handling during LTM cell switch.

To resolve the misalignment between the UE and the network, the RB configuration for each candidate cell should be updated to reflect the change whenever the current RB configuration is changed, i.e., the RRC reconfiguration procedure is required for updating LTM configuration. This can result in frequent RRC signaling. Further, the serving cell should defer triggering the LTM cell switch until the LTM configuration is updated if the LTM configuration needs to be update due to change of RB configuration. In summary, inclusion of RB configuration into the LTM configuration can incur signaling overhead and additional latency.

In order to address the misalignment problem without increasing signaling overhead and latency, we suggest that the RB configuration is optionally provided by the candidate cell configuration. Then, the UE keeps the source RB configuration during LTM cell switch as in legacy mobility. If there exist RBs in the source configuration those do not overlap with target RB configuration, then the RBs are not released and managed by the target cell after LTM cell switch. To this end, the network should ensure that misalignment does not occur between the network and the UE by exchanging RB configuration between the serving cell and each candidate cell whenever necessary.

Proposal 6. RAN2 confirms that the RB configuration (i.e., radioBearerConfig or radioBearerConfig2) is optionally provided by the candidate cell configuration.

Proposal 7. The network ensures that misalignment does not occur between the network and the UE by exchanging RB configuration between the serving cell and each candidate cell whenever necessary, i.e., UE autonomous RB release is not needed. LS to RAN3 is needed.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Observation 1. There may be no benefit to providing the set of L1 RS configuration for measuring LTM candidate cells separately from both the candidate cell configuration and the reference configuration.

Proposal 1. The set of L1 RS configuration for measuring candidate cells is included in the reference configuration.

Observation 2. A separate configuration means the configuration that requires additional operation to create the target configuration during LTM cell switch, e.g., the L1 RS configuration in LTM-Config as captured in the current RRC running CR, which is provided separately from the reference and candidate cell configurations.

Observation 3. For the first LTM execution, it makes sense that the field ltm-ServingCellNoResetID described in the current RRC running CR is provided upon the initial LTM configuration in order for the UE to know which cell set the current serving cell belongs.

Proposal 2. Upon receiving the initial LTM configuration, the UE stores ltm-ServingCellNoResetID as a UE variable.

Proposal 3. For every LTM execution, the UE replaces the UE variable storing ltm-ServingCellNoResetID with ltm-NoResetID included in the LTM target configuration.

Proposal 4. RAN2 adopt a TP given below.

Observation 4. With Proposals 2, 3, and 4, the field ltm-ServingCellNoResetID is not a separate configuration.

Proposal 5. The LTM complete configuration includes all necessary fields for the LTM execution. No separate configuration is needed.

Proposal 6. RAN2 confirms that the RB configuration (i.e., radioBearerConfig or radioBearerConfig2) is optionally provided by the candidate cell configuration.

Proposal 7. The network ensures that misalignment does not occur between the network and the UE by exchanging RB configuration between the serving cell and each candidate cell whenever necessary, i.e., UE autonomous RB release is not needed. LS to RAN3 is needed.

4.	References

[1] Chair Notes, RAN2 Chairman (MediaTek), 3GPP TSG RAN2 Meeting #122, May, 2023

[2] Chair Notes, RAN2 Chairman (MediaTek), 3GPP TSG RAN2 Meeting #121, February, 2023