3GPP TSG-RAN WG2 Meeting #123											    R2-2307675

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on the reporting signaling for shared MBS capability

Agenda Item:	7.11.3

Document for:	Discussion and Decision

# Introduction

According to the RAN2 discussion for the UE capability sharing between broadcast MBS and unicast, RAN2 made the following agreements to allow the UE to report assistance information to the gNB providing the unicast services:

RAN2#122 meeting agreements [1]: - When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. - FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII)

According to the email discussion for the MBS running RRC CR [2] capturing the above agreements, some companies consider that there could be cases that the UE may report the capability assistance information without getting all the assistance information (i.e. frequency, bandwidth and SCS). In this contribution, we provide our understandings on the assistance information reported for the shared MBS capability, and the triggering condition for reporting the shared MBS capability.

# Discussion

## Reporting signalling

1> if nonServingCellMII is included in SIB1 for the PCell; and 1> if the set of MBS frequencies for MBS broadcast reception on non-serving cells, determined in accordance with 5.9.4.3, is not empty: 2> include the carrierFreqMBS, carrierBandwidth and subcarrierSpacing for MBS broadcast reception on the non-serving cell; Editor’s note: FFS whether there are cases where this information is not available at the UE and what happens then.

As quoted above, the assistance information for shared MBS capability includes “carrierFreqMBS, carrierBandwidth and subcarrierSpacing for MBS broadcast reception”. From our understanding, the carrierFreqMBS can be obtained from either SIB21 or the CFR configuration of SIB20. However, the MBS frequency information provided by SIB21 for serving continuity (e.g. cell reselection) is the CD-SSB frequency of MBS cell, which may not be the central frequency of the MBS CFR, as the frequency range/bandwidth of the MBS CFR can be larger than or equal to the initial BWP. Then reporting only the carrierFreqMBS provided by SIB21 does not reflect the UE’s processing capability used for broadcast MBS reception. Thus, we consider that carrierFreqMBS, carrierBandwidth and subcarrierSpacing for MBS broadcast reception shall always be reported together.

Observation: The UE should not report only the frequency provided by SIB21, because the MBS frequency included SIB21 used for cell reselection is the CD-SSB frequency of MBS cell, which may not be the (central) frequency of the MBS CFR.

Proposal 1: Frequency, bandwidth and SCS are reported always together.

According to the RRC running CR [2] of MBS as quoted above, the carrierFreqMBS could have several different interpretations:

- Interpretation 1: The central frequency of MBS CFR

- Interpretation 2: The CD-SSB frequency of the initial BWP

- Interpretation 3: absoluteFrequencyPointA

Since the bandwidth of the MBS assistance information is the number of PRBs used for CFR, reporting “the CD-SSB frequency of the initial BWP” or “absoluteFrequencyPointA” is not able to locate the frequency range used for the MBS reception. As such, we think that the carrierFreqMBS should be the central frequency of MBS CFR, and the carrierBandwidth should be the number of PRBs configured for CFR. Furthermore, as subCarrierSpacing is always reported together with carrierBandwidth, the gNB can know the exact bandwidth (i.e. MHz) by using the number of PRBs configured for CFR and the subCarrierSpacing together.

Proposal 2: The reported frequency is the central frequency of CFR.

Proposal 3: The reported bandwidth is the number of PRBs of CFR.

## Configuration update

#### 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon start or stop receiving at least one MBS broadcast service on a non-serving cell, upon change of bandwidth or subcarrier spacing of MBS broadcast reception on a non-serving cell.

As quoted above from the running RRC CR of MBS, the UE could trigger the MII reporting “upon change of bandwidth or subcarrier spacing of MBS broadcast reception on a non-serving cell”. According to our understanding, it is possible that the network reconfigures any of the central frequency or the bandwidth or the SCS of the MBS CFR. Then the UE should be able to provide the updated configuration to the gNB.

Proposal 4: When the frequency or the bandwidth or the SCS of CFR changes, the UE triggers the MII reporting.

# Conclusion

According to the above analysis, we have the following observations and proposals.

Observation: The UE should not report only the frequency provided by SIB21, because the MBS frequency included SIB21 used for cell reselection is the CD-SSB frequency of MBS cell, which may not be the (central) frequency of the MBS CFR.

Proposal 1: Frequency, bandwidth and SCS are reported always together.

Proposal 2: The reported frequency is the central frequency of CFR.

Proposal 3: The reported bandwidth is the number of PRBs of CFR.

Proposal 4: When the frequency or the bandwidth or the SCS of CFR changes, the UE triggers the MII reporting.

# Reference

[1] RAN2#122 meeting minutes.

[2] R2-2306855, Huawei, HiSilicon, “RRC running CR for eMBS”.