3GPP TSG RAN WG2 Meeting #123           	      	            	                            R2- 2307684

Toulouse, FR, 21st – 25th Aug, 2023

Agenda Item:	7.16.2.1

Source:	Intel Corporation

Title:	AI/ML locality and capability: RAN2 impact

Document for:	Discussion and Decision

# Introduction

In this contribution, we continue discuss RAN2 impact of AI/ML algorithm locality, and AI/ML related capabilities.

# Discussion

## AI/ML Algorithm Locality: RAN2 impact

#### Model Update and Locality

As discussed in RAN1, the AI/ML model for one sub-use case can be site/scenario/configuration-specific. It is also captured in Section 6.2.2, Section 6.3.2 and Section 6.4.2 in TP 38.843 [1], for example, in CSI compression use case, the performance could be degraded if the model is trained with deployment scenario #A and applied for inference with a different deployment scenario #B.

When one UE moves from one cell to another, due to the change of configuration or environment, it is possible that the UE needs to require a new model since the performance of existing model could be degraded when it is applied to another deployment scenario (for scenario specific model)/cell (for cell specific model). On the other hand, if the target cell share similar environment/configuration as the source cell, the UE may still use the existing model. However, it is difficult for the UE to know whether its handover target cell have the same or different configuration/deployment scenario with the current cell.

Even though a generalized model can be trained for general purpose [1], the performance of the generalized model may be worse than the site/scenario/configuration specific models due to lack of uniqueness. It is possible to update the model performance by retraining the model based on a fine-tuned dataset subject to a new deployment scenario, according to Section 6.2.2 in TP 38.843 [1]. However, model update/retraining is time-consuming and could take much longer time after the successful handover of a UE. Moreover, it was also agreed in RAN2 #121bis-e meeting that only offline training is considered.

- R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

Hence, the UE cannot consider to re-train the existing model “online” to accommodate the deployment scenario/configuration of the target cell after its handover. Switching to a new model is needed if performance is impacted. Furthermore, the source cell may also have some prior knowledge about the deployment scenario of the neighbouring cell, e.g. based on prior performance feedback. Based on this prior knowledge, the source cell may be able to let UE be prepared for model update after handover.

There are also other cases where model update may be needed, for example, the UE is handed-over to another NG-RAN node and model update is needed due to lack of interoperability (i.e. model transfer/sharing between NG-RAN nodes are not supported based on RAN3 discussion).

Proposal 1: Considering model locality, Model update may be needed when 1) UE is handed-over to a new target cell which leads to performance down-gradation 2) prior knowledge about deployment scenario of target handover cell 3) handover to another NG-RAN node (e.g., another vendor).

For NW-sided model, it is easier to update a model based on the changed environment or cell, since the model transfer is implemented within NW side. It is up to network model management decision.

As discussed in the companion contribution [3], management entity can either located at NW side or UE side. However, when management entity is located at UE side, the decision making is still guided/triggered by event based on NW configuration. In conclusion, model update for UE-sided and two-sided model is under network control. The network should be able to trigger model update any time it wants.

For UE-sided model, if UE stores multiple models in its own memory, i.e. model transfer from NW is not needed, the UE may decide to switch models according to model management rules. If there’s no other models are available at the UE side and model transfer/delivery is needed, to reduce the overhead caused by model transfer/delivery, the existing model associated to the source NG-RAN node can be reused, until there’s a performance down-gradation.

For two-sided model, the paring model is used at UE side and NW side. When there’s an update at the NW side, the UE should also perform model update correspondingly.

Proposal 2: After UE’s handover, considering model locality and complexity of model transfer/delivery, based on NW control, the UE can keep using the existing model from source cell or update the model according to management decision, with the understanding that the network can always request/provide a model update at the UE side.

#### UE speed

According to the observations captured in RAN1 #113 meeting, UE speed may impact the performance of one AI/ML model. For example, in CSI prediction sub-use case, the performance of AI/ML based solution may vary based on UE speed, and the generalization performance may also degrade if the model is not trained with the dataset subject to a certain UE speed.

Observation For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed: - For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain. - For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed. - For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed. Observation For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, - For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others: - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 4 sources [Xiaomi, CATT, Interdigital, Spreadtrum] observe a generalized performance of less than -2% degradation. - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if UE speed#B is 10km/h and UE speed#A is either 60km/h or 120km/h, 8 sources [Xiaomi, Samsung, Interdigital, Fujitsu, ZTE, ETRI, vivo, Huawei] observe that moderate/significant performance degradations are suffered: - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-2.7% loss), 1 source [Samsung] observes significant degradation (-53%~-61% loss). - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-3% loss), 8 sources [Xiaomi, Interdigital, Fujitsu, vivo, ZTE, Huawei, ETRI, Spreadtrum] observe significant degradation (-6%~-45.6% loss). - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source [ZTE] observes moderate degradation (-3% loss), 7 sources [Samsung, Xiaomi, Fujitsu, ETRI, ZTE, vivo, Spreadtrum] observe significant degradation (-7.8%~-52% loss). - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source [ZTE] observes moderate degradation (-3.4% loss), 4 sources [ZTE, ETRI, vivo, Samsung] observe significant degradation (-7.55%~-32.3% loss). - For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, Huawei, ZTE, Samsung, ETRI, vivo, Spreadtrum]. - For UE speed#B is 10 km/h, minor loss (-0.6%~-1%) are observed by 3 sources [CATT, Xiaomi, Spreadtrum]. - For UE speed#B is 30 km/h, minor loss (-0.08%~-1.34%) are observed by 3 sources [Xiaomi, Apple, Huawei], moderate loss (-2.2%~-4.07%) are observed by 3 sources [Interdigital, vivo, Spreadtrum]. - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources [ZTE, Apple, Xiaomi, Huawei], moderate loss (-2%~-3.76%) are observed by 2 sources [vivo, Spreadtrum]. - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources [vivo, Samsung, ETRI, ZTE]. - Note: For generalization Case 3, 5 sources [ETRI, ZTE, Samsung, Interdigital, Fujitsu] observe significant performance degradations (-5%~-26.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance are still improved.

Therefore, applying/switching to a suitable AI/ML subject to UE speed is important to guarantee the performance of AI/ML solution.

Observation 1: At least for CSI prediction sub-use case, it is beneficial to switch AI/ML models based on UE speed.

As summarized in the companion contribution [3], either gNB or UE can be the management entity.

If management entity is located at UE, since the UE is aware of its speed by itself, the UE can switch/select a suitable AI/ML model if there’s multiple AI/ML model stored at the UE side. Otherwise, if there’s no model stored at the UE side, the UE may request model update (including transfer/delivery) from the network or OTT server. This could be either autonomously or event-triggered. For the later case, the UE speed of requested AI/ML model should also be reported to the network.

If management entity is located at gNB, model selection/switch should be triggered at the network side based on the UE speed reported from the UE. After selecting a new model suitable for the corresponding UE speed, the network should then transfer/delivery the model to the UE side.

As defined in TS 38.331 [2], the UE can report its location information and velocity information over CommonLocationInfo if configured. Therefore, existing signaling can support UE current speed information reporting.

#### – CommonLocationInfo The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information. CommonLocationInfo information element -- ASN1START -- TAG-COMMONLOCATIONINFO-START CommonLocationInfo-r16 ::= SEQUENCE { gnss-TOD-msec-r16 OCTET STRING OPTIONAL, locationTimestamp-r16 OCTET STRING OPTIONAL, locationCoordinate-r16 OCTET STRING OPTIONAL, locationError-r16 OCTET STRING OPTIONAL, locationSource-r16 OCTET STRING OPTIONAL, velocityEstimate-r16 OCTET STRING OPTIONAL } -- TAG-COMMONLOCATIONINFO-STOP -- ASN1STOP

Proposal 3: When management entity is located at NW side, the network can select/swich AI/ML model based on UE’s current speed.

According to the current configuration in ReportConfig, based on the availability, the UE can either periodically report its speed to the network, or triggered by other measurement event, e.g. radio condition, distance, etc. However, the UE cannot report its speed when its speed is changed. The NW cannot know in real-time when speed-based model update/change is triggered and cannot trigger model transfer/delivery from NW to UE when speed is changed. Motivated by this, the network can also configure a speed threshold as a new type of measurement event to trigger UE’s report. The UE can then send its current speed when it is higher/lower than the configured threshold. This can also reduce signaling overhead and UE’s complexity.

Proposal 4: When management entity is located at NW side, network configures one/more speed threshold(s) to the UE, once threshold is met, the UE reports its current speed to the NW.

However, considering the latency introduced by model transfer/delivery, the UE speed may change after the model is received at the UE side. This may lead to degraded performance of the newly received AI/ML model due to mismatch speed. For example, UE reporting speed is 10km/h, however, when the model finishes its transfer, UE’s speed becomes 60km/h. The AI/ML model that the UE just received may have performance down gradation since the UE speed is mismatched. Therefore, to mitigate the UE speed change compared to model transfer/delivery latency, predicted UE speed can be considered.

Observation 2: Current UE speed reporting is supported by existing signaling. However, AI/ML model selected/switched based on current UE speed may be out-dated considering the delay introduced by model transfer/delivery.

With that, the management entity can select/switch the AI/ML model based on the predicted UE speed corresponding to the finish time of model transfer/delivery. The predicted UE speed can either predicted at gNB side based on known UE speed received from CommonLocationInfo (if available), or predicted at UE side then report to gNB.

Proposal 5: If model transfer/delivery is needed, predicted UE speed at the finishing time of model transfer/delivery is considered for model selection based on UE speed.

Proposal 5.a: No specification impact if UE predicted speed is inferenced at gNB-side.

Proposal 5.b: UE to report predicted UE speed to gNB, if UE speed is inferenced at UE-side.

## AI/ML related Capability

In general, there are two types of AI/ML related capability to support AI/ML model running at the UE side:

- Functionality for a given sub-use case

- UE hardware capability for successfully processing/running the AI/ML model

We discuss detailed aspects for these two types of AI/ML capability.

Type 1: Functionality for a given sub-use case

For the first type, compared with legacy physical layer processing, AI/ML model inference may have certain performance shift. If the AI/ML model is purely trained/inferenced at the UE side, e.g. UE-sided model, the network should still be able to control whether the UE is using legacy approach or using AI/ML model to make sure a good system performance. To allow network has the visibility of UE’s capability of AI/ML, the UE needs to report what use case it can support by AI/ML. It is similar to legacy new UE features, where UE capability reporting is used to report the supported feature at UE side. It was agreed in RAN1 #112 meeting with following agreement:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

Proposal 6: At least for functionality based LCM, UE should report supported AI/ML sub-use case(s) and associated functionalities/functionality to the network via existing UE capability reporting procedure, i.e. RRC reporting for CSI/BM use case and LPP reporting for positioning use case (e.g. explicitly or implicitly by supported model ID, depends on the model ID structure discussion).

Type 2: UE hardware capability for successfully processing/running the AI/ML model

Type 2 would be more relevant for two-sided model or UE-sided model where model is downloaded from network side. It is different from legacy UE capability as it requires UE has specific hardware capability to be able to process the received AI/ML model from the network. For example, to successfully receive the model, the UE should have enough memory to store the trained model, therefore, the UE should report its supported maximum memory size to store a model.

Additionally, the UE should also be able to compile and execute the received AI/ML model. To support that, the network needs to understand the supported AI/ML structure by the UE, e.g. CNN, LSTM, RNN, etc.

Performing AI/ML model inference may require a lot of power consumption at UE side. It is better for the UE to report its maximum battery capacity and its current battery status to the network, so that the network may consider a proper duration for model activation and deactivation.

Different from legacy UE capability which is a fixed information, UE supported hardware capability may dynamically change based on UE’s current status, where UE capability reporting procedure may not be sufficient and additional signalling using UE Assistance information will be needed.

Based on above discussion, the UE hardware capability can further split into:

- Static hardware capability via existing UE capability reporting procedure

- Dynamic hardware capability as UE assistance information

Proposal 7: For two-sided model or UE-sided model where model is transferred from network to the UE, UE reports its static hardware capability (e.g. maximum model size, maximum battery capacity, supported model structure, etc) via existing UE capability reporting procedure and dynamic hardware capability (e.g. current battery status, current memory) via UE assistance information as AI/ML related capability to the network.

# Conclusion

In this contribution, we analyze the algorithm locality impact to RAN2 and AI/ML related capability with following observation and proposal:

Proposal 1: Considering model locality, Model update may be needed when 1) UE is handed-over to a new target cell which leads to performance down-gradation 2) prior knowledge about deployment scenario of target handover cell 3) handover to another NG-RAN node (e.g., another vendor).

Proposal 2: After UE’s handover, considering model locality and complexity of model transfer/delivery, based on NW control, the UE can keep using the existing model from source cell or update the model according to management decision, with the understanding that the network can always request/provide a model update at the UE side.

Observation 1: At least for CSI prediction sub-use case, it is beneficial to switch AI/ML models based on UE speed.

Proposal 3: When management entity is located at NW side, the network can select/swich AI/ML model based on UE’s current speed.

Proposal 4: When management entity is located at NW side, network configures one/more speed threshold(s) to the UE, once threshold is met, the UE reports its current speed to the NW.

Observation 2: Current UE speed reporting is supported by existing signaling. However, AI/ML model selected/switched based on current UE speed may be out-dated considering the delay introduced by model transfer/delivery.

Proposal 5: If model transfer/delivery is needed, predicted UE speed at the finishing time of model transfer/delivery is considered for model selection based on UE speed.

Proposal 5.a: No specification impact if UE predicted speed is inferenced at gNB-side.

Proposal 5.b: UE to report predicted UE speed to gNB, if UE speed is inferenced at UE-side.

Proposal 6: At least for functionality based LCM, UE should report supported AI/ML sub-use case(s) and associated functionalities/functionality to the network via existing UE capability reporting procedure, i.e. RRC reporting for CSI/BM use case and LPP reporting for positioning use case (e.g. explicitly or implicitly by supported model ID, depends on the model ID structure discussion).

Proposal 7: For two-sided model or UE-sided model where model is transferred from network to the UE, UE reports its static hardware capability (e.g. maximum model size, maximum battery capacity, supported model structure, etc) via existing UE capability reporting procedure and dynamic hardware capability (e.g. current battery status, current memory) via UE assistance information as AI/ML related capability to the network.

# References

[1] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2] TS 38.331, Radio Access Network; NR; Radio Resource Control (RRC) protocol specification

[3] R2-2307686, model control procedure: RAN2 impact, Intel Corporation