3GPP TSG-RAN WG2 Meeting #123                 	R2-2307691

Toulouse, FR, 21th – 25th Aug, 2023

Agenda Item:	7.17.2

Source: 	vivo

Title:         	Procedures for MUSIM temporary capability restriction

Document for: 	Discussion and Decision

# Introduction

Rel-18 MUSIM WI has achieved some agreements for the below objective [1]. The related agreements are listed in the Appendix for your convenience.

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE

Basically, RAN2 has agreed to use the following UAI based procedure as the baseline for reporting the temporary UE capability restriction information in network A in RRC_CONNECTED state for dual active MUSIM.

- B1: For UAI based solution, the following steps can be used as a baseline: - The UE is in Connected Mode on NW A. - The UE is configured for UE capability update via UAI. - The UE intends to start or stop connection with NW B or is already in Connected mode in NW B. - The UE requests a change (restriction or removal of restriction) of the UE capabilities at NW A via UAI. - NW A reconfigures the UE, if needed, according to its new capabilities (FFS if NW response is mandatory) - The UE operates in NW A with the updated configuration.

RAN2 has further identified several capabilities (i.e., MIMO layer, DC/CA, and measurement gap requirement) that can be changed due to dual active MUSIM. The reporting of these changed capabilities can be used by the UE to request the release of the specific configurations, or to avoid receiving an unexpected configuration in the future. Therefore, RAN2 agreed to introduce “reactive” and “proactive” approaches for the mentioned two cases.

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

This paper will discuss the above FFSs.

# Discussion

## 2.1 UE reporting for “Reactive” and “Proactive” approaches

In current NR UE capability, non-CA, CA and DC capabilities are reported in the supported BC lists. Except for per UE capabilities, the other UE capabilities are reported per band, or per BC, per FS, per FSPC, etc. Hence, the discussion of UE reporting in this paper will be started from the DC/CA capability update for dual active MUSIM.

DC/CA capability:

In RAN2#121bis meeting, the below agreement was made:

6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

As a common framework for “reactive” and “proactive” approaches, the UE can indicate its constrained BCs or constrained bands in a BC or constrained FeatureSetDownlinkPerCC-Id(s) in a band of a BC to the network A. Based on the constrained information reported by the UE, the network can decide to release or not to configure the SCell located on the constrained band/BC or associated to the constrained FeatureSetDownlinkPerCC-Id in a band of a BC. To limit the reporting size, the network A may configure a band filtering list which can include the serving/non-serving bands that the network is interested in.

A common framework can work, but is not efficient. When the UE is going to setup an RRC connection in network B, the UE needs to switch the shared RF quickly from network A to network B. If the shared RF is related to a serving band, and the UE is not configured to report the restriction information for this serving band, the UE anyway will switch the RF from network A, then, there will be data loss in network A. As a result, the network may mandatorily configure serving bands in the band filtering list. Hence, it makes more sense that the band filtering is only applied for non-serving bands (for proactive approach), i.e., the UE does not report the restriction information for the bands that the network is not interested in. And for reactive approach, the UE can directly indicate the serving cell index that to be released to the network A.

Observation 1: A common framework for reactive and proactive UE reporting can work, but is not efficient. The network may mandatorily configure serving bands in the band filtering list.

Band conflict:

RAN2 also agreed that band conflict scenarios will be covered by the temporary UE capability restrictions. When the UE is currently suffering band conflict or predicts there will have band conflict issue in the future, the above solution can be reused by the UE to indicate and solve the band conflict issue.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

Given above, we propose the following.

- A network configured band filtering is only applied for proactive approach. For proactively reporting DC/CA capability change and band conflict issue, if a band filtering is configured, the UE is only allowed to report constrained BCs or constrained bands in a BC or constrained FeatureSetDownlinkPerCC-Id(s) in a BC which has a band included in the band filtering.

- For reactively reporting DC/CA capability change and band conflict issue, the UE includes the serving cell index to be released / SCG release indication in the UAI.

Measurement gap requirement:

In RAN2#121bis meeting, RAN2 has agreed the below agreements for measurement gap requirement change due to dual active MUSIM.

3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B.

UE’s measurement gap requirement is determined according to UE’s current configuration and UE capability. In current specification which was designed for single USIM case, when receiving RRC reconfiguration message, the UE can include measurement gap requirement update in RRC reconfiguration complete message, if network allows. Now, the UE may change measurement gap requirement due to capability change for Rel-18 MUSIM case. From our understanding, only reactive approach is applied to measurement gap requirement change.

Observation 2: Only reactive approach is applied to measurement gap requirement change for dual active MUSIM operation.

If reusing the current needForGapInfoNR in RRC reconfiguration complete message for MUSIM purpose, the network needs to send a RRC reconfiguration message to trigger the measurement gap requirement update reporting even if there is no parameter need to be (re)configured. Also, the latency for waiting the RRC reconfiguration message may cause data interruption on the active serving cells in network A. Thus, it is preferred to extend the similar function in UAI. If network allows, the UE can directly send UAI for updating the measurement gap requirement for each serving frequency, and for each target band included in the requestedTargetBandFilterNR-r16 (RAN2 can discuss whether to introduce a new filter list in OtherConfig) or each supported NR band.

- For Rel-18 MUSIM dual active operation, extend the similar measurement gap requirement change reporting in RRC reconfiguration complete to UAI.

Maximum UL/DL MIMO layer:

In RAN2#121bis meeting, RAN2 has agreed the below agreements for the maximum MIMO layer.

1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

There is an editor note in 38.331 running CR.

Editor note:	FFS what granularity is used for indicating temporary maximum number of MIMO layers.

In RAN2#121bis email discussion, the below options were discussed.

- Option 1: per direction (DL/UL) and per FR, with the same maximum MIMO layer for each serving cell

- Option 2: per FSPC

- Option 3: per serving cell

Option 1 may decrease the UE’s performance because the UE does not need to downgrade the supported maximum MIMO layers on each serving cell, so we prefer Option 2 or 3. Since proactive and reactive approaches have been agreed, it is possible to support both option 2 and option 3. For option 2, the band filtering list can be applied, i.e., the UE only includes the maximum MIMO layer per CC per feature set entry for the BC which contains a band included in the band filtering list. And for option 3, the UE can indicate the maximum MIMO layer for specific serving cells by using the serving cell index. Other capabilities are still FFS, but basically the same principle can be applied.

- If RAN2 agrees maximum MIMO layer change can be reported for specific serving cell, the UE includes the maximum MIMO layer for the specific serving cell by using the serving cell index in the UAI.

- If RAN2 agrees maximum MIMO layer change can be reported per FSPC, the UE only includes the maximum MIMO layer per CC per feature set entry for the BC which contains a band included in the band filtering list in the UAI.

Removal of restriction information:

When the RRC connection in network B ends up, the UE can remove the capability restriction in network A, by not including the detailed capability restriction information in the Rel-18 MUSIM field in the UAI. And it is up to the UE when to send the removal of the capability restriction.

- The UE can remove the MUSIM capability restriction information by not including the detailed fields in Rel-18 MUSIM field in the UAI.

Wait timer for reactive approach:

If the UE requests a change of capabilities proactively, a new RRC configuration may not be needed at the UE side. At the UE side, the UE will switch its capabilities upon UAI is successfully sent to the network. If the UE requests a change of capabilities reactively, it is obvious that the UE is expected to receive a new RRC configuration from network A immediately. However, the network A may still want to use the requested resources for transmission for a short time.  In this case, it is beneficial to introduce a wait timer for receiving the new RRC configuration in network A, which is similar to the scheme adopted in Rel-17 MUSIM for long-leaving network switching. That is, the UE shall maintain the capabilities until the expiry of wait timer. If the UE does not receive a new RRC reconfiguration within the wait timer, the UE is allowed to switch its capabilities locally.

- If the UE requests a change of UE capabilities reactively, the UE starts a wait timer. The UE shall maintain the capabilities until the expiry of wait timer.

## 2.2 NW enable/disable configurations

The question is if “proactive” and “reactive” need to be configured separately.  From network perspective, in some cases, the network may not be interested in the proactive information, for example, the network does not intend to configure more resources to the UE, however, proactive reporting will cause much signalling overhead. When the network has such need, the network can enable proactive reporting. Thus, we think separate configurations for “proactive” and “reactive” can provide full network flexibility.

- Introduce separate network enable configurations for the “proactive” and “reactive” approaches.

Moreover, the below Editor note is captured in the 38.331 running CR:

Editor note:	FFS whether one configuration to control all temporary capabilities update or introduce individual control for each temporary capability update.

From our understanding, if the network can control constrained information reporting for each impacted temporary capability, it’s hard to say this feature can really help the UE to solve the MUSIM hardware conflict issue. And if we consider proactive and reactive framework, we will introduce many configurations for each capability for proactive and reactive, respectively. So, one configuration to control all type of changed capability is simple, thus is preferred to us.

- Not introduce individual control for each temporarily changed capability.

Prohibit timer:

According to the discussion so far, the UE may only request capability change in network A in RRC_CONNECTED due to connection start in network B. And RRC state transition may not be so frequent. Besides, from UE’s performance perspective, the UE will not change its capabilities frequently because capability change may involve RRC reconfiguration in both networks, and reconfiguration in each network may cause an interruption on the other. From our understanding, the UE may not request capability change frequently. So, prohibit timer is not needed.

- Prohibit timer for the signaling of UE capability changes is not supported in Rel-18 MUSIM.

# Conclusion

In this paper, the following proposals are given:

- A network configured band filtering is only applied for proactive approach. For proactively reporting DC/CA capability change and band conflict issue, if a band filtering is configured, the UE is only allowed to report constrained BCs or constrained bands in a BC or constrained FeatureSetDownlinkPerCC-Id(s) in a BC which has a band included in the band filtering.

- For reactively reporting DC/CA capability change and band conflict issue, the UE includes the serving cell index to be released / SCG release indication in the UAI.

- For Rel-18 MUSIM dual active operation, extend the similar measurement gap requirement change reporting in RRC reconfiguration complete to UAI.

- If RAN2 agrees maximum MIMO layer change can be reported for specific serving cell, the UE includes the maximum MIMO layer for the specific serving cell by using the serving cell index in the UAI.

- If RAN2 agrees maximum MIMO layer change can be reported per FSPC, the UE only includes the maximum MIMO layer per CC per feature set entry for the BC which contains a band included in the band filtering list in the UAI.

- The UE can remove the MUSIM capability restriction information by not including the detailed fields in Rel-18 MUSIM field in the UAI.

- If the UE requests a change of UE capabilities reactively, the UE starts a wait timer. The UE shall maintain the capabilities until the expiry of wait timer.

- Introduce separate network enable configurations for the “proactive” and “reactive” approaches.

- Not introduce individual control for each temporarily changed capability.

- Prohibit timer for the signaling of UE capability changes is not supported in Rel-18 MUSIM.

# References

[1] RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo.

# Appendix

## RAN2#121bis Agreements

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities Support “early indication” from UE to network during RRC connection setup/resume procedure. FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.) No consensus to support UE-initiated SCell deactivation for MUSIM in Rel-18. 1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B. 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change. 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case. 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose. RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123. 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

## RAN2#121 Agreements

- A2a: When the UE is in Connected mode in two NR networks, it is up to the UE implementation to select which NW to perform signaling for temporary UE capability restrictions.

- A2b: When the UE is in Connected mode in NR NW A and moving from Idle/Inactive to connected mode in NR NW B, the signaling for temporary UE capability restrictions can happen on NW A. FFS how to handle if UE is moving from IDLE/INACTIVE in NW A and is in CONNECTED with NW B.

- A2c: When the UE is in Connected mode in both networks and one is E-UTRAN, the signaling for temporary UE capability restrictions happens on the NR network.

- A3: The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. FFS whether the UE can indicate if it is already connecting with reduced capabilities during connection set-up/resume.

- A4: RAN2 to discuss whether prohibit timer is needed for the signaling of temporary UE capability restrictions This can wait until after progress is made on the signaling framework.

- A1: UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM. FFS if we have additional signalling (depends on e.g. SCell/SCG deactivation usability for MUSIM)

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

- •	Transmission and reception capabilities (e.g. MIMO layers)

- •	Measurement capabilities (e.g. gaps)

- •	Supported bandwidth

- •	Supported band-combinations (FFS whether this is CA or DC or both)

- FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion).

- FFS whether UE reports some or all of the above or whether we can do something simpler

- A6: For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. FFS if we support deactivation (based on discussion in which case it can be used). It is up to network how to react to UE request.

- RAN2 does not intend to create new procedures for e.g. SCell/SCG deactivation for MUSIM purposes in Rel-18. Existing procedures can be used based on NW choice.

- B4: RAN2 considers the only RAN3 impact may be to support the UE request of SCG/SCell release via SRB3 (if supported) for MUSIM purpose (e.g. cause value).  If this can be done via inter-node messages, RAN2 expects no RAN3 impacts.

- 1: The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

- Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

## 5.2 RAN2#119bis Agreements

The R18 MUSIM solution should work in DC/CA and RAN sharing scenarios (but need not be optimized for RAN sharing). RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction. 2 The following is assumed when defining the solution: - The two networks are independent (i.e. no inter-network communication); - The Core Network is not aware of the temporary restrictions of the UE capability; 1: RAN2 can discuss NW A MN-SN coordination of Rel-18 MUSIM temporary capability restrictions due to UE being configured with NR-DC in NW A. RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN RAN2 needs to discuss which UE capabilities can be impacted by sharing of resources between the MUSIM links. RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details). A7: The UE can initiate signalling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need. A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class. For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected. For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI. Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI) - B1: For UAI based solution, the following steps can be used as a baseline: - The UE is in Connected Mode on NW A . - The UE is configured for UE capability update via UAI. - The UE intends to start or stop connection with NW B or is already in Connected mode in NW B. - The UE requests a change (restriction or removal of restriction) of the UE capabilities at NW A via UAI. - NW A reconfigures the UE, if needed, according to its new capabilities (FFS if NW response is mandatory) - The UE operates in NW A with the updated configuration. - B2: For delta-signaling of UE capability, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE signals the changed UE capabilities to NW A via delta-signaling. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. - B3: The solution for the repetition of UE capability enquiry, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests a UE capabilty update request. - NW A sends UECapabilityEnquiry to the UE - UE sends UECapabilityInformation to the NW A gNB. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory. - The UE operates in NW A with the updated configuration. - B4: The solution based on using UE-profiles for capability restriction, the following steps can be used as a baseline: - The UE signals different temporary UE capability sets during registration (FFS if these profiles can be updated later) - The UE is in Connected Mode in NW A . - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests to switch to a different UE capability profile, e.g. by signaling an index of the profile. - NW A reconfigures the UE according to its new capabilities. - The UE operates in NW A with the updated configuration. - B5 (11/15): A baseline procedure for MAC-CE based SCell (de)-activation can be considered as follows: - The UE is in Connected Mode in NW A . - The UE is configured for MAC-CE based SCell (de)-activation operation. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE sends a request to deactivate SCells via MAC-CE. - NW A deactivates, if needed, the requested SCells (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. CX: RAN2 to continue evaluation of any Xn-AP, F1-AP or RAN4 impact due to dual-active MUSIM operation. 1: RAN2 can consider such Band conflict scenarios for MUSIM in CONNECTED to arrive at a graceful specification-based solution intended to mitigate such conflicts. Wait for RAN4 feedback on MUSIM gap priority.