3GPP TSG-RAN WG2 Meeting #123	R2-2307724

Toulouse, France, August 21-25, 2023

Agenda Item:	7.15.3

Source: 	Xiaomi

Title:  	Discussion on resource allocation and enhanced LCP for SL-U

Document for:	Discussion and Decision

1	Introduction

At RAN2#121 meeting, RAN2 discussed resource selection for SL unlicensed and reached some agreements [1]:

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

In RAN2#122 meeting, RAN2 discussed whether SL LBT failure triggers resource (re)selection or not for MCSt case. However, no conclusion was achieved [2].

- Yes (P3:4806/P4-5:5686)

  - Proposal 4: For MCST case, reselect consecutive resources for dropped PSSCH transmission due to an LBT failure indication from L1

  - Proposal 5: For MCST case, the resource (re)selection is triggered when LBT succeed and there has dropped PSSCH transmission due to an LBT failure indication from L1

- No (P3:4793)

  - Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

- To be concluded next meeting.

In this contribution, we would like to discuss about issues related to resource allocation and provide corresponding proposals.

2	Discussion

2.1 Resource allocation from MCSt

2.1.1 MCSt of single TB

During last RAN2 meeting, RAN2 discussed about whether to trigger resource reselection if LBT fails for MCSt case, however companies cannot reach consensus. According to RAN1, MCSt supports both single TB and multiple TB. For MCSt of single TB, it can be regarded as some kind of repetition and if LBT on the first opportunity passes, UE can transmit all the following repetitions. In this case, if LBT fails and resource reselection is triggered, then UE needs to reselect MCSt resource for this single TB, but actually there is still multiple consecutive retransmission resources for this single TB, it is not desirable to perform resource (re)selection just because LBT failure is detected in one resource out of them, which will bring latency and degrade transmission performance, especially considering the motivation of MCSt is to provide multiple consecutive transmission opportunities to reduce the LBT attempt. In this case, UE should not trigger resource reselection if LBT fails on the transmission occasion and continue to try the next transmission opportunity.

Proposal 1: UE does not trigger resource reselection if LBT fails and continue to try the next transmission opportunity when MCSt is to transmit single TB.

Actually during previous RAN2 meeting, similar issue was discussed for non-MCSt case and RAN2 achieves the following agreement. It seems the agreement is some kind of RAN2 understanding which depends on confirmation from RAN1. Considering no reply LS from RAN1 was received and the situation is quite the same as MCSt case, i.e., UE can reserve the resource for initial transmission and retransmissions, if initial transmission fails due to LBT failure, the resource for retransmissions may be used without resource reselection, it is proposed to have common handling for MCSt case and non-MCSt case for the transmission of single TB.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

Proposal 2: UE does not trigger resource reselection if LBT fails and continue to try the next reserved retransmission opportunity for non-MCSt case.

2.1.2 MCSt of multiple TB

Regarding MCSt of multiple TB, actually according to LS from RAN1, there are three candidate approaches. For both approach 1 and approach 3, the delivered resource from PHY is single-slot resource as shown Figure 1 while for approach 2, the delivered resource from PHY is multiple-slot resource as shown in Figure 2.

Figure 1: MCSt for multiple TB based on approach 1/3

Figure 2: MCSt for multiple TB based on approach 2

In this case, for MCSt of multiple TB based on approach 1/3, if LBT fails on the first transmission occasion of TB1, considering the next transmission occasion is for TB2, it is not feasible to try the next opportunity for TB1. Therefore to achieve MCSt of TB1/2/3, one way is to trigger resource reselection to reselect MCSt resource for TB1/2/3. Another alternative is to try the next MCSt opportunity, i.e., 2nd MCSt in Figure 1.

Proposal 3: RAN2 to discuss the following two alternatives for MCSt of multiple TB based on approach 1/3

- Alternative 1: UE does not trigger resource reselection if LBT fails and continue to try the next MCSt opportunity.

- Alternative 2: UE triggers resource reselection to reselect MCSt resources for multiple TB if LBT fails.

While for MCSt of multiple TB based on approach 2, considering the delivered resource from PHY is multiple-slot resource, if LBT fails on the first transmission occasion of TB1, it is still feasible to try the next opportunity for TB1 to achieve the benefit of MCSt. If all the transmission opportunities for TB1 in 1st MCSt fail, then since the following resources are for TB2/3, UE can either trigger resource reselection to reselect MCSt resources for multiple TB or try the next MCSt opportunity, i.e., 2nd MCSt, similar as the case of MCSt of multiple TB based on approach 1/3.

Proposal 4: UE does not trigger resource reselection if LBT fails and continue to try the next transmission opportunity for MCSt of multiple TB based on approach 2.

2.1.3 Number of slots for MCSt

According to the LS from RAN1 [3], for both approach 2 and approach 3, MAC should deliver a new parameter “number of slots” to PHY to determine the candidate multiple-slot resources. However, how to derive this parameter based on CAPC of LCHs/multiple TBs remains unclear.

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

For approach 2, the multiple-slot resource is for single TB, in this case, this parameter should be determined based on the CAPC of the LCHs to be multiplexed in this TB. Actually each CAPC corresponds to a maximum COT duration, the larger the CAPC/the lower the CAPC priority, the shorter the COT duration. According to the existing agreement, if MAC SDU is multiplexed in the TB, UE selects the lowest CAPC priority as the CAPC of the TB, therefore in order to make best use of the COT duration as well as to ensure the number of slots is not larger than the maximum COT duration of the TB, this parameter should be derived based on the lowest CAPC priority (highest CAPC value) of LCHs of the TB.

Agreement on SL CAPC mapping rule:

1: 	As in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Proposal 5: MAC derives the parameter “number of slots” based on the lowest CAPC priority of LCHs to be multiplexed in the TB for MCSt based on approach 2.

While for approach 3, the multiple-slot resources is for multiple TB, in this case, this parameter should be determined based on the CAPC of multiple TBs. Similar as above, to ensure the number of slots is not larger than the maximum COT duration of multiple TBs, this parameter should be derived based on the lowest CAPC priority of multiple TBs.

Proposal 6: MAC derives the parameter “number of slots” based on the lowest CAPC priority of multiple TBs for MCSt based on approach 3.

2.2 Resource allocation from the LBT impact

Actually we have sent LS to RAN1 to inform RAN1 the conclusion from RAN2 on both inter-UE case and intra-UE case. However during the discussion in RAN1, the majority view is that both inter-UE case and intra-UE case should be discussed by RAN1.

- Approach 1 (4): OPPO, ZTE, LGE, Spreadtrum - No duplicate work between RAN1/RAN2 on same issue, optimization issue, no need to prioritize this issue, respect to RAN2 decision - Approach 2 (13): CATT/GH, QC, Intel, Futurewei, DCM, MediaTek, vivo, Samsung, CMCC, HW/HiSi, Lenovo - RAN1/RAN2 solution scope is different, LBT parameters and resource selection are RAN1 domain

In addition, RAN1 achieved the following WA to handle the LBT blocking issue from another UE (inter-UE case), i.e., option 2 and option 1 are supported separately based on UE capability, see below [4]. At least option 1, which is to avoid selection of consecutive resources that overlaps with the LBT of the reserved resource, can be easily extended to handle the intra-UE case, i.e., similar approach can be applied to MAC when selecting resources from the candidate resource set to avoid the LBT blocking issue. Therefore, we think to avoid the duplicated discussion in different WGs and considering there is no final agreement for inter UE case, it is better to wait for more RAN1 input for intra-UE case.

Working assumption For Type 1 LBT block issue (inter-UE case), the following option 2 and option 1 are supported separately based on UE capability - Option 2: If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE may prioritize/select resource(s) in the slot(s) for transmission. - FFS: details of applying this prioritization, which layer to perform above prioritization behaviour, and if the reserved resource belongs to a MCSt, the COT initiating UE should be able to share the COT to cover the whole MCSt - (pre)configuring enabling/disabling option 2 is supported - Option 1: - UE may avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority. - The value of N can be selected from {0, 1, 2} - The selection of the value of N is up to UE implementation - FFS: unless (pre-)configured or indicated by UE reserved resource in SCI - UE may avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - M is determined based on UE implementation (at least including 0) - FFS: Which layer to perform above behaviour - FFS: any restriction of M - (pre)configuring enabling/disabling option 1 is supported - FFS: Whether the above high priority is determined according to a (pre)configured threshold - Note: both option1 and option2 are optional UE features

Proposal 7: RAN2 to wait for more RAN1 input on type 1 LBT blocking issue for intra-UE case.

2.3 Resource allocation from COT sharing

With the introduction of IUC, UE shall randomly select the resources within the intersection of the received preferred resource set and the resources indicated by the physical layer when preferred resource set is received and UE has its own sensing result. Similarly, even the detailed procedure of resource allocation is still under discussion in RAN1, from MAC perspective, some mechanism that impact the selection procedure in higher layer can be considered by RAN2. Since less effort on LBT is required for resources within the shared COT, when the UE is the responding UE and operates in mode 2, it is beneficial to firstly select resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection. If there is no remaining resource within the shared COT that can be selected, then the UE randomly select resources from the resources indicated by the physical layer as in legacy.

Proposal 8: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

2.4 Enhanced LCP from COT sharing

In RAN2#122 meeting, RAN2 further confirmed CAPC restriction is applicable to enhanced LCP.

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

Before discussing the detailed procedure case by case, we think we should first confirm the WA as agreement to align with RAN1 agreement.

Proposal 9: Confirm the WA “For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement” as agreement.

- Case 1: COT sharing information arrives later than packet generation:

For this case, since the responding UE has already finished the LCP upon reception of the COT sharing information, if the generated packet satisfies the COT requirement, i.e., the initiating UE is the target receiver of the packet and the CAPC of the packet is equal to or smaller than the CAPC indicated in the COT sharing information, then to increase the efficiency of COT sharing, the UE should use the shared COT, i.e., performs type 2 LBT.

Actually during online discussion, there was some concern that even the PDU has been generated, there may be “other PDU” pending for transmission and utilizing the shared COT. However, we don’t think UE should prioritize “other PDU” when there is already a PDU satisfying the requirement, this is because to transmit “other PDU”, UE needs to preform type 1 LBT which is a waste of the shared COT with the access latency/complexity increased. Also there was some voice to leave this to UE implementation whether to perform type 1 LBT or type 2 LBT. However we think if the already generated MAC PDU is able to utilize the shared COT with a type 2 LBT, UE has no motivation to perform a type 1 LBT, it does not make sense to leave this to UE implementation.

Observation 1: To transmit “other PDU” instead of the PDU satisfying the COT requirement is a waste of shared COT with the access latency/complexity increased.

Regarding the decision on LBT type, there was some comment that this is more of PHY scope and transparent to MAC. We think for normal transmission on the unlicensed spectrum, it is purely PHY decision, however the story of COT sharing is different since type 2 LBT is allowed only when the generated MAC PDU can satisfy the shared COT, but actually whether the MAC PDU satisfies the shared COT or not is within the MAC scope. Therefore, we think to perform a type 2 LBT is a MAC and PHY joint decision and is some kind of internal interaction.

Observation 2: It is a MAC and PHY joint decision to perform a type 2 LBT since whether the MAC PDU satisfies the shared COT or not is within the MAC scope.

Based on the above analysis, we think for this case, if the generated packet satisfies the COT requirement, UE should utilize the shared COT with a type 2 LBT executed.

Proposal 10: If the resource to be used is within a shared COT, and if PDU is generated before COT arrival, and the PDU satisfies COT requirement, UE should transmit this generated PDU with a type 2 LBT executed.

- Case 2: COT sharing information arrives before packet generation:

For this case, the responding UE can perform an enhanced LCP to satisfy the shared COT. In last meeting, we discussed about this case and achieved the following agreement.

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

Actually there was some proposal to consider the priority/remaining PDB of the LCHs that have data for transmission when determining whether to perform an enhanced LCP. Even we have some sympathy on this proposal, the whole system can still survive if no such additional conditions are introduced. For UEs operating in both mode 1 and mode 2, either the scheduled grant or the selected grant e.g., initial transmission /retransmission should take the QoS requirement into account, so it is not that serious if the shared COT is utilized to transmit a “satisfied PDU” even not having the highest priority or the most stringent PDB requirement. The “other” LCHs can still survive with the following scheduled/selected transmission resource.

Observation 3: LCHs with high priority and stringent PDB requirement can still survive with the following scheduled/selected transmission resource.

In addition, the existing requirement to utilize the shared COT is already quite complicated, if more conditions are considered, on one hand, the impact on the specification is significant, on the other hand, it is not an easy job to define a specific rule, e.g., to consider either one or both of priority and remaining PDB. Also the bar of priority and/or remaining PDB to restrict the use of enhanced LCP should be determined. Besides priority/remaining PDB, it is also possible to rely on the NW configuration to restrict the enhanced LCP, e.g., NW can flag some LCHs and when these LCHs are having data for transmission, UE is not allowed to perform an enhanced LCP.

Observation 4: It is not an easy job to define a specific rule to restrict the usage of enhanced LCP.

Considering all the mentioned aspects above, we prefer to not define any additional rules to limit the UE to use the enhanced LCP. Or if there is really some concern we think it is also enough to leave to UE implementation whether to perform an enhanced LCP.

Proposal 11: If the resource to be used is within a shared COT, and if PDU is not generated before COT arrival, and there is data satisfying COT requirement, whether to perform enhanced LCP is up to UE implementation.

Regarding how to enhance LCP and use the shared COT, we achieved the following agreement.

Agreement:

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

However, it is not clear how to restrict the destinations to be those allowed by the COT, e.g., whether the CAPC requirement is considered or not during DST selection procedure.

- CAPC requirement is considered during destination selection:

According to the existing destination selection procedure, the UE should select the destination that having at least one of the MAC CE and the LCH with the highest priority among the LCHs that satisfy the existing destination selection conditions [see Annex], e.g., SL data available, CG/HARQ feedback limitation etc. If CAPC is considered, one more condition can be added on top of the other conditions, i.e., associated CAPC is equal to or smaller than the CACP indicated in the COT sharing information.

In this case, at the first step, when selecting the destination, the responding UE should judge whether the initiating UE has a highest priority LCH/MAC CE satisfying the existing destination selection conditions as well as the CAPC requirement. If so, the responding UE can select the initiating UE as the destination and then continue to select LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions [see Annex] and have a CAPC that is equal to or smaller than the indicated CAPC, then the responding UE performs type 2 LBT. However, if the initiating UE has no highest priority LCH/MAC CE that satisfying the existing destination selection conditions as well as the CAPC requirement, i.e., the initiating UE cannot be selected as the destination, then the responding UE can fallback to legacy LCP procedure to select DST and LCHs, and performs a type 1 LBT.

Figure 1: CAPC requirement is considered during destination selection

- CAPC requirement is not considered during destination selection:

In this case, the responding UE relies on the legacy destination selection procedure to judge whether the initiating UE can be selected as the destination or not with no consideration on the CAPC requirement, i.e., whether the initiating UE has a highest priority MAC CE/LCH satisfying the existing destination selection conditions or not.

If so, the responding UE can select the initiating UE as the destination, then further check if there is any LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions and the CAPC requirement. If there is any LCH can be selected, the responding UE performs a type 2 LBT; otherwise, if there is any LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions but cannot meet the CAPC requirement, the responding UE select these LCHs, then performs a type 1 LBT; if the initiating UE cannot be selected as the destination based on the existing destination selection condition, the responding UE selects performs a legacy LCP to select the DST and LCHs, and performs a type 1 LBT.

Figure 2: CAPC requirement is not considered during destination selection

Based on the above analysis, it is much easier to implement the enhanced LCP procedure if CAPC requirement is considered during destination selection procedure. Therefore, we have the following proposals:

Proposal 12: If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, CAPC requirement is considered during destination selection procedure.

Proposal 13: If the initiating UE has a LCH satisfying the existing destination selection conditions as well as the CAPC requirement, the responding UE selects the initiating UE as the destination, otherwise, the responding UE falls back to legacy LCP procedure.

Then the next step after DST selection is to select LCHs, with P3/4, during DST selection, the CAPC requirement is already considered, therefore, during LCH selection procedure, the responding UE can select at least one LCH. Also, in order to guarantee the generated packet satisfies the CAPC requirement, all the other selected LCHs should have a CAPC smaller than or equal to that indicated in the shared COT information. This is because we have agreed the CAPC of the MAC PDU is the lowest priority CAPC of the LCHs included in the MAC PDU, in this case, in order to satisfy the CAPC requirement, all the selected LCHs should have a CAPC smaller than or equal to that indicated in the shared COT information. In addition, there are some existing conditions [see Annex] for LCH selection, the selected LCHs should also meeting these legacy conditions.

Agreement on SL CAPC mapping rule:

1: 	As in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Proposal 14: If the initiating UE is selected as the destination, further select the LCHs satisfying the existing conditions and having CAPC smaller than or equal to the CAPC value indicated in the COT sharing information.

3	Conclusion

In this contribution, we discussed about resource allocation and enhanced LCP for SL-U and have the corresponding proposals:

Proposal 1: UE does not trigger resource reselection if LBT fails and continue to try the next transmission opportunity when MCSt is to transmit single TB.

Proposal 2: UE does not trigger resource reselection if LBT fails and continue to try the next reserved retransmission opportunity for non-MCSt case.

Proposal 3: RAN2 to discuss the following two alternatives for MCSt of multiple TB based on approach 1/3

- Alternative 1: UE does not trigger resource reselection if LBT fails and continue to try the next MCSt opportunity.

- Alternative 2: UE triggers resource reselection to reselect MCSt resources for multiple TB if LBT fails.

Proposal 4: UE does not trigger resource reselection if LBT fails and continue to try the next transmission opportunity for MCSt of multiple TB based on approach 2.

Proposal 5: MAC derives the parameter “number of slots” based on the lowest CAPC priority of LCHs to be multiplexed in the TB for MCSt based on approach 2.

Proposal 6: MAC derives the parameter “number of slots” based on the lowest CAPC priority of multiple TBs for MCSt based on approach 3.

Proposal 7: RAN2 to wait for more RAN1 input on type 1 LBT blocking issue for intra-UE case.

Proposal 8: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

Observation 1: To transmit “other PDU” instead of the PDU satisfying the COT requirement is a waste of shared COT with the access latency/complexity increased.

Observation 2: It is a MAC and PHY joint decision to perform a type 2 LBT since whether the MAC PDU satisfies the shared COT or not is within the MAC scope.

Observation 3: LCHs with high priority and stringent PDB requirement can still survive with the following scheduled/selected transmission resource.

Observation 4: It is not an easy job to define a specific rule to restrict the usage of enhanced LCP.

Proposal 9: Confirm the WA “For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement” as agreement.

Proposal 10: If the resource to be used is within a shared COT, and if PDU is generated before COT arrival, and the PDU satisfies COT requirement, UE should transmit this generated PDU with a type 2 LBT executed.

Proposal 11: If the resource to be used is within a shared COT, and if PDU is not generated before COT arrival, and there is data satisfying COT requirement, whether to perform enhanced LCP is up to UE implementation.

Proposal 12: If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, CAPC requirement is considered during destination selection procedure.

Proposal 13: If the initiating UE has a LCH satisfying the existing destination selection conditions as well as the CAPC requirement, the responding UE selects the initiating UE as the destination, otherwise, the responding UE falls back to legacy LCP procedure to select DST and LCHs.

Proposal 14: If the initiating UE is selected as the destination, further select the LCHs satisfying the existing conditions and having CAPC smaller than or equal to the CAPC value indicated in the COT sharing information.

4	Reference

[1] RAN2#121 meeting minutes.

[2] RAN2#122 meeting minutes.

[3] R1-2304257 LS on MCSt resource (re-)selection, RAN1

[4] RAN1#113 meeting minutes.

5 Annex: LCP procedure

5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured according to TS 38.331 [5]:

2>	if the new transmission is associated to a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon:

3>	select a Destination associated with NR sidelink discovery as specified in TS 23.304 [26], that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

4>	SL data for NR sidelink discovery is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant.

2>	else:

3>	select a Destination associated to one of unicast, groupcast and broadcast (excluding the Destination(s) associated with NR sidelink discovery as specified in TS 23.304 [26]), that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

4>	SL data for NR sidelink communication is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

4>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

1>	else:

2>	select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

3>	SL data is available for transmission; and

3>	SBj > 0, in case there is any logical channel having SBj > 0; and

3>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

3>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	sl-HARQ-FeedbackEnabled is set to the value that satisfies the following conditions:

3>	if PSFCH is configured for the sidelink grant associated to the SCI and the UE is capable of PSFCH reception:

4>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

4>	sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

3>	else:

4>	sl-HARQ-FeedbackEnabled is set to disabled.

NOTE 2:	HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE or Sidelink DRX Command MAC CE or Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.