3GPP TSG-RAN WG2 Meeting #123	R2-2307726

Toulouse, France, August 21-25, 2023

Agenda Item:	7.15.6

Source: 	Xiaomi

Title:  	Discussion on carrier aggregation for NR sidelink

Document for:	Discussion and Decision

1	Introduction

In RAN#99 meeting, the objective on carrier aggregation for NR sidelink was approved [1].

In previous RAN2 meetings, we discussed about NR sidelink CA and achieved several agreements. In addition, one LS was sent to SA2 to ask for the carrier mapping for unicast.

In this contribution, we would like to discuss some remaining issues related to carrier aggregation for NR sidelink based on the LTE sidelink CA design and have corresponding proposals.

2	Discussion

2.1 TX Carrier (re)selection/keeping

In last RAN2 meeting, we discussed about per-carrier CBR and achieved the following WA.

- Working assumption: Option1 (same principle as LTE V2X CA)

Agreements on per carrier CBR

1: 	Same principle as LTE V2X CA is applied.

Considering there is no issue if we follow LTE principle, it is proposed to confirm the WA as agreement

Proposal 1: In NR sidelink CA, same principle as LTE V2X CA is applied to determine per-carrier CBR.

In previous RAN2 meeting, we discussed about the threshold for carrier selection/reselection and keeping groupcast/broadcast.

Agreement:

Proposal 5	NR SL CA TX carrier (re)selection follows LTE CA solution, i.e., define 1) per- carrier-per-priority CBR threshold for carrier (re)selection, and 2) per-carrier-per-priority CBR threshold for carrier keeping. And final carrier selection is done based on the lowest CBR value across carriers. Where the priority is the LCH priority.

FFS on unicast case.

In last RAN2 meeting, carrier selection/reselection and keeping for unicast was discussed and agreed to follow the same principle as groupcast/broadcast.

Agreements on TX carrier (re)selection triggers, LCP impact, and CBR-based carrier reselection/keeping for UC

1: 	Agreements made for GC/BC (RAN2#121bis-e) are also applicable for UC. TX carrier reselection is done among the carriers that peer UE also supports.

With the agreement to define per-carrier-per-priority CBR threshold for carrier keeping and per-carrier-per-priority CBR threshold for carrier (re)selection, it still remains unclear how to (re)select/keep the carrier with the defined threshold. In LTE sidelink CA, the UE should consider the carrier as a candidate carrier if the measured CBR of the carrier is lower than the threshold and the UE shall stay at the same TX carrier as before if the measured CBR of the carrier is lower than the configured threshold. Similar principle should apply to NR sidelink CA.

In addition, in last RAN2 meeting, we discussed about HARQ-based SL RLF and agreed the counting is per carrier.

Agreement on DTX based SL RLF in SL CA

1:	The counting is calculated per carrier.

2:	Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

In this case, it is possible that the DTX counting for some carriers may reach the maximum, but with other carriers available for SL CA, UE will not declare legacy SL RLF. Then during carrier selection/reselection and carrier keeping, the impact from HARQ-based DTX counting should be considered. To be more detailed, for carrier selection/reselection/keeping, on top of CBR threshold and/or higher layer restriction, UE should take the DTX counting of the carrier into account, i.e., the HARQ-based DTX counting for any unicast link associated with the UE should not extend the maximum, then the carrier can be considered as a candidate carrier for carrier selection/reselection or the UE can continue to stay at this carrier.

Proposal 2: In NR sidelink CA, the UE considers a TX carrier as candidate carrier if the measured CBR is lower than the configured per-carrier-per-priority CBR threshold for carrier (re)selection and the HARQ-based DTX counting for any unicast link associated with the UE on this carrier does not reach the maximum.

Proposal 3: In NR sidelink CA, the UE stays at the same TX carrier as before if the measured CBR is lower than the configured per-carrier-per-priority CBR threshold for carrier keeping and the HARQ-based DTX counting for any unicast link associated with the UE on this carrier does not reach the maximum.

2.2 PDCP duplication/SL CA for SL SRB

In last RAN2 meeting, we discussed about how to enable PDCP duplication and achieved the following agreement.

Agreement on criterion for packet duplication

1:	SLRB configures whether PDCP duplication is used or not

In this case, for a SL DRB, the UE should rely on (pre)configuration of SLRB to determine whether PDCP duplication is enabled or not. If PDCP duplication is enabled, PDCP entity of such SL DRB is associated with two RLC bearer configurations for two separate RLC bearers. Otherwise, only one RLC bearer configuration is (pre)configured.

In last RAN2 meeting, we discussed about SL CA for SL SRB and achieved the following WA.

Agreement on PDCP duplication/SL CA for SL SRB

1:	Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

Whether to support SL CA for SL SRB, actually we think SL CA can be supported as long as both TX UE and RX UE supports carrier aggregation, which can be started after the exchange of UE capability. Therefore, we think we can confirm the support of SL CA for SL SRB as agreement.

Proposal 4: RAN2 agree to support SL CA for SL SRB after SL link is established and UE capability is exchanged.

However, regarding the support of PDCP duplication for SL SRB, different from SL DRB, the PDCP/RLC/MAC configuration for SL SRB is specified in the specification, in this case, if PDCP duplication is to be supported, RLC/MAC configuration associated with the duplicated leg for each SRB is required to be specified in the specified SCCH configuration table (i.e. 9.1.1.4 in TS 38.331).

Proposal 5: RAN2 agree RLC/MAC configuration associated with the duplicated leg for each SRB is specified in the specified SCCH configuration table.

Regarding how to enable/disable PDCP duplication, since the support of carrier aggregation is a prerequisites of the support of PDCP duplication, which also depends on the UE capability, it is not reasonable to always enable PDCP duplication for SRB. In this case, it can be up to UE implementation to enable/disable PDCP duplication for SRB.

Proposal 6: RAN2 agree it is up to UE implementation to enable/disable PDCP duplication for SRB.

2.3 HARQ based DTX

In last RAN2 meeting, we discussed about HARQ-based SL RLF and agreed the counting is per carrier.

Agreement on DTX based SL RLF in SL CA

1:	The counting is calculated per carrier.

2:	Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

Based on the above agreement, the UE variable used for HARQ-based Sidelink RLF detection, i.e., numConsecutiveDTX is maintained per carrier per unicast connection and the maximum allowable value sl-MaxnumConsecutiveDTX is reused.

Therefore, similar as legacy, the Sidelink HARQ Entity shall (re-)initialize numConsecutiveDTX associated with each carrier to zero for each PC5-RRC connection, upon establishment of the PC5-RRC connection or (re)configuration of sl-maxNumConsecutiveDTX.

Proposal 7: Sidelink HARQ Entity shall (re-)initialize numConsecutiveDTX associated with each carrier to zero for each PC5-RRC connection, upon establishment of the PC5-RRC connection or (re)configuration of sl-maxNumConsecutiveDTX.

In addition, if HARQ based counting on one carrier reaches the maximum allowable value, since SL-RLF is not triggered considering there is any other available carrier, UE should trigger resource/carrier reselection.

Proposal 8: UE triggers resource/carrier reselection if the numConsecutiveDTX associated with any carrier reaches the maximum for the established PC5-RRC connection.

2.4 Backward compatibility

As described in the WID, the carrier aggregation feature is backward compatible

- This feature is backwards compatible in the following regards

A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it re Agreements on backward compatibility issue in SL CA (for GC/BC)

1: 	Consider a case that a V2X service which needs to be mapped into multiple carriers while there is at least one legacy UE to receive this V2X service. RAN2 further discuss whether TX profile approach can be supported.

Actually there was a proposal to rely on the Service-to-carrier mapping from upper layer to guarantee the backward compatibility for BC/GC.

- For a service targeting at legacy CA-incapable UE, this service is mapped to a single frequency carrier.

- For a service targeting at enhanced CA-capable UE, this service is mapped to multiple frequency carriers.

However, during last RAN2 meeting, there was some concern that the existing R16/17 framework already allows mapping the V2X services to multiple carriers, therefore case 1 is not aligned with the current scheme and the following agreement was achieved.

Agreements on backward compatibility issue in SL CA (for GC/BC)

1: 	Consider a case that a V2X service which needs to be mapped into multiple carriers while there is at least one legacy UE to receive this V2X service. RAN2 further discuss whether TX profile approach can be supported.

However, for 2), considering some legacy UE may also need to receive this V2X service, since TX UE has no knowledge among these services which is only for R18 UEs and which should also be received by legacy UEs, to avoid any legacy UEs missing any packets that is interested to receive, TX UE can only enable PDCP duplication for all the services that can be mapped to multiple carriers if no TX profile is defined.

Therefore, in order to distinguish the services that can be mapped to multiple carriers for legacy UE and those for R18 UE only, as well as to achieve the benefit of data split on multiple carriers to increase the data rate for those for R18 only, it is proposed to define a TX profile for the service/L2 ID, which is quite similar as the R17 SL DRX scheme.

Proposal 9: Define a TX profile to distinguish between services that can be mapped to multiple carriers for legacy UE and services that can be mapped to multiple carriers for R18 only.

To be more detailed, a TX profile associated with a service for BC/GC indicates the support of carrier aggregation/data split. This TX profile is provided from upper layer to AS layer for each service and a UE only assumes carrier aggregation/data split for the given destination L2 ID when all the associated TX profiles correspond to support of carrier aggregation/data split. For a given destination L2 ID, all TX and RX UEs should be configured with the same set of TX profile(s).

Proposal 10: A UE only assumes carrier aggregation/data split for the given BC/GC destination L2 ID when all the associated TX profiles correspond to support of carrier aggregation/data split.

Proposal 11: For a given BC/GC destination L2 ID, all TX and RX UEs should be configured with the same set of TX profile(s).

Regarding the detailed UE behaviour, from TX UE perspective, for a given destination BC/GC L2 ID, if carrier aggregation/data split is supported, TX UE shall transmit on multiple carriers with data split for the corresponding RBs if PDCP duplication is configured as disabled or transmit on multiple carriers with PDCP duplication for the corresponding RBs if PDCP duplication is configured as enabled.

Proposal 12: For a given BC/GC destination L2 ID, if carrier aggregation/data split is supported, TX UE shall transmit on multiple carriers with data split or enable PDCP duplication as configured.

While for a given destination BC/GC L2 ID, if carrier aggregation/data split is not supported, TX UE can transmit on the legacy carrier only if the corresponding RB is configured as PDCP duplication disabled or enable PDCP duplication if the corresponding RB is configured to enable PDCP duplication.

Proposal 13: For a given BC/GC destination L2 ID, if carrier aggregation/data split is not supported, TX UE shall transmit on single carrier or enable PDCP duplication as configured.

From RX UE perspective, RX UE should receive on multiple carriers if there is at least one destination L2 ID that is interested to receive corresponds to support of carrier aggregation/data split.

Proposal 14: RX UE should receive on multiple carriers if there is at least one destination L2 ID that is interested to receive corresponds to support of carrier aggregation/data split.

3	Conclusion

In this contribution, we discussed about the issues related to carrier aggregation in NR sidleink and provide corresponding observations and proposals:

Proposal 1: In NR sidelink CA, same principle as LTE V2X CA is applied to determine per-carrier CBR.

Proposal 2: In NR sidelink CA, the UE considers a TX carrier as candidate carrier if the measured CBR is lower than the configured per-carrier-per-priority CBR threshold for carrier (re)selection and the HARQ-based DTX counting for any unicast link associated with the UE on this carrier does not reach the maximum.

Proposal 3: In NR sidelink CA, the UE stays at the same TX carrier as before if the measured CBR is lower than the configured per-carrier-per-priority CBR threshold for carrier keeping and the HARQ-based DTX counting for any unicast link associated with the UE on this carrier does not reach the maximum.

Proposal 4: RAN2 agree to support SL CA for SL SRB after SL link is established and UE capability is exchanged.

Proposal 5: RAN2 agree RLC/MAC configuration associated with the duplicated leg for each SRB is specified in the specified SCCH configuration table.

Proposal 6: RAN2 agree it is up to UE implementation to enable/disable PDCP duplication for SRB.

Proposal 7: Sidelink HARQ Entity shall (re-)initialize numConsecutiveDTX associated with each carrier to zero for each PC5-RRC connection, upon establishment of the PC5-RRC connection or (re)configuration of sl-maxNumConsecutiveDTX.

Proposal 8: UE triggers resource/carrier reselection if the numConsecutiveDTX associated with any carrier reaches the maximum for the established PC5-RRC connection.

Proposal 9: Define a TX profile to distinguish between services that can be mapped to multiple carriers for legacy UE and services that can be mapped to multiple carriers for R18 only.

Proposal 10: A UE only assumes carrier aggregation/data split for the given BC/GC destination L2 ID when all the associated TX profiles correspond to support of carrier aggregation/data split.

Proposal 11: For a given BC/GC destination L2 ID, all TX and RX UEs should be configured with the same set of TX profile(s).

Proposal 12: For a given BC/GC destination L2 ID, if carrier aggregation/data split is supported, TX UE shall transmit on multiple carriers with data split or enable PDCP duplication as configured.

Proposal 13: For a given BC/GC destination L2 ID, if carrier aggregation/data split is not supported, TX UE shall transmit on single carrier or enable PDCP duplication as configured.

Proposal 14: RX UE should receive on multiple carriers if there is at least one destination L2 ID that is interested to receive corresponds to support of carrier aggregation/data split.

4	Reference

[1] RP-230077 WID revision: NR sidelink evolution.

[2] RAN2#122bis meeting minutes.