3GPP TSG-RAN WG2 Meeting #123	       R2-2307736

Toulouse, France, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	On flightpath reporting

Agenda item:		7.8.3

Document for:		Discussion and Decision

- Introduction

In RAN2#121-bis meeting, following agreements are reached.

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flightpath indication.  FFS if further differentiation is needed if we decide to have delta signaling

In RAN2#122[2]:

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This contribution provides further consideration on some of the FFS issues.

- Discussion

In last RAN2 meeting, following cases of update of way points are discussed[3]:

Case 1.	Some outdated waypoints are removed as the UAV UE has already travelled through them.

Case 2.	Some waypoints’ timestamp is updated but location remains the same.

Case 3.	Some waypoints’ location (and possibly timestamp if it was indicated earlier) is updated due to change in planned path.

Case 4.	Some new waypoint(s) is/are inserted due to change in planned path.

First of all, we doubt whether case 1 is a valid case to trigger flightpath plan update. In our understanding, the flightpath plan is well planed. It is meaningful to trigger update procedure if the plan is changed. But if flightpath plan update is allowed to be triggered due to waypoints are travelled through, there will be a lot of flightpath plan updates and out of network’s control. Further, since network already has the flightpath, it is questionable that network needs to know which waypoints are travelled through.

Proposal 1: Flightpath plan update notification shall not be triggered due to some waypoints are travelled through.

As to delta reporting, some companies think it is beneficial to reduce the size of the report. However, we should first figure out how often flightpath update will happen. In understanding, usually the flightpath is fully updated since the flightpath is well planned carefully. Proponents think there is case only time stamp is changed due to whether reasons. But we still think this is not a frequent case.

Observation 1: Flightpath is usually fully updated. It is not frequent case where only time stamp is changed.

As to case 4, current ASN.1 structure ToAddModList and ToRemvoeList does not support inserting an entry in the middle of a list. But the flightpath waypoints should be ordered otherwise it is meaningless for the network. One way to reuse ToAddModList and ToRemvoeList for this case is to transfer all waypoints which locates after the changed waypoint. But the benefit of delta reporting no longer exists. Or, we may need to define a new set of ASN.1 structure or enhance the current delta signaling mechanism. But such method will be too complicated for infrequent flightpath update.

proposal 2: Delta flightpath reporting is NOT supported.

In RAN2#121bis[1] , it was agreed:

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flightpath indication. FFS if further differentiation is needed if we decide to have delta signaling

Previous agreement as copied above implies it is meaningful to differentiate initial and updated flightpath indication if delta signaling is supported. However, we think the differentiation of flightpath indication and delta signaling should be decoupled. In our understanding, delta signaling impacts only the signaling size. While the differentiation of flightpath indication is used for network to decide whether to acquire the flightpath.

Observation 2: Whether further differentiation in flightpath indication is needed is irrelevant to whether delta reporting is supported.

In our understanding, whether further differentiation in flightpath indication is necessary depends on whether the differentiation provides enough information for the network to distinguish whether the updated flightpath is meaningful for RRM or mobility decision. Unfortunately, we think such decision can only be determined by the network based on the full information of the flightpath, e.g. the location of waypoints in a cell. A simple differentiation/categorization by UE itself cannot provide enough information.

Observation 3: Whether an update is necessary can be only determined by the network based on the flightpath, but not based on the simple differentiation made by UE.

Based on above considerations, we propose that:

Proposal 3: No further differentiation in flightpath indication is needed.

- Conclusion and proposals

Observation 1: Flightpath is usually fully updated. It is not frequent case where only time stamp is changed.

Observation 2: Whether further differentiation in flightpath indication is needed is irrelevant to whether delta reporting is supported.

Observation 3: Whether an update is necessary can be only determined by the network based on the flightpath, but not based on the simple differentiation made by UE.

Proposal 1: Flightpath plan update notification shall not be triggered due to some waypoints are travelled through.

proposal 2: Delta flightpath reporting is NOT supported.

Proposal 3: No further differentiation in flightpath indication is needed.

- Referenced

- RAN2#121bis-e chairman notes

- RAN2#122 chairman notes

- R2-2305109, Delta reporting of flight path plan, Qualcomm Incorporated