3GPP TSG-RAN WG2 Meeting #123	R2-2307738

Toulouse, France, 21st – 25th Aug 2023

Agenda Item:	7.12.2.2

Source: 	vivo

Title:         	Discussion on Mobile IAB mobility enhancements

Document for: 	Discussion and Decision

# Introduction

RAN2 has previously made the following agreements:

- Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection. - FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

At last RAN2 meeting RAN2 has further made the following agreements:

R2 direction (solution agreements at later stage, no other directions will be considered): - RAN2 acknowledges following two problems to be addressed for idle/inactive UEs: - - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low. - - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells. - - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

In this contribution, we further discuss idle/Inactive mobility enhancement based on above identified problems

# Discussion

RAN2 discussion on mIAB cell indication has concluded with two direction problems to be addressed for idle/inactive UEs:

- Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

It is further considered that such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. Some assistance information to identify the presence of such mobile IAB-cell(s) may be used, if broadcasted, e.g., SIB4.

For problem 1, As the UE is on the mobile IAB, the UE should not prioritize inter-frequency cell re-selection of a stationary cell, but rather the UE should prioritize inter-frequency cell of the mobile-IAB-cell. For problem 2, the UE should only consider sell reselection of on mobile IAB cell frequency. By always prioritizing mobile IAB cell frequency can avoid UE camping or reselection to stationary cell. This can be done by prioritizing broadcasted mobile IAB frequencies.

Therefore,

- To address Problem 1, UE can only prioritize broadcasted inter-frequency of mobile IAB cells, i.e., UE can prioritize inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

For Problem 2, as the UE is already camping on the mobile IAB, to avoid the reselecting to stationary cell, prioritizing mobile IAB intra-frequency cells.

Therefore,

- To address Problem 2, UE can only prioritize broadcasted intra-frequency of mobile IAB cells, i.e., UE can prioritize intra -frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

As discussed above, in both Problem 1 and Problem 2, the UE just has to prioritize the broadcasted intra-frequency (for Problem 1) and inter-frequency (for Problem 2) of mobile IAB cells. No additional enhancement is required.

- No additional enhancement is required to address Problem 1 and Problem 2.

When it comes to whether to capture UE behavior in stage 2 or stage 3, we think stage description is enough, e.g., UE behavior captured in TS 38.304.

- Stage 2 description is sufficient to capture on-board UE behaviour, e.g., stage 2 description in TS 38.304

# Conclusion

In this paper, the following proposal are given:

- To address Problem 1, mobile IAB can only prioritize broadcasted inter-frequency of mobile IAB cells, i.e., mIAB-cell can prioritize inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

- To address Problem 2, mobile IAB can only prioritize broadcasted intra-frequency of mobile IAB cells, i.e., mIAB-cell can prioritize intra -frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

- No additional enhancement is required to address Problem 1 and Problem 2.

- Stage 2 description is sufficient to capture on-board UE behaviour, e.g., stage 2 description in TS 38.304

# References